3GPP TSG-RAN WG2 Meeting #106		                                                   R2-1905874

Reno, USA, 13th - 17th May 2019

Source:	CATT

Title:	Cell Selection and Reselection Issue between NTN and TN System

Agenda Item:	11.6.4.2

Document for:	Discussion and Decision

Introduction

Compared to current TN system, NTN is a new system. But so far, no much discussion is involved to identify the mobility issue between NTN and TN System. In this document, we’d like to discuss the mobility issue between NTN and TN System.

Discussion

Both NTN and TN system have its specific benefit. For NTN system, it can cover much more earth area compared to TN system, while TN system has less delay for the service. So if NTN and TN co-exist in the same area, what’s the UE behavior for mobility? If the UE is on the sea and there is no TN coverage, we think NTN cell is more desirable in this case. But when UE goes back to city, should UE keep stay in NTN cell? I guess UE would prefer to stay in TN cell considering the expense and the service delay. In order to simplify the analysis, we divide this issue into two parts.

2.1 Case1: NTN and TN share the same PLMN

In this case, the cell (re)selection background is quite similar to that in the TN system within the same PLMN. Currently, for idle/inactive UEs, mobility is controlled by UE itself based on dedicated frequency priorities or common frequency priorities. But the key issue is that NTN cell is much larger than TN cell, it’s hard for NTN cell to broadcast all the TN cell frequencies info within a NTN cell. More addition, all the satellites apart from GEO have speed relative to ground and a satellite cell will cover different TN cells by time. So the common frequency priorities broadcast in the NTN cell may be changed frequently. If UEs want to use the common frequency priorities to select a TN cell from NTN cell, some modification is needed for common frequency priorities.

As for dedicated frequency priorities, some modification is also needed. Too many TN cells are included in a NTN cell. How does the network set the dedicated frequency priorities while NTN cell is still moving by time? If the network always has the valid UE location info, maybe the network can set the UE specific frequency priorities based on UE location. But actually, we can’t assume that the network can always have a valid UE location. Even the network has the valid UE location info, the network should know the TN cell group info nearby the UE location in advance.

When UE is out of coverage or in any cell selection state in NTN cell, what’s the UE behavior? Should UE find a NTN cell or a TN cell to camp first? Or just by UE implementation?

Proposal 1: RAN2 is kindly asked to discuss the following issues for the case when NTN and TN share the same PLMN:

- Whether any modification is needed for common/dedicated frequency priorities if UEs want to use the common/dedicated frequency priorities to (re)select a TN cell from NTN cell.

- What the UE behavior is when UE is out of coverage or in any cell selection state in NTN cell.

2.2 Case2: NTN and TN have separate PLMN

According to the current specification, if the registered PLMN is available among these PLMNs, the MS shall not choose a different PLMN [1]. So if the NTN cell which belongs to registered PLMN is always available, UE will never leave the NTN cell automatically. But from UE perspective, it’s more power saving if UE is camping on the TN cell as UE doesn’t need to do the cell (re)selection frequently. More addition, UE will experience less time delay in TN cell compared to NTN cell. If UE can get the service either in NTN cell or in TN cell, are UEs only allowed to (re)select a TN cell from NTN cell manually?

What’s the UE behavior when UE is out of coverage or in any cell selection state in NTN cell? Should UE find a NTN cell or a TN cell to camp first? Or just by UE implementation?

Observation: According to the current specification, if the registered PLMN is available among these PLMNs, the MS shall not choose a different PLMN.

Proposal 2: RAN2 is kindly asked to discuss the following issues for the case when NTN and TN have separate PLMN:

- Are UEs only allowed to (re)select a TN cell from NTN cell manually?

- What the UE behavior is when UE is out of coverage or in any cell selection state in NTN cell.

Conclusion

In conclusion, we propose the followings:

Proposal 1: RAN2 is kindly asked to discuss the following issue for the case when NTN and TN share the same PLMN:

- Whether any modification is needed for common/dedicated frequency priorities if UEs want to use the common/dedicated frequency priorities to (re)select a TN cell from NTN cell.

- What the UE behavior is when UE is out of coverage or in any cell selection state in NTN cell.

Observation: According to the current specification, if the registered PLMN is available among these PLMNs, the MS shall not choose a different PLMN.

Proposal 2: RAN2 is kindly asked to discuss the following issues for the case when NTN and TN have separate PLMN:

- Are UEs only allowed to (re)select a TN cell from NTN cell manually?

- What the UE behavior is when UE is out of coverage or in any cell selection state in NTN cell.

References

[1] 3GPP TS 23.122