3GPP TSG RAN WG2 #106	 R2-1906195

Reno, US, 13 – 17 May, 2019	Revision of R2-1903884

Agenda Item:	12.3.3.2 Stage-3 aspects CHO

Source: 	Ericsson

Title:  	Configuration of Conditional handover in LTE

Document for:	Discussion and Decision

# Introduction

In RAN2#104 in Spokane it was agreed for LTE that conditional handover is one solution that should be considered for improving the handover robustness, as part of the equivalent work item for Further Enhanced Mobility Enhancements. In this contribution we discuss the structure of the CHO configuration containing triggering conditions and dedicated RRC configuration(s).

# Discussion

## Conditional Handover

In conditional handover the network configures the UE with triggering conditions for when a handover should be executed. When the conditions are fulfilled, the UE executes the handover without any further order from the network. The advantage of the procedure is that the HO Command may be provided to the UE at an earlier stage before the radio conditions have become poor, which increases the chance of a successful transmission of the message.

When the source eNB decides to trigger a handover, source eNB provides the target eNB with the UE’s current configuration in a HO Request over X2. If the target eNB accepts the incoming handover, it prepares a target configuration to be provided to the UE in the handover command. This is basically the configuration the UE shall apply when it executes the handover.

Applying the same principles of legacy handover, the source node provides the UE’s configuration to each target node with a candidate cell and, each target node candidate prepares a UE configuration to be applied if CHO is executed.

Source eNB provides each target candidate with a UE configuration.

Each candidate target eNB prepares a UE configuration to be applied if CHO is executed.

## Conditional handover configuration message

If the source node prepares multiple candidate cells for CHO, the UE may need to receive an RRC configuration for each target cell candidate. That would basically be the content of an RRCConnectionReconfiguration message containing a mobilityControlInfo for each target cell candidate. Associated to each of these configurations, the UE receives a triggering condition configuration (e.g. like an A3 event configuration). Hence, the signalling for the CHO configurations needs to consider the possibility to configure multiple target cell candidates.

One approach to configure CHO could be to enhance the existing RRCConnectionReconfiguration message with CHO-specific fields/IEs for the target cell candidate, such as the inclusion in the RRCConnectionReconfiguration with mobilityControlInfo and the triggering condition configurations, to distinguish a conditional HO from a legacy handover. Upon reception, the UE shall not apply the message until the triggering condition is fulfilled. A first potential implication to be discussed for this option is the impact on the procedure text e.g. in the actions upon reception of an RRCConnectionReconfiguration. In current procedure text the UE performs the actions when receiving the message, whereas for conditional handover only some of the actions should be executed immediately and others when the conditional handover is executed. This type of behaviour does not exist in the current structure. A second implication is in generating the CHO configuration(s) on the network side. Since it is the source eNB that should generate the triggering condition configuration(s), and the target that should generate the RRC configuration, the source eNB would have to add the trigger configuration in a message created by target.

Another possible implication is the need to provide the UE with multiple RRCConnectionReconfiguration messages for the same conditional handover procedure, and the fact that these messages will not be received and processed at the same time. Imagine that the UE reports a set of triggered cells (i.e. cells that could be CHO target candidates, like cells 1, 2 and 7). Then, the network prepares these cells and decides to configure the UE with CHO. If for each of them, an RRCConnectionReconfiguration is to be provided, the UE will process message by message, i.e., it will start to monitor the trigger condition for the first message, then for the second, and so on. Hence, as the monitoring may lead to the fulfilment of triggering condition, the UE may execute a CHO even before it receives all intended messages, i.e. for all cell candidates, from the network. This implication may not necessarily lead to a conditional handover failure, but the UE may end up executing CHO to a candidate cell that is not the best, which may lead to a subsequent handover. Also, it the handover is not executed immediately the UE will end up sending several Complete messages which causes unnecessary signalling.

Another approach that seems to be simpler is to define one message (e.g. RRCConnectionConditionalReconfiguration) carrying one or multiple bit string RRCConnectionReconfiguration(s) and trigger condition configurations associated to each target cell candidate. This seems to be a cleaner approach to solve the potential issues described above. In addition, it may also simplify the procedure text in the specification. Also, if only one reconfiguration message is sent, only one Complete message needs to be sent if the handover does not occur immediately, which reduces the amount of signalling. However, it may require the creation of a new message.

Both approaches work, so we suggest RAN2 to discuss these implications and the two solutions.

- Proposal 1: Discuss the two alternatives for configuring CHO:

    - RRCConnectionReconfiguration;

    - RRCConnectionConditionalReconfiguration;

## Handling of subsequent reconfigurations

One more aspect needs to be considered when deciding how the CHO configuration will be signalled. It may happen that other reconfigurations are made after the UE has been configured with conditional handover, but before any handover has taken place and it needs to be clear how to handle these reconfigurations.

With a structure to add, modify or remove conditional handover configurations, the network has full flexibility about what to do in such a case. It can keep the conditional handover configuration, modify it or delete it. If the target cells are in the same eNB, the eNB knows if the configuration needs to be modified and can make a correct decision on its own. If the target cells are in a different eNB, the source eNB can ask the target eNB about a new target configuration and in such case modify the conditional handover configuration or simply delete the conditional handover configuration.

It is assumed that it is not the most typical case that the configuration changes between the conditional handover configuration and the execution of the handover and in those few cases it can be an implementation choice by the network what to do depending on the number of target cells configured, if the cells are in the same eNB or not and what type of configuration change it is.

Hence, from RRC perspective, one only needs to define an efficient signalling allowing the network to add, modify and remove CHO configurations i.e. dedicated RRC configurations for target cell candidates.

Proposal 2: Conditional handover configurations can be added, modified or removed per target cell candidate.

Notice that both approaches described in proposal 9 may achieve what is being proposed i.e. possibility to add, remove and modify CHO configuration. However, there may be different implications for the different solutions. In the case of enhancing the existing RRCConnectionReconfiguration with mobilityControlInfo, the removal may be done by adding a flag and/or defining appropriated need codes for the field/IEs associated to conditional handover e.g. absence of certain fields indicating that the UE shall stop the monitoring the condition for the associated cell and remove the stored RRC configuration. In our view, this seems complicated as most fields in the RRCConnectionReconfiguration will be common to CHO and to legacy handover e.g. measConfig, and, for the legacy case, need codes have already been defined to allow delta signalling upon reception, while herein the absence would indicate something to be clearly specified (i.e. Need OP). Also, the removal/addition or modification of multiple CHO configurations may lead to the transmission of multiple RRC messages, which may lead to race conditions and additional processing time.

In the second approach, i.e., where a specific message e.g. RRCConnectionConditionalReconfiguration is defined, removal, addition and modification could be achieved by a structure similar to the AddMod and Remove lists in measurement configuration, for example. Therein a single message may be transmitted to add, remove or modify CHO configurations decided by the network, which minimizes the risk of race conditions and reduces processing delay.

## Number of target cells for conditional handover

In RAN2#104 it was agreed that it should be possible to configure multiple target cells for conditional handover. The exact number to be configured is FFS. The exact number is better decided when the stage 3 CR is done and it is known how the rest of the concept will work. It doesn’t give any extra value to decide it early, and there only is a risk for a bad decision when it is now clear exactly how the feature will be designed.

Proposal 3: Decide the maximum number of potential target cells for conditional handover when stage 3 is done.

# Summary

RAN2 is kindly asked to discuss the following proposals:

- Proposal 1: Discuss the two alternatives for configuring CHO:

- RRCConnectionReconfiguration;

- RRCConnectionConditionalReconfiguration;

Proposal 2: Conditional handover configurations can be added, modified or removed per target cell candidate.

Proposal 3: Decide the maximum number of potential target cells for conditional handover when stage 3 is done

# References

- RP-181475, Work Item on Even further Mobility enhancements in E-UTRAN, China Telecom, RAN#80, La Jolla, USA, May 21 – May 25 2018 September 2018.