3GPP TSG-RAN WG2 #107	Tdoc R2-1909608

Prague, Czech Republic, 26th – 30th August 2019

Agenda Item:	11.12.2

Source:	Ericsson

Title:	SN triggered configuration for logged MDT

Document for:	Discussion, Decision

# 1	Introduction

In the study item, “Study on RAN-centric data collection and utilization for LTE and NR (FS_LTE_NR_data_collect)”, Minimization of Drive Tests (MDT) is one of the use cases to be studied for NR and LTE enhancement.

In RAN2 #105, the following high-level agreements were made pertaining to support of MDT in NR:

Agreements of NR MDT: 1.Logged MDT, immediate MDT and accessibility report should be supported for NR MDT. LTE MDT measurements/failures could be the baseline 2.Logged MDT should also be supported for RRC_INACTIVE and RRC_IDLE. 3.Management based and signalling based trace procedure in LTE can be reused in NG-RAN MDT.

In RAN2 #106, the following high-level agreements were made pertaining to support of MDT in NR:

Agreements.

1	Immediate MDT configurations are supported for DC scenario.

2	Logged MDT configurations can come from SN node in DC scenario.

3	The existing MDT framework is the baseline for the SCG cells related MDT configuration.

4	The triggers for MDT measurements associated to MCG and SCG are separate.

5	MN-SN coordination is required for MDT measurements’ configuration and reporting in DC framework.

6	If SRB3 is not configured, SN related measurements are transmitted to MN via SRB1/2 and then forwarded to SN.

7	If SRB3 is configured, MN related measurements are transmitted to MN via SRB1/2, SN related measurements are transmitted to SN via SRB3

Since, the baseline agreements on NR MDT have been made now, we can discuss the dual connectivity aspects of MDT as well. In this paper, we discuss the support for SN based logged MDT measurements and the current framework available in LTE to support such measurements.

# Discussion

## 2.1 Logged MDT configurations can come from SN node in DC scenario

In RAN 2#106, an agreement was made that SN node would be able to configure the logged MDT towards the UE independent of the MN. The intention was to provide flexibility in the MDT framework for SN to request logged MDT measurements on its own.

In the same decision chain, it was also agreed that the SCG configuration for MDT would follow the LTE framework. One thing that needs to be analyzed is the information available in the logged MDT measurements for SCG cells. Currently, there is no explicit configuration available for

If we analyzed the current MDT framework for LTE logged measurements, the UE reports detailed logged measurement reports for intra and inter RAT neighbor cells already in current measurement framework.

Fig. 1 describes the current information elements reported in LTE framework for reporting neighbor cell measurements during logged MDT. The neighbor measurements are standardized for all RAT’s except NR and include measurements related to the following information elements:

- Cell RSRP

- Cell RSRQ

- Physical Cell Id

- CGI

- Carrier Frequency

- SINR values

- PLMN information

-- ASN1START

VarLogMeasReport-r10 ::=				SEQUENCE {

traceReference-r10					TraceReference-r10,

traceRecordingSessionRef-r10			OCTET STRING (SIZE (2)),

tce-Id-r10							OCTET STRING (SIZE (1)),

plmn-Identity-r10					PLMN-Identity,

absoluteTimeInfo-r10				AbsoluteTimeInfo-r10,

logMeasInfoList-r10					LogMeasInfoList2-r10

}

VarLogMeasReport-r11 ::=			SEQUENCE {

traceReference-r10					TraceReference-r10,

traceRecordingSessionRef-r10		OCTET STRING (SIZE (2)),

tce-Id-r10							OCTET STRING (SIZE (1)),

plmn-IdentityList-r11				PLMN-IdentityList3-r11,

absoluteTimeInfo-r10				AbsoluteTimeInfo-r10,

logMeasInfoList-r10					LogMeasInfoList2-r10

}

LogMeasInfoList2-r10 ::=				SEQUENCE (SIZE (1..maxLogMeas-r10)) OF LogMeasInfo-r10

LogMeasInfoList2-r10 ::=                SEQUENCE (SIZE (1..maxLogMeas-r10)) OF LogMeasInfo-r10

LogMeasInfo-r10 ::=		SEQUENCE {

locationInfo-r10					LocationInfo-r10		OPTIONAL,

relativeTimeStamp-r10				INTEGER (0..7200),

servCellIdentity-r10				CellGlobalIdEUTRA,

measResultServCell-r10				SEQUENCE {

rsrpResult-r10						RSRP-Range,

rsrqResult-r10						RSRQ-Range

},

measResultNeighCells-r10			SEQUENCE {

measResultListEUTRA-r10				MeasResultList2EUTRA-r9		OPTIONAL,

measResultListUTRA-r10				MeasResultList2UTRA-r9		OPTIONAL,

measResultListGERAN-r10				MeasResultList2GERAN-r10	OPTIONAL,

measResultListCDMA2000-r10			MeasResultList2CDMA2000-r9	OPTIONAL

}	OPTIONAL,

...,

[[	measResultListEUTRA-v1090			MeasResultList2EUTRA-v9e0	OPTIONAL

]],

[[	measResultListMBSFN-r12				MeasResultListMBSFN-r12	OPTIONAL,

measResultServCell-v1250			RSRQ-Range-v1250			OPTIONAL,

servCellRSRQ-Type-r12				RSRQ-Type-r12				OPTIONAL,

measResultListEUTRA-v1250			MeasResultList2EUTRA-v1250	OPTIONAL

]],

[[	inDeviceCoexDetected-r13			ENUMERATED {true}			OPTIONAL

]],

[[	measResultServCell-v1360			RSRP-Range-v1360			OPTIONAL

]],

[[	logMeasResultListBT-r15			LogMeasResultListBT-r15		OPTIONAL,

logMeasResultListWLAN-r15			LogMeasResultListWLAN-r15		OPTIONAL

]]

}

MeasResultList2EUTRA-r9 ::=                SEQUENCE (SIZE (1..maxFreq)) OF MeasResult2EUTRA-r9

MeasResult2EUTRA-r9 ::=                SEQUENCE {

carrierFreq-r9                        ARFCN-ValueEUTRA,

measResultList-r9                    MeasResultListEUTRA

}

MeasResultListEUTRA ::=				SEQUENCE (SIZE (1..maxCellReport)) OF MeasResultEUTRA

MeasResultEUTRA ::=	SEQUENCE {

physCellId							PhysCellId,

cgi-Info							SEQUENCE {

cellGlobalId						CellGlobalIdEUTRA,

trackingAreaCode					TrackingAreaCode,

plmn-IdentityList					PLMN-IdentityList2				OPTIONAL

}															OPTIONAL,

measResult							SEQUENCE {

rsrpResult							RSRP-Range						OPTIONAL,

rsrqResult							RSRQ-Range						OPTIONAL,

...,

[[	additionalSI-Info-r9				AdditionalSI-Info-r9		OPTIONAL

]],

[[	primaryPLMN-Suitable-r12			ENUMERATED {true}			OPTIONAL,

measResult-v1250					RSRQ-Range-v1250			OPTIONAL

]],

[[	rs-sinr-Result-r13					RS-SINR-Range-r13			OPTIONAL,

cgi-Info-v1310						SEQUENCE {

freqBandIndicator-r13				FreqBandIndicator-r11		OPTIONAL,

multiBandInfoList-r13				MultiBandInfoList-r11		OPTIONAL,

freqBandIndicatorPriority-r13		ENUMERATED {true}			OPTIONAL

}																OPTIONAL

]],

[[

measResult-v1360					RSRP-Range-v1360					OPTIONAL

]],

[[

cgi-Info-5GC-r15		SEQUENCE (SIZE (1..maxPLMN-r11)) OF CellAccessRelatedInfo-5GC-r15		OPTIONAL

]]

}

}

-- ASN1STOP

Fig. 1 Intra and inter RAT Neighbor Info in logged MDT (36.331)

Fig. 2 describes the current information elements reported in NR framework for reporting neighbor cell measurements. Currently, logging framework is not specified in NR, but these measurements could be used to extend the logging framework eventually to cover intra and inter RAT logged MDT measurements in NR.  Also, the current LTE framework needs to be enhanced to cover the inter RAT NR measurements as well.

-- ASN1START

MeasResultListNR ::=                    SEQUENCE (SIZE (1..maxCellReport)) OF MeasResultNR

MeasResultNR ::=                        SEQUENCE {

physCellId                              PhysCellId                                                                  OPTIONAL,

measResult                              SEQUENCE {

cellResults                             SEQUENCE{

resultsSSB-Cell                         MeasQuantityResults                                                 OPTIONAL,

resultsCSI-RS-Cell                      MeasQuantityResults                                                 OPTIONAL

},

rsIndexResults                          SEQUENCE{

resultsSSB-Indexes                      ResultsPerSSB-IndexList                                             OPTIONAL,

resultsCSI-RS-Indexes                   ResultsPerCSI-RS-IndexList                                          OPTIONAL

}                                                                                                               OPTIONAL

},

...,

[[

cgi-Info                                CGI-InfoNR                                                                    OPTIONAL

]]

}

MeasResultListEUTRA ::=                 SEQUENCE (SIZE (1..maxCellReport)) OF MeasResultEUTRA

MeasResultEUTRA ::=                     SEQUENCE {

eutra-PhysCellId                        PhysCellId,

measResult                              MeasQuantityResultsEUTRA,

cgi-Info                                CGI-InfoEUTRA                                                               OPTIONAL,...

}

MultiBandInfoListEUTRA ::=              SEQUENCE (SIZE (1..maxMultiBands)) OF FreqBandIndicatorEUTRA

MeasQuantityResults ::=                 SEQUENCE {

rsrp                                    RSRP-Range                                                                  OPTIONAL,

rsrq                                    RSRQ-Range                                                                  OPTIONAL,

sinr                                    SINR-Range                                                                  OPTIONAL

}

-- ASN1STOP

Fig. 2 Intra and inter RAT Neighbor measurement in 38331

Based on the above discussion, it is quite clear that the current LTE framework for logged MDT provide quite substantial information for inter and intra RAT neighbors. These inter and intra RAT neighbors could be SCG cells as well since SCG cells should have coverage overlap with serving cell. Thus, UE would provide logged MDT measurements for SCG cells as part of current framework as well.

Current LTE framework for logged MDT provides quite substantial information for inter and intra RAT neighbors.

One concern with the current reporting mechanism is that CGI reporting for neighbor cells is optional which might be very relevant in some cases. Another issue with relying on neighbor cell information is that information could be very broad since there could be multiple neighbouring frequencies and this is not the desired behavior if the operator is only interested to see MDT report for a specific frequency, i.e mmwave bands. Furthermore, If the operators would like UEs camping on LTE cells to collect the measurements on high-frequency NSA NR neighboring cells, there are no way for UE to check if a detected neighbor cell is one of the desired NR neighbor cell since the current way (in LTE) is that UE only checks if the serving cell is a part of the Area Configuration.

Current LTE framework have limitations specifically the broad list of neighbors reported, and it does not provide possibility to receive report on specific neighbor frequencies.

Another problem with current MDT mechanism is that the configuration do not allow inter RAT movement although there would be valid scenarios with such movement after release 15, i.e cell reselection from EN-DC to NR stand alone cell. So, the MDT framework should be able to provide precise report covering multiple RATs in dual connectivity scenarios.

MDT framework should be able to provide precise report covering specific RATs, frequency or cells in dual connectivity scenarios.

Since RAN 2 have already proposed to have SN based configuration in dual connectivity scenarios, the MDT configuration should be done so that the MDT report provides coverage and QoS on specific RAT or frequency instead of a diverse report with multiple frequencies and RATs. So, the MDT mechanism should be able to provide precise reports for scenarios where only MN configures the MDT report or both MN and SN configures the MDT report.

To increase the precision of the report, one option is to define a sub area configuration on top of area configuration which defines specifically the cells, frequency or RAT which needs to be reported as part of neighbor measurements. The sub area configuration can be done in different ways covering different targets including:

  - a frequency layer is configured in the subAreaConfiguration for which the UE is expected to log the measurements. Whenever the UE has measurements available for this frequency, the UE shall log the cells for which the measurements are performed and also the location.

  - a set of PCIs of a frequency layer is configured. In this scenario, only when the configured cells of that frequency are measured, the UE shall log these measurements.

  - a set of globally unique IDs of cells in a frequency layer is configured.

In option a and b, the UE should be able to report either the PCI only as the cell identifier or both PCI and globally unique IDs (requires reading SIB1) are logged.

This sub area configuration should be configured by both MN and SN since the UE might switch MN and SN and the configuration should still be relevant. Another reason is that the MN and SN could have different OAM which can provide MDT configuration independent of each other.

MN and SN could have different OAM which can provide MDT configuration independent of each other.

A sub area configuration should be included in Logged MDT framework to narrow the neighbor MDT measurements to specific RAT, frequency, PCI or cell ID.

The sub area configuration should be configurable by both MN and SN.

Another consideration is that the MDT reports for MN and SN should be independent and reported to respective node that triggered it since OAM can be separate for MN and SN.

Logged MDT report for MN and SN should be reported to respective node providing the initial configuration.

# Conclusion

In the previous sections we made the following observations:

Observation 1	Current LTE framework for logged MDT provide quite substantial information for inter and intra RAT neighbors.

Observation 2	Current LTE framework have limitations specifically the broad list of neighbors reported, and it does not provide possibility to receive report on specific neighbor frequencies.

Observation 3	MN and SN could have different OAM which can provide MDT configuration independent of each other.

Based on the discussion in the previous sections we propose the following:

- MDT framework should be able to provide precise report covering specific RATs, frequency or cells in dual connectivity scenarios.

A sub area configuration should be included in Logged MDT framework to narrow the neighbor MDT measurements to specific RAT, frequency, PCI or cell ID.

The sub area configuration should be configurable by both MN and SN.

Logged MDT report for MN and SN should be reported to respective node providing the initial configuration.

# References

R2-1900652, MDT Overview, Ericsson, RAN2 #105, Athens, Greece, 25 Feb – 1 Mar 2019

TS 37.320, Radio measurement collection for Minimization of Drive Tests (MDT), 3GPP, V15.0.0, 2018-07

TS 36.331, Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification, 3GPP, V15.4.0, 2019-02