3GPP TSG-RAN WG2 #107	Tdoc R2-1910143

Prague, Czech Republic, 26th – 30th August 2019

Agenda Item:	11.16 (NR_eMIMO-Core)

Source:	Ericsson

Title:	Protocol structure for Multi-TRP operation with multi-PDCCH/PDSCH

Document for:	Discussion, Decision

# 1	Introduction

RAN1 agreed to support so-called Multi-TRP operation, i.e., several transmission points transmitting to a UE on the same carrier frequency. In their LS [1] RAN1 describes various modes of operation that they intend to support, and they hint at certain things that RRC signalling should take care of.

In this document we analyse how the multi-PDCCH/multi-PDSCH variant of the Multi-TRP operation could be realized from User- and Control Plane Protocol perspective. In short, we conclude that one can re-use the existing carrier aggregation framework with few or even no changes to RRC signalling and to user plane protocols.

# 2	Discussion

## 2.1	Background

RAN1 concluded to support Non-Coherent Joint Transmission (NC-JT) in two main variants:

Single-PDCCH/PDSCH

  - A single common PDCCH schedules a single PDSCH with two layer-groups (one per TRP)

  - Typically, one HARQ process handles the data for all layers

  - ACK/NACK transmitted on a single PUCCH

  - Transparent from RAN2 UP perspective

Multi-PDCCH/PDSCH

  - PDCCH+PDSCH transmitted separately from each TRP

  - The two PDSCHs should be served by separate HARQ processes

  - ACK/NACK transmitted either on a single-PUCCH towards one TRP or on separate PUCCHs towards the two TRPs.

  - TRPs may belong to different cells (different PCI) or to the same cell (same PCI)

RAN1 did not consider NC-JT in uplink as it would basically require splitting the uplink resources (frequency & power) among TRPs and therefore result most likely in worse uplink performance than transmitting towards just one TRP. Therefore, a PUSCH exists only between the UE and a master TRP. Whether the network attempts to receive that PUSCH also from other TRPs (coherently) is up to network implementation and out of the scope of this WI.

In this contribution we focus on the multi-PDCCH/PDSCH variant of multi-TRP operation.

## 2.2	User-Plane Protocol Structure

Naturally, RAN1 did not discuss the L2 protocol stack beyond the need to use additional HARQ processes handling the transport blocks arriving via the PDSCH of the second TRP.

Figure 1 and Figure 2 depict two schematic protocol views of the network side. They reflect what RAN1 describes in their LS [1] and what we summarized in section 2.1 for the multi-PDCCH/PDSCH variant. They differ in the way how HARQ processes and HARQ entities are associated to the two TRPs and the PDSCHs therein.

Figure 1: single HARQ entity with increased number of HARQ processes serving the two PDSCH instances in the primary TRP and in the secondary TRP respectively.

Figure 2: two HARQ entities serving one TRP each.

| Figure 1: Common PDCP, RLC, MAC and HARQ entity | Figure 2: Common PDCP, RLC, MAC. Separate HARQ entities located in master TRPs |
| --- | --- |

A single HARQ entity could allow to perform the initial transmission of a HARQ process via one TRP and to send subsequent retransmissions via the other TRP. If realized with two HARQ entities, RLC would typically perform the retransmission which could then end up in a HARQ process of the other HARQ entity. The latter might appear as a drawback. However, the channel quality towards the two TRPs will likely differ significantly so that the appropriate transport block sizes will differ, too. And therefore, RLC re-segmentation and/or concatenation will often be necessary anyway to finalize transmission on another TRP.

A key benefit of using one HARQ entity per TRP (Figure 2) is that it matches exactly the protocol structure of carrier aggregation, i.e., each HARQ entity serves a PDSCH. Feedback is received via a single PUCCH (of the PCell) or via separate PUCCHs per cell (PUCCH-SCell). The number of HARQ processes per HARQ entity and consequently the DCI formats would not need to be changed.

A protocol structure with separate HARQ entities per TRP matches exactly what is well-established for carrier aggregation (one HARQ entity per serving cell; common RLC&MAC)

Serving the two TRPs by separate HARQ entities would also allow a network side implementation where the HARQ entity for the second TRP is inside that TRP (Figure 3).

Figure 3: Common PDCP, RLC, MAC. Separate HARQ entities located in respective TRP

From UE perspective the two variants (Figure 2 & Figure 3) are equivalent. From network perspective the latter might be preferable to co-locate the HARQ entity with the reception point of the PUCCH (in case of multi-PUCCH configuration). If the interface between the two TRPs is not negligible, co-locating the HARQ entities could reduce the HARQ RTT for retransmissions but it would of course shift the delays upwards (between RLC and HARQ).

A protocol structure with separate HARQ entities per TRP allows network implementations where the HARQ entities are either in one “master TRP” or in the TRP that they serve.

In general, RAN2 should establish a common understanding, which inter-TRP latencies could be supported.

If the inter-TRP link is non-ideal (larger than some 10 µs), the structures according to Figure 1 and Figure 2 suffer from non-negligible latency between RLC/MAC/Scheduler on one side and the PDCCH/PDSCH transmission on the other side. This requires that the protocol entities in the primary TRP pre-schedule the actual PDCCH/PDSCH transmission by the maximum expected inter-TRP latency. Obviously, this increases the effective HARQ RTT. With multi-PUCCH configuration, the ACK/NACK will experience the same delay a second time on the way to the MAC/Scheduler.

With a structure according to Figure 3 the inter-node delay occurs between the scheduler (in the secondary TRP) and the RLC entity (in the primary TRP) causing effectively also a longer HARQ RTT since the scheduling decision (in the transmitting TRP) has to be taken so far in advance that data can be collected from the RLC entity.

Therefore, and as discussed already in the context of LTE-DC in Rel-12, all protocol structures shown above suffer from the fact that the inter-TRP delay is within the RLC/MAC/HARQ loop. Increasing the RTT in this loop increases the e2e latency since retransmissions on HARQ and RLC layer take longer. An increased delay makes also link adaptation less accurate and may therefore reduce the L1 throughput. If the inter-TRP connection is under-dimensioned congestion may occur and result in packet loss. Since RLC spans across that interface it would attempt to recover from those losses making the situation even worse (hiding the congestion from TCP, taking even longer to recover from the loss due to queue on the interface; ...).

Therefore, the protocol stacks shown above are preferably used with an ideal interface (latency in the order of a few µs) between the transmission points. But delays up to a few (lower single digit) ms could be feasible provided that the latency is predictable (upper-bound) and that no congestion related losses occur between the TRPs.

A CA-like UP protocol architecture is primarily suitable for TRPs connected with ideal backhaul but latency up to a few ms could be feasible if congestion related losses are guaranteed not to occur on the inter-TRP connection.

If one wanted to cope with longer inter-TRP delays, RLC and HARQ should be terminated in the respective nodes. In other words, a dual-connectivity like structure should be applied as shown in Figure 4.

Figure 4: Common PDCP. Separate RLC, MAC and HARQ entities located in master TRP

However, as mentioned above, RAN1 did not consider it feasible or desirable to operate a separate (NC-JT) PUSCH between the UE and the secondary TRP. Without such PUSCH (shown in red in Figure 4), it is not possible to feed MAC CEs (PHC, BSR) nor RLC status reports from the UE to the secondary TRP. The lack of the secondary PUSCH in RAN1’s Rel-16 concept rules out the DC protocol stack for multi-TRP operation.

Due to lack of a PUSCH from the UE to the secondary TRP it is not possible to apply the protocol stack of Dual Connectivity for multi-TRP operation.

If one intends to RRC-configure several (more than two) TRPs but use only a subset (e.g. two) at a time, the SCell Activation/Deactivation framework could be used for this purpose. For example, one could configure one primary TRP and several secondary TRPs and ensure by MAC Activation/Deactivation MAC CE that only one of the secondary TRPs is active at a time.

If desirable, one may RRC-configure one primary TRP and several secondary TRPs and ensure by the existing MAC Activation/Deactivation Control Element that only one of the secondary TRPs is active at a time.

Based on the analysis in this section we propose the following:

Re-use the User Plane protocols stack of Carrier Aggregation with one HARQ entity per TRP to realize the Multi-TRP operation with multi-PDCCH/PDSCH.

## 2.3	RRC Signalling Structure

In the previous section we showed that the user plane protocol stack of carrier aggregation can be re-used for multi-TRP operation. In this section we analyse whether the RRC signalling structure for CA would also match to RAN1’s multi-TRP plans.

To start with, we show the simplified ASN.1 structure of a “Serving Cell” (SpCell or SCell). Only the high-level IEs and those fields that seem most relevant for multi-TRP operation are visualized.

1> ServingCell 2> ServCellIndex 2> physCellId 2> downlink 3> ARFCN 3> PDSCH-ServingCellConfig 4> pucch-Cell 3> BWP 4> PDCCH-Config 5> CORESET 7> pdcch-DMRS-ScramblingID 5> SearchSpace 4> PDSCH-Config 5> dataScramblingIdentityPDSCH 5> DMRS 7> scramblingID0 7> scramblingID1 5> TCI-States 2> uplink 3> ARFCN 3> BWP 4> PUCCH-Config 4> PUSCH-Config 5> dataScramblingIdentityPUSCH 5> DMRS 7> scramblingID0 7> scramblingID1 4> SRS-Config 4> RACH-Config 3> CSI-MeasConfig 4> NZP-CSI-RS-Resources 6> scramblingID 4> CrossCarrierSchedulingConfig

Figure 5: Simplified ASN.1 structure for configuring a ServingCell

In the following we investigate how one could populate the existing RRC parameters to configure the UE for operation with two TRPs so that each ServingCell object represents one TRPs. As said in the previous section, we consider here the case with separate PDCCH and PDSCH per TRP.

### Frequency, PCI and ServCellIndex

Both TRPs operate on the same spectrum. Hence, the two ServingCell objects would indicate the same SSB frequency, the same “Point A” and the same SCS-SpecificCarrier(s). In Figure 5 we summarized those parameters as “ARFCN”.

Two ServingCell objects with same SSB- and Point-A frequency and with the same SCS-SpecificCarrier configuration can be used to configure two TRPs.

According to RAN1, the cooperating TRPs may belong to the same cell (same PCI) or to different cells (different PCI). Already today it is not forbidden to configure two serving cells with the same PCI as the two cells are anyway known to be different due to different carrier frequencies. In Multi-TRP operation the ServingCell objects would be on the same frequency (see Observation 7). If they also have the same PCI the UE could deduce that they are actually two TRPs of the same cell. If the PCI is different, they are known to be two TRPs on the same frequency but associated with different cells.

The two ServingCell objects representing the two TRPs may have the same or different physCellId. This enables RAN1’s scenarios where the TRPs belong to the same or different cells.

To distinguish the ServingCell objects in RRC signalling, they are associated with the ServCellIndex (or SCellIndex). If used for multi-TRP operation, this must still be the case, i.e., each ServingCell object must have a unique ServCellIndex.

Each ServingCell object must have a unique ServCellIndex (or SCellIndex).

### PDSCH and PDCCH

The RAN1 LS [1] mentions that the number of CORESETs needs to be adjusted for Multi-PDCCH operation (“For multi-PDCCH based multi-TRP operation, increase the maximum number of CORESETs per “PDCCH-config” to 5, according to UE capability”). When instantiating one ServingCell object per TRP, we get also (at least) one PDCCH-Config per TRP. Therein, the network may configure separate ControlResourceSet and SearchSpace configurations with different scrambling or different time/frequency resources. Hence, unlike suggested by RAN1, the additional CORESETs for the second TRP would not be configured in the PDCCH-Config of the main TRP but in the separate PDCCH-Config of that TRP. Hence, the number of CORESETs per PDCCH-Config does not need to be increased and the RRC signalling does not need to be modified.

The existing PDCCH-Config allows configuring the PDCCHs for the TRPs with independent CORESET- and SearchSpace configurations.

Instantiating separate ServingCell objects with separate PDCCH-Config:s per TRP avoids increasing the number of CORESETs per PDCCH-Config and thereby allows re-using existing RRC ASN.1 as is.

The ServingCell objects for the TRPs should also contain a PDSCH-Config. It provides among others the “dataScramblingIdentityPDSCH”. If there is one PDSCH-Config per TRP, this fulfils RAN1’s request to configure one such scrambling ID for each TRP.

The PDSCH-Config also informs the UE about DMRS configurations which also contain scrambling IDs as well as a list of TCI-States which usually refer to a set of CSI reference signals.

Configuring PDCCH and PDSCH for one TRP inside a ServingCell object implicitly provides a mapping between them: PDCCH DCIs decoded according to the PDCCH/CORESET/SearchSpace configuration within one ServingCell object are implicitly known to carry assignments for the PDSCH configured by the PDSCH-Config in the same ServingCell object.

The existing PDSCH-Config allows configuring the PDSCHs for the TRPs with independent scrambling IDs, DMRS settings and TCI-States.

In their LS, RAN1 suggests extending the CRS rate matching configuration to cater for cases where different TRPs represent different LTE cells (“For rate matching mechanism used for multi-DCI based multi-TRP/panel transmission and for LTE CRS, extending lte-CRS-ToMatchAround to be configured with multiple CRS patterns in a serving cell”). When configuring the connection to each TRP by a separate ServingCell object, the lte-CRS-ToMatchAround exists in each and thereby offers what RAN1 requested without further additions in ASN.1.

By configuring a ServingCell object per TRP, the lte-CRS-ToMatchAround can be configured independently per TRP.

### PUCCH

In CA the ACK/NACK for all configured PDSCHs are by default carried on the PUCCH of the PCell. This corresponds to the multi-TRP configuration with single-PUCCH, i.e., all ACK/NACKs for the primary- and secondary-TRP’s PDSCH are carried on the PUCCH of the primary-TRP. In this case, only the ServingCell object representing the primary TRP must provide the uplink branch with the PUCCH-Config.

To enable the multi-PUCCH setup (ACK/NACK sent to the TRP transmitting the PDSCH), a PUCCH-Config could be provided also in the second ServingCell object that represents the second TRP. In this case the PUCCH-Configs may define PUCCH resources that differ in time domain to avoid power sharing issue and to enable easy distinction at the network side. Since the PUCCH-Config is in the same ServingCell object as the PDSCH-Config the UE would send the ACK/NACK for the PDSCH received from the Master-TRP on the PUCCH-Resource defined in the ServingCell object representing that master TRP. The PUCCH feedback for the PDSCH of the other TRP would be time multiplexed using the PUCCH resource configured in the PUCCH-Config of the other serving cell object (see existing field description of pucch-Cell).

In more complex configuration with several carriers (actual CA) served by several TRPs, one may want to carry the ACK/NACKs for all PDSCHs from one TRP on the PUCCH towards that same PUCCH. This is configurable in the same manner as the PUCCH-SCell, i.e., by setting the pucch-Cell field in PDSCH-ServingCellConfig to the ID of the serving cell object (TRP/SCell) with the PUCCH-Config to be used.

The existing PUCCH-Config allows single-PUCCH (all ACK/NACK to one TRP) or multi-PUCCH (ACK/NACK to the TRP where the PDSCH came from) including combinations of Multi-TRP and carrier aggregation.

In their LS, RAN1 requested the configuration of an additional index inside the CORESET configuration to enable transmission of ACK/NACK for the two PDSCHs on the same PUCCH. We do not see a need for such additional index considering that the PDCCH/PDSCHs of the different TRP are anyway distinguished by the ServCellId and bearing in mind that separate ACK/NACK codebooks are already used in CA in the same manner.

Providing an additional index in the ControlResourceSet for identifying ACK/NACK codebooks as requested by RAN1 seems unnecessary: If TRPs are represented by separate ServingCell configs they are anyway distinguishable by a ServCellId (like in CA)

### CSI-MeasConfig

The CSI-MeasConfig is used to configure the NZP-CSI-RS resources as well as the CSI-Reports. That means, it tells the UE what to measure and what to report. The CSI-ReportConfig tells the UE which measurements to report in which uplink resource. For CSI reporting on PUCCH, the CSI-ReportConfig points to a set of PUCCH resources defined in a PUCCH-Config. If the CSI report should be sent on the PUCCH of another serving cell, the CSI-ReportConfig indicates also the ServCellIndex of that ServingCell object in the field called “carrier” and the UE uses the PUCCH-Config therein.

The same configuration can be re-used for multi-TRP operation for scenarios with a single PUCCH and with multiple PUCCHs.

The existing CSI-MeasConfig allows configuring measurements on NZP-CSI-RS from the two TRPs and reporting thereof to either one TRP (single-PUCCH) or both TRPs (multi-PUCCH).

Based on the analysis in this section we propose the following:

Re-use the RRC signalling structure of Carrier Aggregation to configure a UE for Multi-TRP operation with multi-PDCCH/PDSCH, i.e., use ServingCell objects on the same carrier frequency to configure operation towards cooperating TRPs.

# 4	Conclusion

In the previous sections we made the following observations:

Observation 1	In this contribution we focus on the multi-PDCCH/PDSCH variant of multi-TRP operation.

Observation 2	A protocol structure with separate HARQ entities per TRP matches exactly what is well-established for carrier aggregation (one HARQ entity per serving cell; common RLC&MAC)

Observation 3	A protocol structure with separate HARQ entities per TRP allows network implementations where the HARQ entities are either in one “master TRP” or in the TRP that they serve.

Observation 4	A CA-like UP protocol architecture is primarily suitable for TRPs connected with ideal backhaul but latency up to a few ms could be feasible if congestion related losses are guaranteed not to occur on the inter-TRP connection.

Observation 5	Due to lack of a PUSCH from the UE to the secondary TRP it is not possible to apply the protocol stack of Dual Connectivity for multi-TRP operation.

Observation 6	If desirable, one may RRC-configure one primary TRP and several secondary TRPs and ensure by the existing MAC Activation/Deactivation Control Element that only one of the secondary TRPs is active at a time.

Observation 7	Two ServingCell objects with same SSB- and Point-A frequency and with the same SCS-SpecificCarrier configuration can be used to configure two TRPs.

Observation 8	The two ServingCell objects representing the two TRPs may have the same or different physCellId. This enables RAN1’s scenarios where the TRPs belong to the same or different cells.

Observation 9	Each ServingCell object must have a unique ServCellIndex (or SCellIndex).

Observation 10	The existing PDCCH-Config allows configuring the PDCCHs for the TRPs with independent CORESET- and SearchSpace configurations.

Observation 11	Instantiating separate ServingCell objects with separate PDCCH-Config:s per TRP avoids increasing the number of CORESETs per PDCCH-Config and thereby allows re-using existing RRC ASN.1 as is.

Observation 12	The existing PDSCH-Config allows configuring the PDSCHs for the TRPs with independent scrambling IDs, DMRS settings and TCI-States.

Observation 13	By configuring a ServingCell object per TRP, the lte-CRS-ToMatchAround can be configured independently per TRP.

Observation 14	The existing PUCCH-Config allows single-PUCCH (all ACK/NACK to one TRP) or multi-PUCCH (ACK/NACK to the TRP where the PDSCH came from) including combinations of Multi-TRP and carrier aggregation.

Observation 15	Providing an additional index in the ControlResourceSet for identifying ACK/NACK codebooks as requested by RAN1 seems unnecessary: If TRPs are represented by separate ServingCell configs they are anyway distinguishable by a ServCellId (like in CA)

Observation 16	The existing CSI-MeasConfig allows configuring measurements on NZP-CSI-RS from the two TRPs and reporting thereof to either one TRP (single-PUCCH) or both TRPs (multi-PUCCH).

Based on the discussion in the previous sections we propose the following:

Proposal 1	Re-use the User Plane protocols stack of Carrier Aggregation with one HARQ entity per TRP to realize the Multi-TRP operation with multi-PDCCH/PDSCH.

Proposal 2	Re-use the RRC signalling structure of Carrier Aggregation to configure a UE for Multi-TRP operation with multi-PDCCH/PDSCH, i.e., use ServingCell objects on the same carrier frequency to configure operation towards cooperating TRPs.

# 5	References

R1-1907966, LS on MIMO enhancement for NR, NR_eMIMO-Core, from RAN1, 2019-05-28

From RAN1 LS:_“At least for eMBB with M-DCI NCJT in order to generate different PDSCH scrambling sequences, support enhancing RRC configuration to configure multiple dataScramblingIdentityPDSCH”

From RAN1 LS:

•	For separate ACK/NACK feedback for PDSCHs received from different TRPs, the UE should be able to generate separate ACK/NACK codebooks identified by an index, if the index is configured and applied across all CCs.

o	The index to be used to generate separated ACK/NACK codebook is a higher layer signalling index per CORESET. Note that the index may not be configured for scenarios if there is no ambiguity of codebook generation at the UE, e.g. slot based PUCCH resource allocation per TRP. This does not preclude configuring the index for other purposes.