3GPP TSG-RAN WG2 #107	Tdoc R2-1911760

Prague, Czech Republic, 26th – 30th August 2019

Agenda Item:	10.4.1.9

Source:	Ericsson

Title:	Report of Offline discussion #039

Document for:	Discussion, Decision

# 1	Introduction

R2-1910055 Determining L2 buffer size  Ericsson        discussion

-     Samsung think we do need something for this data rate calculation. OK with P1 but not sure about the other proposals.

-     Ericsson think the principle is that the capabilities from the MN the SN is not expected to understand and vice versa.

-     Nokia have a different understanding. This is based on the peak L1 data rate possible in MN and SN and each node doesn't need to know the peak data rate of the other node by the UE buffer requirement is based on the 2 rates. Think as long as each node complies the total buffer size can't be exceeded.

-     DOCOMO think the Ericsson scenario is valid as PDCP entity one one noce is served by both nodes.

-     Huawei have similar understanding as Nokia as the total buffer size is based on the sum of the 2 data rates and hence nothing is broken.

=>                       Offline discussion to conclude if there is anything to solve (Offline discussion 39)

This document further addresses the discussion started above, based on TDoc R2-1910055.

# 2	Discussion

## 2.1	Relation of L1 data rate and L2 buffer size

Based on various capabilities reported by the UE, the network can determine the maximum L1 data rate, i.e., the amount of data that the gNB can send to the UE in one slot or in one ms.

To sustain downlink reception and uplink transmission at that L1 data rate while ensuring also in-sequence delivery to higher layers, the UE must implement a L2 (reordering) buffer.

If this buffer would be infinitely large, both nodes (SN and MN) could continue transmitting and receiving at the peak L1 data rate continuously. Even if HARQ has to perform several retransmissions and if RLC has to take over and perform several retransmissions, too, all packets would be received eventually and could be delivered in sequence towards the IP layer.

However, an infinite buffer isn’t feasible and therefore RAN2 estimated/agreed a maximum reordering depths which is considered sufficient most of the time. But there would still be occasions where the amount of L2 buffer is not sufficient to store all incoming packets while waiting for an old retransmission. This may not only happen if RLC/MAC need unusually many retransmission attempts. It happens also when the L1 data rate on one path is very high while the L1 data over the other path degrade unexpectedly. The packets queued in the “slow gNB” stall the lower PDCP window edge. At the same time the packets coming quickly from the other gNB advance the upper window edge very quickly.

If the amount of packets received out-of-order exceeds the UEs L2 buffer, the UE will drop some packets. It may either drop the oldest outstanding (at the lower RLC/PDCP window edge) or the newest incoming (at the upper RLC/PDCP window edge). If the UE is guaranteed to drop from the upper edge of the RLC window and to consider/report those PDUs as not received (NACK), the only consequence would be a bunch RLC retransmissions wasting radio resources. But if a UE drops previously ACKed packets or if it simply advances the lower window edge, the losses would become visible to the IP layer. The latter triggers TCP to slow down which harms the e2e performance significantly (in particular while operating at high data rates).

To avoid L2 buffer overflow in the UE’s RX buffer, the gNB’s PDCP TX entity could monitor the amount of outstanding (transmitted but not-yet-acknowledged) bytes and stop injecting more data into the two RLC entities as it reaches the UEs L2 buffer size limit. This requires of course that the PDCP entity knows the L2 buffer size (usable by this PDCP TX entity).

Transmitting at the L1 peak data rate for an unexpectedly long period of time may exceed the UEs L2 buffer size.

Depending on whether the UE drops only packets arriving at the upper windows edge or also those arriving at the lower window edge, packet loss will be visible to IP/TCP or not. Packet loss at IP/TCP level should preferably be avoided.

Dropping packets only at the upper window edge and NACKing them to the RLC transmitter will lead to unnecessary retransmissions, but in this way the UE could avoid IP packet loss. This could be OK assuming that it happens in rare occasions.

If the UE cannot ensure to avoid IP packet losses upon exceeding the L2 buffer, the network would need to know and enforce the L2 buffer size limit.

## 2.1	Computing the L2 buffer size in case of EN-DC

The L2 buffer size is “shared” across all PDCP entities established for one UE, and it must be used to limit the amount of data in flight to the UE and thereby avoid buffer overflow in the UE. However, for the cases of bearers terminated in both MN and SN, it is not clear how much of this L2 buffer size each node can use.

One could think that this is not an issue as long as both MN and SN stay within their maximum capacity for MCG and SCG receptively. However, as in the Figure below from 37.340 (for EN-DC), there is an issue for split bearers:

1) PDCP entities terminated in the MN with split bearers will not be able to determine how much data to push through since it cannot know the L2 buffer size accounting for the SCG leg, i.e., it can only compute its own L1 data rate;

2) PDCP entities terminated in the SN with split bearers will not be able to determine how much data to push through since it cannot know the L2 buffer size accounting for the MCG leg, i.e., it can only compute its own L1 data rate;

Figure 1: Network side protocol termination options for MCG, SCG and split bearers in MR-DC with EPC (EN-DC).

Therefore, the L2 buffer size should be known by both nodes as well as the corresponding share for each one.

To calculate this, according to 38.306, the following formula is used:

______________________________________________________________________________________

The required total layer 2 buffer size in MR-DC and NR-DC is the maximum value of the calculated values based on the following equations:

-	MaxULDataRate_MN * RLCRTT_MN + MaxULDataRate_SN * RLCRTT_SN + MaxDLDataRate_SN * RLCRTT_SN + MaxDLDataRate_MN * (RLCRTT_SN + X2/Xn delay + Queuing in SN)

-	MaxULDataRate_MN * RLCRTT_MN + MaxULDataRate_SN * RLCRTT_SN + MaxDLDataRate_MN * RLCRTT_MN + MaxDLDataRate_SN * (RLCRTT_MN + X2/Xn delay + Queuing in MN)

______________________________________________________________________________________

This MaxUL/DLDataRate defined in the formula is also defined in 38.306, as follows:

______________________________________________________________________________________

### 4.1.1	General

The DL and UL max data rate supported by the UE is calculated by band or band combinations supported by the UE. A UE supporting MR-DC shall support the calculated DL and UL max data rate defined in 4.1.2.

### 4.1.2	Supported max data rate

For NR, the approximate data rate for a given number of aggregated carriers in a band or band combination is computed as follows.

wherein

J is the number of aggregated component carriers in a band or band combination

Rmax = 948/1024

For the j-th CC,

is the maximum number of supported layers given by higher layer parameter maxNumberMIMO-LayersPDSCH for downlink and maximum of higher layer parameters maxNumberMIMO-LayersCB-PUSCH and maxNumberMIMO-LayersNonCB-PUSCH for uplink.

is the maximum supported modulation order given by higher layer parameter supportedModulationOrderDL for downlink and higher layer parameter supportedModulationOrderUL for uplink.

is the scaling factor given by higher layer parameter scalingFactor and can take the values 1, 0.8, 0.75, and 0.4.

is the numerology (as defined in TS 38.211 [6])

is the average OFDM symbol duration in a subframe for numerology , i.e. . Note that normal cyclic prefix is assumed.

is the maximum RB allocation in bandwidth  with numerology , as defined in 5.3 TS 38.101-1 [2] and 5.3 TS 38.101-2 [3], where  is the UE supported maximum bandwidth in the given band or band combination.

is the overhead and takes the following values

0.14, for frequency range FR1 for DL

0.18, for frequency range FR2 for DL

0.08, for frequency range FR1 for UL

0.10, for frequency range FR2 for UL

NOTE:	Only one of the UL or SUL carriers (the one with the higher data rate) is counted for a cell operating SUL.

______________________________________________________________________________________

As show above the data rate is computed based on a given band combination, which requires the knowledge of feature sets for E-UTRA (for bearers with MCG leg in EN-DC) and feature sets for NR (for bearers with SCG leg in EN-DC). However, the two nodes are not required to comprehend the other RAT's capability container. Hence, none of the two nodes can compute both the (EUTRA) MCG and (NR) SCG L1 data rates. And hence, none of the two nodes is able to compute the UEs total L2 buffer size.

MN and SN cannot compute both (MCG and SCG) L1 data rates. And hence, none of the two nodes can compute the UEs total L2 buffer size.

Question 1: Do companies agree with the Observation 1? Please provide details in case you do not agree.

| Company | Yes/No | Comments |
| --- | --- | --- |
| DOCOMO | Yes |  |
| Nokia | No | The buffer-dimensioning equations in 38.306 allow continuous transmission at L1 peak rates. The analysis seems to overlook flow-control feedback on X2-U. If a link rate suddenly deteriorates such that local queuing delays explode, the node can indicate “radio link outage” to the PDCP node, which in turn can do local retransmissions and/or slow down its rate. |
| Samsung | Yes |  |
| MediaTek | No | Agree with Nokia that the buffer calculation equations in 38.306 allow continuous transmission at L1 peak rates. |

As discussed in section 2.1, we believe that the network side needs to determine the UE’s L2 buffer size and it should ensure that the amount of data that is “in-flight” does not exceed this limit. However, other companies expressed the view that UEs L2 buffer will anyway suffice to avoid any buffer overflow or packet loss.

Question 2: Do companies think that the NW needs to know the UE’s L2 buffer size and ensure that the amount of data in-flight on PDCP level does not exceed this limit? If not, do companies agree that in this case the UE must ensure not to drop IP packets when exceeding its L2 buffer?

| Company | Yes/No | Comments |
| --- | --- | --- |
| DOCOMO | Yes | Especially, if both MN terminated DRB and SN terminated DRB are established for the same UE simultaneously. |
| Nokia | No |  |
| Samsung | Yes |  |
| MediaTek | No | The buffer calculaiton actually consdiers the worst case scenario that network can continously transmit at L1 peak rate without L2 feedback. It should be rare for UE to drop IP packets due to exceeding buffer. |

For companies that answered Yes to question 1 and 2 above (network must enforce L2 buffer size limit), a further question should be answered on how this L2 buffer size should be calculated. A way forward is: when the SN has selected a Band Combination (BC) and a corresponding Feature Set (FS) entry, it may compute the uplink and downlink L1 data rates that it may achieve on the SCG; if it would return these L1 data rate values to the MN, the MN could use it together with the L1 data rate values achievable with the same BC+FS at the MCG to compute the overall L2 buffer size.

Question 3: For companies that answered Yes to question 1 and 2 above, do you agree that the SN reports the l1DataRateUplink and l1DataRateDownlink for the selected BandCombination and FeatureSet to the CG-Config inter-node message? Please provide details in case you do not agree.

| Company | Yes/No | Comments |
| --- | --- | --- |
| DOCOMO | Yes | It’s a simple extension to iron out the issue w/o affecting Uu, although it is static configuration and so not so the perfect solution. If UE could report the outstanding data for each DRB to both MN and SN, both nodes would perfectly scheduling the data w/o incurring the buffer overflow. Of course, such the UE based solution is beyond the correction and future improvement. |
| Samsung | Yes | It seems at least some signaling for knowing the L1 data rate is need for split DRB cases, and the proposed way is the candidate option and seems feasible. |

# 3	Conclusion

It is proposed to have an offline discussion for the next meeting to further check whether there is any issue to be resolved related to L2 buffer size determination and usage.

# 4	References

R2-1910055  “Determining L2 buffer size”, Ericsson, RAN2-107, Prague, 26th – 30th August 2019

# 5	Annex - Text Proposal

## 5.1	One pair of L1 UL+DL Data rates

The following text proposal depicts how the SN could return one pair of L1 UL and DL data rates for the selectedBandCombinationNR.

#### –	CG-Config

This message is used to transfer the SCG radio configuration as generated by the SgNB or SeNB.

Direction: Secondary gNB or eNB to master gNB or eNB.

CG-Config message

-- ASN1START

-- TAG-CG-CONFIG-START

CG-Config ::=                   SEQUENCE {

criticalExtensions                  CHOICE {

c1                                  CHOICE{

cg-Config                   CG-Config-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture            SEQUENCE {}

}

}

CG-Config-IEs ::=           SEQUENCE {

scg-CellGroupConfig                 OCTET STRING (CONTAINING RRCReconfiguration)    OPTIONAL,

scg-RB-Config                       OCTET STRING (CONTAINING RadioBearerConfig)     OPTIONAL,

configRestrictModReq                ConfigRestrictModReqSCG                         OPTIONAL,

drx-InfoSCG                         DRX-Info                                        OPTIONAL,

candidateCellInfoListSN             OCTET STRING (CONTAINING MeasResultList2NR)     OPTIONAL,

measConfigSN                        MeasConfigSN                                    OPTIONAL,

selectedBandCombination             BandCombinationInfoSN                           OPTIONAL,

fr-InfoListSCG                      FR-InfoList                                     OPTIONAL,

candidateServingFreqListNR          CandidateServingFreqListNR                      OPTIONAL,

nonCriticalExtension                CG-Config-v1540-IEs                             OPTIONAL

}

CG-Config-v1540-IEs ::= SEQUENCE {

pSCellFrequency                     ARFCN-ValueNR                                   OPTIONAL,

reportCGI-RequestNR                 SEQUENCE {

requestedCellInfo                   SEQUENCE {

ssbFrequency                        ARFCN-ValueNR,

cellForWhichToReportCGI             PhysCellId

}                                                                               OPTIONAL

}                                                                                   OPTIONAL,

ph-InfoSCG                          PH-TypeListSCG                                  OPTIONAL,

nonCriticalExtension                CG-Config-v1560-IEs                             OPTIONAL

}

CG-Config-v1560-IEs ::= SEQUENCE {

pSCellFrequencyEUTRA                ARFCN-ValueEUTRA                                    OPTIONAL,

scg-CellGroupConfigEUTRA            OCTET STRING                                        OPTIONAL,

candidateCellInfoListSN-EUTRA       OCTET STRING                                        OPTIONAL,

candidateServingFreqListEUTRA       CandidateServingFreqListEUTRA                       OPTIONAL,

needForGaps                         ENUMERATED {true}                                   OPTIONAL,

drx-ConfigSCG                       DRX-Config                                          OPTIONAL,

reportCGI-RequestEUTRA              SEQUENCE {

requestedCellInfoEUTRA          SEQUENCE {

eutraFrequency                             ARFCN-ValueEUTRA,

cellForWhichToReportCGI-EUTRA              EUTRA-PhysCellId

}                                                                                   OPTIONAL

}                                                                                       OPTIONAL,

nonCriticalExtension                CG-Config-v15xy-IEs                                   OPTIONAL

}

CG-Config-v15xy-IEs ::=             SEQUENCE {

l1DataRateUplink                    INTEGER (1..20000000000)                        OPTIONAL,

l1DataRateDownlink                  INTEGER (1..20000000000)                        OPTIONAL,

nonCriticalExtension                SEQUENCE {}                                     OPTIONAL

}

PH-TypeListSCG ::=                      SEQUENCE (SIZE (1..maxNrofServingCells)) OF PH-InfoSCG

PH-InfoSCG ::=                          SEQUENCE {

servCellIndex                       ServCellIndex,

ph-Uplink                           PH-UplinkCarrierSCG,

ph-SupplementaryUplink              PH-UplinkCarrierSCG                             OPTIONAL,

...

}

PH-UplinkCarrierSCG ::=                 SEQUENCE{

ph-Type1or3                             ENUMERATED {type1, type3},

...

}

MeasConfigSN ::= SEQUENCE {

measuredFrequenciesSN               SEQUENCE (SIZE (1..maxMeasFreqsSN)) OF NR-FreqInfo  OPTIONAL,

...

}

NR-FreqInfo ::= SEQUENCE {

measuredFrequency                   ARFCN-ValueNR                                       OPTIONAL,

...

}

ConfigRestrictModReqSCG ::=         SEQUENCE {

requestedBC-MRDC                    BandCombinationInfoSN                               OPTIONAL,

requestedP-MaxFR1                   P-Max                                               OPTIONAL,

...,

[[

requestedPDCCH-BlindDetectionSCG    INTEGER (1..15)                                     OPTIONAL,

requestedP-MaxEUTRA                 P-Max                                               OPTIONAL

]]

}

BandCombinationIndex ::= INTEGER (1..maxBandComb)

BandCombinationInfoSN ::=   SEQUENCE {

bandCombinationIndex                BandCombinationIndex,

requestedFeatureSets                FeatureSetEntryIndex

}

FR-InfoList ::= SEQUENCE (SIZE (1..maxNrofServingCells-1)) OF FR-Info

FR-Info ::= SEQUENCE {

servCellIndex       ServCellIndex,

fr-Type             ENUMERATED {fr1, fr2}

}

CandidateServingFreqListNR ::= SEQUENCE (SIZE (1.. maxFreqIDC-MRDC)) OF ARFCN-ValueNR

CandidateServingFreqListEUTRA ::= SEQUENCE (SIZE (1.. maxFreqIDC-MRDC)) OF ARFCN-ValueEUTRA

-- TAG-CG-CONFIG-STOP

-- ASN1STOP

CG-Config field descriptions

candidateCellInfoListSN Contains information regarding cells that the source secondary node suggests the target secondary gNB to consider configuring.

candidateCellInfoListSN-EUTRA Includes the MeasResultList3EUTRA as specified in TS 36.331 [10]. Contains information regarding cells that the source secondary node suggests the target secondary eNB to consider configuring. This field is only used in NE-DC.

candidateServingFreqListNR, candidateServingFreqListEUTRA Indicates frequencies of candidate serving cells for In-Device Co-existence Indication (see TS 36.331 [10]).

drx-ConfigSCG This field contains the complete DRX configuration of the SCG. This field is only used in NR-DC.

drx-InfoSCG This field contains the DRX long and short cycle configuration of the SCG. This field is used in (NG)EN-DC and NE-DC.

configRestrictModReq Used by SN to request changes to SCG configuration restrictions previously set by MN to ensure UE capabilities are respected. E.g. can be used to request configuring an NR band combination whose use MN has previously forbidden.

fr-InfoListSCG Contains information of FR information of serving cells that include PScell and Scells configured in SCG.

l1DataRateDownlink The maximum L1 downlink data rate in bit/s that the UE is able to achieve on its SCG serving cells according to the selected selectedBandCombinationNR.

l1DataRateUplink The maximum L1 uplink data rate that in bit/s the UE is able to achieve on its SCG serving cells according to the selected selectedBandCombinationNR.

needForGaps In NE-DC, indicates wheter the SN requests gNB to configure measurements gaps.

measuredFrequenciesSN Used by SN to indicate a list of frequencies measured by the UE.

ph-InfoSCG Power headroom information in SCG that is needed in the reception of PHR MAC CE of MCG

ph-SupplementaryUplink Power headroom information for supplementary uplink. In the case of (NG)EN-DC and NR-DC, this field is only present when two UL carriers are configued for a serving cell and one UL carrier reports type1 PH while the other reports type 3 PH.

ph-Type1or3 Type of power headroom for a certain serving cell in SCG (PSCell and activated SCells). Value type1 refers to type 1 power headroom, value type3 refers to type 3 power headroom. (See TS 38.321 [3]).

ph-Uplink Power headroom information for uplink.

pSCellFrequency, pSCellFrequencyEUTRA Indicates the frequency of PSCell in NR (i.e., pSCellFrequency) or EUTRA (i.e., pSCellFrequencyEUTRA). In this version of the specification, pSCellFrequency is not used in NE-DC whereas pSCellFrequencyEUTRA is only used in NE-DC.

reportCGI-RequestNR, reportCGI-RequestEUTRA Used by SN to indicate to MN about configuring reportCGI procedure. The request may optionally contain information about the cell for which SN intends to configure reportCGI procedure. In this version of the specification, the reportCGI-RequestNR is used in (NG)EN-DC and NR-DC whereas reportCGI-RequestEUTRA is used only for NE-DC.

requestedPDCCH-BlindDetectionSCG Requested value of the reference number of cells for PDCCH blind detection allowed to be configured for the SCG.

requestedP-MaxEUTRA Requested valume for the maximu power for the serving cells the UE can use in E-UTRA SCG. This field is only used in NE-DC.

requestedP-MaxFR1 Requested value for the maximum power for the serving cells on frequency range 1 (FR1) in this secondary cell group (see TS 38.104 [12]) the UE can use in NR SCG.

requestedBC-MRDC Used to request configuring an NR band combination and corresponding feature sets which are forbidden to use by MN.

scg-CellGroupConfig Contains the RRCReconfiguration message: - to be sent to the UE, used upon SCG establishment or modification, as generated (entirely) by the (target) SgNB. In this case, the SN sets the RRCReconfiguration message in accordance with clause 6 e.g. regarding the "Need" or "Cond" statements. or - including the current SCG configuration of the UE, when provided in response to a query from MN, or in SN triggered SN change in order to enable delta signaling by the target SN. In this case, the SN sets the RRCReconfiguration message in accordance with clause 11.2.3. The field is absent if neither SCG (re)configuration nor SCG configuration query nor SN triggered SN change is performed, e.g. at inter-node capability/configuration coordination which does not result in SCG (re)configuration towards the UE. This field is not applicable in NE-DC.

scg-CellGroupConfigEUTRA Includes the E-UTRA RRCConnectionReconfiguration message as specified in TS 36.331 [10]. In this version of the specification, the E-UTRA RRC message can only include the field scg-Configuration. Used to (re-)configure the SCG configuration upon SCG establishment or modification, as generated (entirely) by the (target) SeNB. This field is absent when master gNB uses full configuration option. This field is only used in NE-DC.

scg-RB-Config Contains the IE RadioBearerConfig: - to be sent to the UE, used to (re-)configure the SCG RB configuration upon SCG establishment or modification, as generated (entirely) by the (target) SgNB or SeNB. In this case, the SN sets the RadioBearerConfig in accordance with section 6, e.g. regarding the "Need" or "Cond" statements. or - including the current SCG RB configuration of the UE, when provided in response to a query from MN or in SN triggered SN change in order to enable delta signaling by the target SN. In this case, the SN sets the RRCReconfiguration message in accordance with section 11.2.3. The field is absent if neither SCG (re)configuration nor SCG configuration query nor SN triggered SN change is performed, e.g. at inter-node capability/configuration coordination which does not result in SCG RB (re)configuration.

selectedBandCombination Indicates the band combination selected by SN for the (NG)EN-DC, NE-DC, and NR-DC.

BandCombinationInfoSN field descriptions

bandCombinationIndex The position of a band combination in the supportedBandCombinationList

requestedFeatureSets The position in the FeatureSetCombination which identifies one FeatureSetUplink/Downlink for each band entry in the associated band combination