3GPP TSG-SA WG2 Meeting #136	S2-1912379

Reno, USA, 18-22 November 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1798 | rev | 2 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of NG-RAN sharing options for NPN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Qualcomm Incorporated, NICT, Deutsche Telekom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2019-11-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | TS 23.501 states that 5G MOCN supports RAN sharing by multiple SNPNs, and RAN shared by one or multiple SNPNs and one or multiple PLMNs. For other combination of sharing SA2 asked for RAN WG feedback and RAN2 replied in S2-1908685 and RAN3 in S2-1908695 basically stating that all options are possible with the following clarification from RAN3: " logical cell (i.e. defined by a cell identity) would be associated with a single access mode (i.e. a logical cell would either be part of a public or a private network, and in the latter case, support PNI-NPN, or SNPN). In this case, a physical cell may host several logical cells, and therefore may support different network types (and therefore sharing of the physical resource). " With such principle all the sharing combinations SA2 asked for can be supported and it is therefore proposed to move forward with supporting all combinations. The support for Network Sharing with multiple cell-ID broadcast is described in TS 38.300 and TS 38.401. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Support for following NG-RAN sharing options are added - RAN sharing between a PNI-NPN (with CAG) and an SNPN - RAN sharing between a PLMN and a PNI-NPN with CAG And existing NG-RAN sharing option for sharing between a PLMN and an SNPN is clarified. - Clarify that CAG Identifiers may be in Broadcast system information Rev2: - Aligned Cell ID to Cell Identity |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | All applicable NG-RAN sharing options will not be supported for NPN type of deployments. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.18.1, 5.18.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev "-": S2-1909370 Rev "1": S2-1911028 |  |  |  |  |  |  |  |  |

*** BEGIN CHANGES ***

## 5.18	Network Sharing

### 5.18.1	General concepts

A network sharing architecture shall allow multiple participating operators to share resources of a single shared network according to agreed allocation schemes. The shared network includes a radio access network. The shared resources include radio resources.

The shared network operator allocates shared resources to the participating operators based on their planned and current needs and according to service level agreements.

In this Release of the specification, only the 5G Multi-Operator Core Network (5G MOCN) network sharing architecture, in which only the RAN is shared in 5G System, is supported. 5G MOCN for 5G System, including UE, RAN and AMF, shall support operators' ability to use more than one PLMN ID (i.e. with same or different country code (MCC) some of which is specified in TS 23.122 [17] and different network codes (MNC)). 5G MOCN supports NG-RAN Sharing with or without multiple Cell Identity broadcast as described in TS 38.300 [27].

5G MOCN also supports the following sharing scenarios involving non-public networks:

-	NG-RAN is shared by multiple SNPNs (each identified by PLMN ID and NID);

-	NG-RAN is shared by one or multiple SNPNs and one or multiple PLMNs;

-	NG-RAN is shared by one or more PNI-NPNs (with CAG) and one or more SNPNs, and

-	NG-RAN is shared by one or multiple PLMNs and one or multiple PNI-NPNs (with CAG).

NOTE 1:	PNI-NPNs (without CAG) are not explicitly listed above as it does not require additional NG-RAN sharing functionality compared to sharing by one or multiple PLMNs.

In all non-public network sharing scenarios, each Cell Identity is associated with one of the following configuration options:

-	one or multiple SNPNs;

-	one or multiple PNI-NPNs (with CAG); or

-	one or multiple PLMNs only.

NOTE 2:	Different PLMN IDs (or combinations of PLMN ID and NID) can also point to the same 5GC.

NOTE 3:	There is no standardized mechanism to avoid paging collisions if the same 5G-S-TMSI is allocated to different UEs by different PLMNs or SNPNs of the shared network, as the risk of paging collision is assumed to be very low. If such risk is to be eliminated then PLMNs and SNPNs of the shared network needs to coordinate the value space of the 5G-S-TMSI to differentiate the PLMNs and SNPNs of the shared network.

Figure 5.18.1-1: A 5G Multi-Operator Core Network (5G MOCN) in which multiple CNs are 
connected to the same NG-RAN

### 5.18.2	Broadcast system information for network sharing

If a shared NG-RAN is configured to indicate available networks (PLMNs and/or SNPNs) for selection by UEs, each cell in the shared radio access network shall in the broadcast system information include available core network operators in the shared network.

The Broadcast System Information broadcasts a set of PLMN IDs and/or PLMN IDs and NIDs and one or more additional set of parameters per PLMN e.g. cell-ID, Tracking Areas, CAG Identifiers. All 5G System capable UEs that connect to NG-RAN support reception of multiple PLMN IDs and per PLMN specific parameters. All SNPN-enabled UEs support reception of multiple combinations of PLMN ID and NID and SNPN-specific parameters.

The available core network operators (PLMNs and/or SNPNs) shall be the same for all cells of a Tracking Area in a shared NG-RAN network.

UEs not set to operate in SNPN access mode decode the broadcast system information and take the information concerning available PLMN IDs into account in PLMN and cell (re-)selection procedures. UEs set to operate in SNPN access mode decode the broadcast system information and take the information concerning available PLMN IDs and NIDs into account in network and cell (re-)selection procedures. Broadcast system information is specified in TS 38.331 [28] for NR, TS 36.331 [51] for E-UTRA and related UE access stratum idle mode procedures in TS 38.304 [50] for NR and TS 36.304 [52] for E-UTRA.

*** END OF CHANGES ***