3GPP TSG RAN WG2#109-e	R2-2000086

Electronic meeting, 24th February - 6th March, 2020

3GPP TSG-RAN WG1 Meeting #99		R1-1913696

Reno, USA, November 18th – 22nd, 2019

Title:	Reply LS on sidelink synchronization under multiple synchronization sources with different timing

Response to:	R1-1913588 (R2-1916465)

Release:	Rel-16

Work Item:	5G_V2X_NRSL-Core

Source:	RAN1

To:	RAN2, RAN4

Cc:

Contact Person:

# Name:	               Sudhir Baghel

E-mail Address:	sudhirb@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:    -

# 1. Overall Description:

RAN1 would like to thank RAN2 for the LS R1-1913588 regarding sidelink synchronization under multiple synchronization sources with different timing. For the following question to RAN1:

  - Question: RAN1 to provide guidance on the async RAN nodes (including eNB and gNB) issue and whether/what RAN2 signalling is required with respect to following scenarios:

- UE1 and UE2 synced to two different gNBs, but the 2 gNBs are with different timing - UE1 and UE2 synced to two different eNBs, but the 2 eNBs are with different timing - UE1 synced to eNB, UE2 synced to gNB, eNB and gNB are with different timing

RAN1 has made following agreement in RAN1#98Bis:

- The procedure for signalling, identifying priority for one or more synchronization references and selecting the synchronization reference from the LTE is re-used (as a working assumption) for NR SL

It is RAN1 understanding that asynchronous deployment scenarios also existed in LTE-V2X. RAN1 believes that the “offsetDFN” parameter used in LTE-V2X is also necessary in NR-V2X for aligning sidelink timing in some scenarios when GNSS is used as high priority synchronization source.

RAN1 has not discussed all details of the scenarios mentioned above and based on the working assumption there is no specific signaling support for timing adjustment in these scenarios.

# 2. Actions:

To RAN WG2

RAN1 respectfully asks RAN2 to take the above information into account, and similar to LTE-V2X include “offsetDFN” in their specifications for NR-V2X.

To RAN WG4

RAN1 respectfully asks RAN4 to take the above information into account in their work.

# 3. Date of Next RAN1 Meetings:

TSG RAN WG1 Meeting #100			24 - 28 February 2020        				     Athens, Greece

TSG RAN WG1 Meeting #100bis			20 - 24 April 2020       	 				     Busan, Korea