3GPP TSG RAN WG2#109-e	R2-2002363

Electronic meeting, 24th February - 6th March, 2020

3GPP TSG RAN WG1 #100e			R1-2001375

e-Meeting, February 24th – March 6th, 2020

Title:	LS on NR-U enhancements to initial access procedures

Release:	Rel-16

Work Item:	NR_unlic-core

Source:	RAN WG1

To:	RAN WG2

Cc:	-

Contact Person:

Name:	Amitav Mukherjee

E-mail Address:	Amitav_dot_Mukherjee_at_charter_dot_com

1. Overall Description:

During RAN1#100-e, RAN WG1 has made the following agreements related to RRM and random access procedure enhancements for NR-U:

Agreement:

The RMTC-Config can indicate a reference SCS and CP as one of {15 kHz, 30 kHz, 60 kHz-NCP, 60 kHz-ECP}

Agreement:

The L1 averaging duration of RSSI measurements (within a configured measurement duration) is limited to 1 OFDM symbol of a configured reference subcarrier spacing.

Agreement:

Keep value range for rmtc-Period-r16 and rmtc-SubframeOffset-r16 the same as for LTE-LAA.

Agreement:

Keep value range for rssi-Result-r16 and channelOccupancyThreshold-r16 the same as for LTE-LAA and inform RAN2 of this decision.

Agreement:

For RSSI (and channel occupancy measurements) layer 3 filtering is not applied.

Agreement:

LBT parameter applied for HARQ-ACK transmission corresponding to Msg. B reception is provided via each success RAR in Msg. B PDSCH.

2. Actions:

To RAN WG2.

ACTION: RAN WG1 kindly asks RAN WG2 to take above agreements for developing their specifications.

3. Date of Next TSG-RAN WG1 Meetings:

TSG-WG1 Meeting #100-e-bis 	20th – 24th Apr 2020		Online

TSG-WG1 Meeting #101			25th – 29th May 2020		Athens, GR