3GPP TSG-RAN WG2 Meeting #109bis-e	R2-20XXXXX

- Electronic, 20 April – 30 April 2020

Agenda item:	5.4.1.1

Source:	Samsung, ZTE Corporation, Sanechips

Title:	Offline-005: L2 Configuration

Document for:	Discussion & Decision

# Introduction

This is a summary of the following offline discussion on L2 parameters and configuration:

- [AT109bis-e][006][NR15] L2 Configuration (Samsung, ZTE)

Scope: Treat R2-2002917, R2-2002948, R2-2002949, R2-2002886

Part 1: Determine which issues that need resolution, find agreeable proposals. Deadline: April 23 0700 UTC

Part 2: For the parts that are agreeable, discussion will continue to agree on CRs

This document covers the following contributions submitted to RAN2#109bis-e meeting:

R2-2002917	Clarification on the presence of ssb-perRACH-Occasion for the CSI-RS based CFRA	ZTE Corporation, Sanechips, Ericsson (Rapporteur)	CR	Rel-15	38.331	15.9.0	1449	1	F	NR_newRAT-Core	R2-2000664

R2-2002948	Change of pdcp-Duplication at RRC Reconfiguration	Samsung	discussion	Rel-15	NR_newRAT-Core

R2-2002949	Clarification on pdcp-Duplication at RRC Reconfiguration	Samsung	CR	Rel-15	38.331	15.9.0	1534	-	F	NR_newRAT-Core

R2-2002886	Corrections on the allowedSCS-List and AllowedServingCells in LogicalChannelConfig	Samsung	CR	Rel-15	38.331	15.9.0	1532	-	F	NR_newRAT-Core

Companies are invited to provide their views for each issue.

# Discussion: Part 1

## 2.1 Issue #1. Presence of ssb-perRACH-Occasion for the CSI-RS based CFRA

The first issue from R2-2002917 is about presence condition of ssb-perRACH-Occasion.

In the current TS38.331, the field ssb-perRACH-Occasion is mandatory present if the field resources in CFRA is set to ssb; otherwise it is absent, which means this field is absent when the field resources in CFRA is set to csirs.

However, according to the following description in TS38.213, the ssb-perRACH-Occasion is still needed for UE to decide the association pattern period and understand the index of the PRACH occasions indicated by ra-OccasionList.

------------------------------------------------------From TS38.213- start----------------------------------------------------

An association pattern period includes one or more association periods and is determined so that a pattern between PRACH occasions and SS/PBCH blocks repeats at most every 160 msec.

.....

For a PRACH transmission triggered upon request by higher layers, a value of ra-OccasionList [12, TS 38.331], if csirs-ResourceList is provided, indicates a list of PRACH occasions for the PRACH transmission where the PRACH occasions are associated with the selected CSI-RS index indicated by csi-RS. The indexing of the PRACH occasions indicated by ra-OccasionList is reset per association pattern period.

------------------------------------------------------From TS38.213- end-----------------------------------------------------

Thus, it is suggested in R2-2002917 to change the presence condition of ssb-perRACH-Occasion and this field should be mandatory present when the field resources in CFRA is set to ssb or csirs.

Q1) Do companies agree that ssb-perRACH-Occasion is mandatorily present?

| Company | Yes/No | Comments (if any) |
| --- | --- | --- |
| Huawei, Hisilicon | Double check needed | Simply changing the presence condition from “conditional absent” to “mandatory” may introduce NBC issues, as the old UE may not store the parameter signalled by the new gNB, and may assume some other parameter used in this case. |
| Ericsson | Yes |  |
| Nokia | Maybe not | We don't fully understand the reason for change - 213 does not mention anywhere the ssb-perRACH-Occasion parameter would be needed to derive the RACH occasions (ROs) with parameter ra-OccasionList. In principle, each RO is an individual RO regardless of how many SSBs is mapped to it and, hence, the indexing with ra-OccasionList should work without the other parameter. We think this should be checked with RAN1 if this is the case as this is out of RAN2 scope. |
| CATT |  | After checking we tend to agree that there is an issue. Basically, RAN1 spec is so that the association pattern period relates to PRACH occasions and SS/PBCH blocks. So for the case when resources is set to csirs ssb-perRACH-Occasion may still be needed. We can discuss further how to fix this issue. Huawei has a point that this is going to be NBC. |

## 2.2 Issue #2. Change of pdcp-Duplication at RRC Reconfiguration

The next issue is whether the value of pdcp-Duplication IE in PDCP-config can be changed or not. This issue was discussed in RAN2#108 but postponed without any conclusion. Since this issue is related with Rel-16 IIOT discussion on initial state. A contribution R2-2002948 and corresponding CR R2-2002949 are proposing to have a decision.

In the contribution, Samsung sees that value of pdcp-Duplication can be changed due to the following reasons:

- Current specifications does not prohibit but seems to allow the change.

  - In MAC specification, it is explicitly indicated that “The PDCP duplication for the configured DRB(s) is activated and deactivated by indication by RRC.” Although the original intention was initial state, the description does not say anything about in the current text.

  - In PDCP specification, there is no expression on activation by “lower layer” or “MAC CE”. Our understanding is that since activation/deactivation by RRC is possible, lower layer indication cannot be specified here.

  - In RRC specification, the field description of pdcp-Duplication explicit says that it is uplink duplication status “at the time of receiving this IE”, not configuration of this field. The field description also mentions that the value indicates “the initial state of the duplication.” In sum, we could interpret as initial state at the time of receiving the IE.

- UE behaviour at reconfiguration is not clear if the initial value cannot be changed.

  - Assuming current duplication status is different from the initial state and UE receives the reconfiguration, it is not clear if UE falls back to the original initial state or ignore it. Both ways are not efficient.

    - If UE falls back to the original initial state, gNB may need to send a MAC CE immediately after the configuration, for NW’s intended duplication. The data transmission with duplication might be delayed. This case can happen frequently in handover scenario.

    - If UE ignores the initial state, gNB should include pdcp-Duplication set to the original configuration which is not used by UE at all. There will be a meaningless signalling.

The corresponding CR proposes the following changes:

pdcp-Duplication Indicates whether or not uplink duplication status at the time of receiving this IE is configured and activated as specified in TS 38.323 [5]. The presence of this field indicates that duplication is configured. PDCP duplication is not configured for CA packet duplication of LTE RLC bearer. The value of this field, when the field is present, indicates the state of the duplication at the time of receiving this IE. If set to true, duplication is activated. The value of this field is always true, when configured for a SRB.

Note that detailed wording can be improved during part 2 discusison. .

Q2) Do companies agree that the value of pdcp-Duplication can be changed by RRC reconfiguration?

| Company | Yes/No | Comments (if any) |
| --- | --- | --- |
| Huawei, Hisilicon | Yes | We assume the existing wording in RRC already allows change/reconfiguration of this parameters, but also fine to clarify this understanding. |
| Ericsson | No | In RAN2-NR#2 meeting, the following agreements were made regarding PDCP duplication: - MAC CE enables per DRB control of activation/deactivation of packet duplication for DRBs with packet duplication configured by RRC. Further, in RAN2 AH1801, the following agreements were made: - When configuring duplication, RRC can also set the initial state (active or inactive) for DRBs. - If SRB is configured to use duplication, the state is always active This it means that the RRC configures the initial state of the PDCP duplication that in case of SRB is always “activated” and in case of DRB can be “activated” or “deactivated”. Then, in case of DRB the MAC CE is used to control the PDCP duplication state (that is NOT the initial state) and switch it between inactive or active. Given this, in the field description of pdcp-duplication in 38.331, is clearly state that “The value of this field, when the field is present, indicates the initial state of the duplication.” Therefore, the current RRC specification is in line with the agreement and nothing needs to be clarified. In top of this, what Samsung is proposing here is a big NBC that should not be pursued at this stage of Rel-15. |
| Nokia | Yes | - We think the "initial state" always refers to the case when UE receives the IE and hence would say this was already allowed. - We don't recall a discussion the state would not be allowed to be changed by RRC, e.g., upon duplication reconfiguration. Or why would it be disallowed? - Hence we think this is intended by the current description and how it should be, at least for HO case. Agree also with Huawei that the CR from Samsung seems fine and makes it clearer. |

## 2.3 Issue #3. Corrections on the allowedSCS-List and allowedServingCells in LogicalChannelConfig

According to the field descriptions of allowedSCS-List/ServingCells, UE apples any configured numerologies/serving cells if this field is absent, but the current need code of both fields is Need R (i.e. UE release this field if this field is absent). However, MAC specification describe the UE behaviour when they are not configured (or released).

TS 38.321 5.4.3.1.2 Selection of logical channels The MAC entity shall, when a new transmission is performed: 1> select the logical channels for each UL grant that satisfy all the following conditions: 2> the set of allowed Subcarrier Spacing index values in allowedSCS-List, if configured, includes the Subcarrier Spacing index associated to the UL grant; and 2> maxPUSCH-Duration, if configured, is larger than or equal to the PUSCH transmission duration associated to the UL grant; and 2> configuredGrantType1Allowed, if configured, is set to true in case the UL grant is a Configured Grant Type 1; and 2> allowedServingCells, if configured, includes the Cell information associated to the UL grant. Does not apply to logical channels associated with a DRB configured with PDCP duplication within the same MAC entity (i.e. CA duplication) for which PDCP duplication is deactivated.

What UE should do when the field is not configured, is a separate aspects i.e. it should not affect the need code. There are 2 cases in which UE can end up with field not being configured:

a) not configured so far,

b) configured initially and later absent/ released.

The need code is relevant for b), but it is not exactly explained based on the UE behaviour in MAC specification. The proposal of a CR R2-2002886 is to align the text in the field description to MAC specification.

allowedSCS-List If configured, UL MAC SDUs from this logical channel can only be mapped to the indicated numerology. Otherwise, UL MAC SDUs from this logical channel can be mapped to any configured numerology. Only the values 15/30/60 kHz (for FR1) and 60/120 kHz (for FR2) are applicable. Corresponds to 'allowedSCS-List' as specified in TS 38.321 [3].

allowedServingCells If configured, UL MAC SDUs from this logical channel can only be mapped to the serving cells indicated in this list. Otherwise, UL MAC SDUs from this logical channel can be mapped to any configured serving cell of this cell group. Corresponds to 'allowedServingCells' in TS 38.321 [3].

Q3) Do companies agree the TP above, i.e. CR R2-2002886?

| Company | Yes/No | Comments (if any) |
| --- | --- | --- |
| Huawei, Hisilicon | Maybe No | We think “if present” is appropriate for “Need R” fields in RRC spec, and it is ok for other specs (e.g. MAC or PHY) using “if configured”. We don’t see an ambiguity here. |
| Ericsson | No | In general, we think there is no room for any of such misunderstanding at this stage of Rel-15. Therefore, strictly speaking this CR is not needed. Further, we note that this is related to some TEI16 issues addressed in see R2-2002741. Therefore, if companies are eager to have such change, we would be ok to keep this on hold for now until the TEI16 CR is handled. Yes, this is Rel-15 and TEI16 is Rel-16, but this would need a shadow for Rel-16 and then it could become messy. |
| Nokia | No | Fully agree with Ericsson and Huawei. Samsung seems to have confused the understanding of the field description with an already configured value. Hence no need for the CR in our view. |
| CATT | No | In our understanding nothing is broken with the current field description and texts in MAC. |

# Conclusion: Part 1

Based on the above, RAN2 is request to agree the following proposals:

TBD