3GPP TSG-RAN WG2 Meeting #109bis electronic	R2-200xxxx

20th – 24th April, 2020

Agenda item:       5.4.1.2

Source: 	Nokia, Nokia Shanghai Bell, Huawei, HiSilicon

Title: 	Summary of	[AT109bis-e][010][NR15] Measurements (Huawei, Nokia)

Document for:		Discussion and Decision

- 1.	Introduction

This is a summary of offline discussion for the following documents:

- [AT109bis-e][010][NR15] Measurements (Huawei, Nokia)

Scope: Treat all docs under AI 5.4.1.2

Part 1: Determine which issues that need resolution, find agreeable proposals. Deadline: April 23 0700 UTC (chair comment: expect R2-2002692 and 2693 to be easy agreements as we already have agreed them).

Part 2: For the parts that are agreeable, discussion will continue to agree on CRs.

All CRs under AI 5.4.1.2 are covered: R2-2002692, R2-2002693, R2-2003701, R2-2003702, R2-2003734, R2-2003735

- 2. Discussion

## 2.1	R2-2002692 and R2-2002693 Clarification for SSB-ToMeasure

Reason for change: The way UE should use the IE SSB-ToMeasure is not clear in the RRC specification. A clarification to this was agreed during RAN2108 but is not reflected in the specifications. During RAN2109e the principle of this CR was also deemed agreeable as per R2-2002155 but a revision to the original CR was never submitted to RAN2109e due to mistake.

Q) Do companies agree with the changes in the CR R2-2002692 (Rel-15) and R2-2002693 (Rel-16)?

| Company | Yes/No | Comments (if any) |
| --- | --- | --- |
| Nokia | Yes | Proponent |
| Qualcomm | No | In RAN2#108, this issue has been discussed. And the conclusion is that it is common understanding to use absolute SSB index which doesn’t have any ambiguity, and thereby CR is not required (Chair has concluded “not pursued”). We should respect previous conclusion. R2-1915425 CR to 36.331 on SSB-ToMeasure clarifications Ericsson CR Rel-15 36.331 15.7.0 4147 - F NR_newRAT-Core DISCUSSION - Huawei think CR is not needed. - MTK think we can agree P1 but think CR is not needed. QC can also agree P1 but think the CRs are not correct. - QC think that “within SMTC duration” is R1 text and is ok because if SMTC is not configured, the UE will anyway have a default assumption. - RAN2 confirms that the SSB-ToMeasure is based on the absolute SSBIndexes. - Not pursued |
| ZTE | prefer Yes | The intention is correct, and the wording from CR is correct. We understand this was discussed before, but to avoid continuously clarification from companies in the future, maybe it is ok to capture it clearly in spec. |

## 2.2	R2-2003701 and R2-2003702 Correction to inter-RAT SFTD measurements

Reason for change: "Reason for change: According to 38.331 SFTD measurement configured by NR PCell will be triggered as long as UE has a measId associated to a reportConfig with reportType set to reportSFTD: TS 38.331 cl. 5.5.3.1 1 for each measId included in the measIdList within VarMeasConfig: 2 if the reportType for the associated reportConfig is set to reportSFTD: 3 if the reportSFTD-Meas is set to true: 5 perform SFTD measurements between the PCell and the NR PSCell; However Unlike LTE autonomous measId removal is not allowed in NR. So UE will perform SFTD measurements on and on even after UE already sends a measurement report. It is nessesary to clarify that UE only performs SFTD measurement when no measurement report is sent. Similar to CGI measurement a timer (T322) is also used for DRX idle period based SFTD measurement. RAN2 has already agreed that CGI measurement is only performed when T321 is running. It shall be the same for SFTD measurement. There is a typo in the description of IE reportRSRP. In current version of 38.331 it says: reportRSRP Indicates whether UE is required to include RSRP result of NR PSCell in SFTD measurement result derived based on SSB. If it is set to true the network should ensure that ssb-ConfigMobility is included in the measurement object for NR PSCell. Since reportRSRP is also used when reportSFTD-NeighMeas TRUE (i.e. UE performs SFTD on neighbour cells rather than PSCell) the description shall be changed to include RSRP result of NR PSCell or neighbour cells in SFTD measurement result ...".

Q) Do companies agree with the changes in the CR R2-2003701 (Rel-15) and R2-2003702 (Rel-16)?

| Company | Yes/No | Comments (if any) |
| --- | --- | --- |
| Nokia | Yes | The CR is OK to clarify when the SFTD measurement should be performed. |
| Qualcomm | Yes | 1st change is good clarification that SFTD measurement is one-shot 2nd change is similar to CGI reporting. Thus, it is fine |
| ZTE | Yes | Seems we use different ways for SFTD and normal periodical measurement. For periodical measurement, the UE will autonomously remove the measID when numberOfReportsSent equals reportAmount. While for SFTD, the network has to explicitly release the corresponding measID. |

## 2.3	R2-2003734 and R2-2003735 Correction to inter-RAT SFTD measurements

Reason for change: "Reason for change: According to 36.133 inter-RAT SFTD measurement configured on neighbours cells shall be terminated upon NR PSCell is added. TS 36.113 cl. 8.1.2.4.25 In case an NR PSCell is added the UE shall terminate the inter-RAT SFTD measurement. However measId of inter-RAT SFTD measurement isnt autonumously removed after NR PSCell addition. As a result if an inter-RAT SFTD measurement doesnt trigger measurement reporting before PSCell addition the UE has to perform inter-RAT SFTD measurement after entering EN-DC since performing SFTD measurements is not relevent to PSCell addition. It conflicts with 38.133. TS 36.331 cl. 5.5.3.1 4 if the reportSFTD-Meas is set to neighborCells in the associated reportConfig: 5 perform SFTD measurements between the PCell and NR cell(s) on the frequency indicated in the associated measObject; So it is nessesary to autonumously remove measId of SFTD with reportSFTD-Meas set to neighborCells after NR PSCell addition."

Q) Do companies agree with the changes in the CR R2-2003734  (Rel-15) and R2-2003735  (Rel-16)?

| Company | Yes/No | Comments (if any) |
| --- | --- | --- |
| Nokia | Yes | The CR is OK to clarify when the SFTD measurement should be performed. |
| Qualcomm | Yes with comments | Basically, we think this CR is fine. But NOTE 1 seems to also need to be corrected accordingly: "NOTE 1: The above UE autonomous removal of measId's applies only for measurement events A1, A2, A6, and also applies for events A3 and A5 if configured for PSCell and W2 and W3 and V1 and V2 and event involving reportSFTD-Meas set to pSCell, if configured.” |
| ZTE | Yes with comments | We are fine to use “autonomous measId removal” in this case, just curious whether it causes NBC problem? It assumes all UEs have already implemented like this, so network does not need to explicitly release the measIDs during SN addition. |

# Conclusion

Based on the discussion in the previous sections we propose the following:

# References

SSB-ToMeasure

[1] R2-2002692	Clarification for SSB-ToMeasure	Nokia, Nokia Shanghai Bell	CR	Rel-15	38.331	15.9.0	1457	1	F	NR_newRAT-Core	R2-2000859

[2] R2-2002693	Clarification of SSB-ToMeasure	Nokia, Nokia Shanghai Bell	CR	Rel-16	38.331	16.0.0	1516	-	A	NR_newRAT-Core

Inter-RAT SFTD

[3] R2-2003734	Correction to inter-RAT SFTD measurements	Huawei, HiSilicon	CR	Rel-15	36.331	15.9.0	4285	-	F	NR_newRAT-Core

[4] R2-2003735	Correction to inter-RAT SFTD measurements	Huawei, HiSilicon	CR	Rel-16	36.331	16.0.0	4286	-	A	NR_newRAT-Core

[5] R2-2003701	Correction to inter-RAT SFTD measurements	Huawei, HiSilicon	CR	Rel-15	38.331	15.9.0	1578	-	F	NR_newRAT-Core

[6] R2-2003702	Correction to inter-RAT SFTD measurements	Huawei, HiSilicon	CR	Rel-16	38.331	16.0.0	1579	-	A	NR_newRAT-Core