3GPP TSG-RAN WG2 Meeting #109bis-e					R2-20xxxxx

Electronic, 20 April – 30 April 2020

Agenda Item:		5.4.1.3	System information

Source:			Huawei, Ericsson, Apple

Title:				Summary on NR 15 system information and other

WI code(s):			NR_newRAT-Core, TEI15

Document for:		Discussion and Decision

## 1	Introduction

In section 5.4.1.3 System information, there is an email discussion as below:

- [AT109bis-e][011][NR15] System Information & Other (Huawei, Ericsson, Apple)

Scope: Treat all docs under AI 5.4.1.3 and AI 5.4.1.5

Part 1: Determine which issues that need resolution, find agreeable proposals. Deadline: April 23 0700 UTC

Part 2: For the parts that are agreeable, discussion will continue to agree on CRs.

The relevant papers are listed in [1] ~ [10]. This paper is to progress on topics under AI 5.4.1.3 and AI 5.4.1.5.

## 2	Discussion

### 2.1	Part 1 discussion

For Part 1 discussion, it is suggested to put some questions for each topic and then companies can provide comments if any.

Topic 1: SIB discussion ([1], [2])

Question 1 for SIB discussion:

Regarding which issues that need resolution, it is suggested to use the reason for change from [1] as the input:

In current RRC spec, ServingCellConfigCommonSIB and uplinkConfigCommon in ServingCellConfigCommonSIB is optional.

For NR SA Cell for UE camping, ServingCellConfigCommonSIB and uplinkConfigCommon in SIB1 should be provided.

Do you agree with the reason for change in [1]?

| Company | Yes/No | Comments |
| --- | --- | --- |

Question 2 for SIB discussion:

If your answer is Yes for Q1 for SIB discussion, do you agree with the changes made in [1]?

| Company | Yes/No | Comments |
| --- | --- | --- |

Topic 2: PWS and MG ([3], [4], [5])

Question 1 for PWS and MG:

Regarding which issues that need resolution, it is suggested to use the following two observations from [3] as the input.

Observation 1: It is complex and in some cases impossible for the network to avoid for every UE any overlap between measurement gaps and SIB6/SIB7/SIB8 scheduling.

Observation 2: Measurement gaps and SIB6/SIB7/SIB8 scheduling do not overlap persistently, i.e. there are typically many consecutive measurement gaps that do not overlap.

Do you agree with both observation 1 and 2 in [3]?

| Company | Yes/No | Comments |
| --- | --- | --- |

Question 2 for PWS and MG:

In [3], there is a proposal as below:

Proposal: The UE should try to acquire the first SIB6/7/8 SIB after reception of etwsAndCmasIndication even when it overlaps with a measurement gap.

If your answer is Yes for Q1 for PWS and MG, do you agree with the proposal in [3]?

| Company | Yes/No | Comments |
| --- | --- | --- |

Topic 3: Need code for CMAS ([6], [7], [8], [9], [10])

Question 1 for Need code for CMAS:

Regarding which issues that need resolution, it is suggested to use the following two observations from [6] as the input.

Observation 1: For UE behaviors upon handling warningAreaCoordinatesSegment-r15, the procedural text and ASN1 part are not aligned. This issue exists for both LTE (SIB12) and NR (SIB8).

Observation 2: For ASN1 part of warningAreaCoordinatesSegment-r15, i.e. Need OR, it may lead to the release of previously received information so that the UE may fail to assemble the geographical area coordinates. In contrast, the procedural text is reasonable.

Do you agree with both observation 1 and 2 in [6]?

| Company | Yes/No | Comments |
| --- | --- | --- |

Question 2 for Need code for CMAS:

In [6], there is a proposal as below:

Proposal 1: It is proposed RAN2 to confirm that the procedural text of warningAreaCoordinatesSegment-r15 is correct and make clarifications to Need code in ASN1.

If your answer is Yes for Q1 for Need code for CMAS, do you agree with proposal 1 in [6]?

| Company | Yes/No | Comments |
| --- | --- | --- |

## 3	Conclusions

[To be updated]

## 4	References

SIB1

[1] R2-2002818	Clarification on the essential fields in SIB1	Apple	CR	Rel-15	38.331	15.9.0	1525	-	F	NR_newRAT-Core

[2] R2-2002819	Clarification on the essential fields in SIB1	Apple	CR	Rel-16	38.331	16.0.0	1526	-	F	NR_newRAT-Core

PWS and MG

[3] R2-2003283	ETWS and CMAS acquisition during measurement gaps	Ericsson, Qualcomm, NTT DOCOMO INC, Nokia, InterDigital	discussion	Rel-15	NR_newRAT-Core

[4] R2-2003282	Clarification for SIB6, SIB7 and SIB8 acquisition during a measurement gap	Ericsson, Qualcomm, NTT DOCOMO INC, Nokia, InterDigital	CR	Rel-15	38.331	15.9.0	1551	-	F	NR_newRAT-Core

[5] R2-2003527	Clarification for SIB6, SIB7 and SIB8 acquisition during a measurement gap	Ericsson, Qualcomm, NTT DOCOMO INC, Nokia, InterDigital	CR	Rel-16	38.331	16.0.0	1566	-	A	NR_newRAT-Core

5 tdocs moved here from 4.5:

[6] R2-2003569	Discussion on Need code for CMAS	Huawei, HiSilicon	discussion	Rel-15	TEI15

[7] R2-2003570	Correction on Need code for CMAS	Huawei, HiSilicon	draftCR	Rel-15	36.331	15.9.0	F	TEI15

[8] R2-2003571	Correction on Need code for CMAS	Huawei, HiSilicon	draftCR	Rel-16	36.331	16.0.0	A	TEI15

[9] R2-2003572	Correction on Need code for CMAS	Huawei, HiSilicon	draftCR	Rel-15	38.331	15.9.0	F	TEI15

[10] R2-2003573	Correction on Need code for CMAS	Huawei, HiSilicon	draftCR	Rel-16	38.331	16.0.0	A	TEI15