3GPP TSG-RAN WG2 Meeting #109bis-e					R2-20xxxxx

Electronic, 20 April – 30 April 2020

Agenda Item:		5.4.3

Source:			Huawei, Nokia

Title:				[AT109bis-e][013][NR15] UE Cap Codebook parameters (Nokia, Huawei)

WI code(s):			NR_newRAT-Core

Document for:		Discussion and Decision

## 1	Introduction

This document is to kick off the below offline discussion:

- [AT109bis-e][013][NR15] UE Cap Codebook parameters (Nokia, Huawei)

Scope: Treat R2-2002552, R2-2002990, R2-2003456, R2-2003816, R2-2003817, R2-2003457, R2-2003458

Part 1: Determine which issues that need resolution, find agreeable proposals.

Deadline: April 23 0700 UTC

Part 2: For the parts that are agreeable, discussion will continue to agree on CRs

## 2	Part1 Discussion

### 2.1	Clarification on basic CSI feedback (R2-2003456, R2-2003816, R2-2003817)

The intention is to clarify the reported values for the supportedCSI-RS-ResourceList

Please note that R2-2003456 introduced the background and motivation, R2-2003816 and R2-2003817 are the latest CRs agreed by all the cosigners.

Do companies agree with the clarification described in the above papers?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Nokia | Yes fine with the intention but we have some questions. | codebookParameters Indicates the codebooks and the corresponding parameters supported by the UE. Parameters for type I single panel codebook (type1 singlePanel) supported by the UE, which are mandatory to report: - supportedCSI-RS-ResourceList; - The UE shall report at least one triplet in supportedCSI-RS-ResourceList with maxNumberTxPortsPerResource greater than or equal to 8 for FR1; - The UE shall report at least one triplet in supportedCSI-RS-ResourceList with maxNumberTxPortsPerResource greater than or equal to 2 for FR2. - modes indicates supported codebook modes (mode 1, both mode 1 and mode 2); - maxNumberCSI-RS-PerResourceSet indicates the maximum number of CSI-RS resource in a resource set. Question for clarification: Since the UE is mandated to report at least one triplet with the above mentioned requirement, what about the values in the other triplets? What if a UE ends up reporting lower values there? We are not so sure how to read the requirement so that the UEs are reporting these unambiguously. Please help clarify this. |
| NTT DOCOMO | No | The proposed text seems to imply that mandatory w/o feature (i.e. basic CSI feedback 2-32) is degraded from the existing functionalities. For instance, 2/4Tx codebooks are not supported for FR1. To make sure, the relevant basic features are: 1) Type I single panel codebook based PMI (further discuss which mode or both to be supported as mandatory) 2) 2Tx codebook for FR1 and FR2 3) 4Tx codebook for FR1 4) 8Tx codebook for FR1 when configured as wideband CSI report We don’t agree on changing mandatory w/o features at this late stage for Rel-15. |
| Orange | Yes | First of all we want to clarify that if a UE reports the triplet (Pmax,Rmax,Nmax) then any triplet (P<=Pmax,R<=Rmax,N<=Nmax) is supported by the UE. Furthermore, Nmax>=Pmax since Nmax=Pmax when Rmax=1.Therefore, if a UE reports at least one triplet (Pmax>=8,Rmax,Nmax) then it supports (8,1,8), (4,1,4) and (2,1,2), i.e., any value of maxNumberTxPortsPerResource below or equal to 8 is supported in the case of a single active CSI-resource . We consider that it is important to make the linkage between 2-36 signalling and basic CSI feedback 2-32 for many reasons: 1) UE feature group 2-32 defines minimum mandatory values of maxNumberTxPortsPerResource in FR1 and FR2 which can be overridden by 38.331 capability signaling related to 2-36 starting from p2, i.e., any value among {p2, p4, p8, p12, p16, p24, p32} is allowed, 2) for the sake of consistency, we do not understand that beamManagement SSB-CSI-RS has minimum mandatory values in TS38.306 while codebookParameters has not, 3) for the sake of the minimum coverage and capacity of NR since the number of supported antenna ports by the UE for codebook type I single panel is a building block of MIMO (particularly SU-MIMO), 4) for the sake of clarifying the relation between 2-32 and 2-36 since the IE codebookParameters has no differentiation between wideband and subband reporting. |
| Intel | Partially ok | From a rapporteur’s perspective, we are not comfortable to list the UE mandatory without capability features to be listed in 38.306. There are many others that would have to be listed in such a case and 38.306 list the capabilities that are optional. However, in this case we understand that this is added in relation to an optional capability. |
| MediaTek | Yes | We support the intention to clarify problematic mandatory capability w/o signalling in TS. From R1 LS, our understanding is the UE is required to explicitly report supportedCSI-RS-ResourceList and network can understand/operate the UE based on the signalling. This means UE has to report at least one triplet. We cannot find fallback definition in current spec, therefore, we can discuss whether to define fallback or have UE to report more triplet. |
| CATT | Yes | We support the proposed changes. From R1 LS in R2-2002552, supportedCSI-RS-ResourceList is mandatory to report (as no default value is defined). The proposed changes make sense as explained in the document above. |
| Telecom Italia | Yes | We support the proposed change as it is clear that a linkage between 2-36 and 2-32 UE features exists, hence we need to ensure that signalling associated to 2-36 is consistent with the minimum mandatory values of maxNumberTxPortsPerResource in FR1 and FR2 so to avoid any impact on the coverage and capacity provided by NR MIMO as stated by Orange |
| Huawei, Hisilicon | Yes (Proponent) | As we clarified in the RAN2 reflector, we think 2-36 capability reporting shall respect to the values defined in 2-32 which is mandatory without capability signalling. Regarding Nokia’s question, we think this is the usual way that the UE shall respect to what the specification is 4ritten and if the UE reports unexpected values, it is regarded wrong implementation from the UE side. |
| Ericsson | Yes | We think the clarification is needed. On further clarification mentioned above, we think the change would not preclude the report of triplet with lower values, it just clarifies that at least one triplet should be reported with maxNumberTxPortsPerResource of 8 or greater. In our understanding this report of maxNumberTxPortsPerResource of 8 implies that lower values are also supported for that triplet. |
| Deutsche Telekom | Yes | We support the proposed changes in order to clarify that the the UE, which is expected to provide the supportedCSI-RS-ResourceList IE with its capability signalling according to RAN1 LS in R2-2002552, will report at least one triplet with a value in maxNumberTxPortsPerResource for FR1 and respectively a value for FR2 that reflect the mandatory Tx codebook values as per the 2-32 MIMO feature as described in R2-2003456. |

### 2.2 clarification on consequences if not supported (R2-2002552, R2-2002990)

R2-2002552 is the reply LS from RAN1 on default values for codebook parameters.

R2-2002990 is to add clarifications for the cases where capability parameters are not reported and have default values including response to R2-2002552. The changes are made in multiple sections.

Do companies agree with the proposed changes? (Companies are encouraged to list the specific issues you agree/disagree)

| Company | Yes/No | Comments | Clarifications from rapporteur company |
| --- | --- | --- | --- |
| Nokia | Yes | We ensured to align the default values on codebook parameters based on the RAN1 LS response. The other capabilities were already discussed and were agreeable last meeting but only this one has been pending. | - |
| NTT DOCOMO | Yes | Prefer to collect all of the changes related to default values in to a single CR. | - |
| Intel | Needs to be discussed!! | The below change can create ambiguity: For rel-15 336 is not defined in RAN1 spec! One option is to choose 48, but we are not sure if there is a better way to solve this! beamSwitchTiming Indicates the minimum number of OFDM symbols between the DCI triggering of aperiodic CSI-RS and aperiodic CSI-RS transmission. The number of OFDM symbols is measured from the last symbol containing the indication to the first symbol of CSI-RS. The UE includes this field for each supported sub-carrier spacing. If this field is not included, the maximum beam switch timing is up to 336 OFDM symbols. For the below, we prefer to add as highlighted to clarify more: codebookParameters Indicates the codebooks and the corresponding parameters supported by the UE. Parameters for type I single panel codebook (type1 singlePanel) supported by the UE, which are mandatory to report: - supportedCSI-RS-ResourceList. The UE is mandated to report one or more instances; - modes indicates supported codebook modes (mode 1, both mode 1 and mode 2). The UE is mandated to report “mode 1”; - maxNumberCSI-RS-PerResourceSet indicates the maximum number of CSI-RS resource in a resource set. The UE is mandated to report atleast 1 CSI-RS resource in a resource set. | Clarification for beam switch timing [Amaanat] The maximum value is 336 symbols but RAN1 excel file said, “No recommendation on the desired beam switching timing”. Maybe some Ues may support less than 336 but without an explicit signalling network cannot make any assumption. An intermediate value might not be correct as we have no basis for discussion in RAN2 and RAN1 has no clear recommendation. During the email discussion, Ericsson suggested to have upper limit 336 which Docomo also was okay with. Hence the value. Clarification for codebook parameters [Accepted] Should be okay to have at least 1 if other companies do not have stronger opinion. |
| MediaTek | Not yet, need further discussion | We do not understand the change for csi-RS-ForTracking> maxSimultaneousResourceSetsPerCC, our understanding is UE is allowed to report more than 1, but the CR mandate UE to report 1? Was the intention to mandate UE to report csi-RS-ForTracking? maxSimultaneousResourceSetsPerCC indicates the maximum number of TRS resource sets per CC which the UE can track simultaneously. <In this release UE is mandated to report value 1>; CSI-RS-ForTracking ::= SEQUENCE { maxBurstLength INTEGER (1..2), maxSimultaneousResourceSetsPerCC INTEGER (1..8), maxConfiguredResourceSetsPerCC INTEGER (1..64), maxConfiguredResourceSetsAllCC INTEGER (1..256) } We believe some of the mandate support was built-in in ASN.1, so there is no need for further clarification. For example, many “..UE is mandated to report X or higher values..” clarifications are already clear in ASN.1. We suggest to remove those and only focus on those with different mandated value for FR1/2, which is not clear from ASN.1. For codebookParameters, there are R1 LS and HW and Nokia CR, we prefer to discuss corresponding changes together and to have a separate CR. Since R1 LS already mandate the UE to report the capability, the change “If this field is not included, the Type I single panel code book shall be supported.” Is obsolete. We also hope we can have a matching title for the CR  | [Clarification 1] [Ericsson – Lian] proposed defaults for these based on Hakon’s recommendation. maxBurstLength INTEGER (1..2) // 2 maxSimultaneousResourceSetsPerCC INTEGER (1..8) // 1 maxConfiguredResourceSetsPerCC INTEGER (1..64) // 8 for FR1, 16 for FR2 maxConfiguredResourceSetsAllCC INTEGER (1..256) // 16 for FR1, 32 for FR2 [Clarification] Please see above email discussion where this was agreed earlier. [Clarification] So you would like to approve all the other parts and just have the codebook one discussed separately? You were the one that suggested otherwise in previous meeting  R2-1915508 Report for [107bis#35] [NR R15] Consequences if not supported (Nokia) Nokia, Nokia Shanghai Bell CR Rel-15 38.306 15.7.0 0176 - F NR_newRAT-Core R2-1913964 Late - Nokia think we need to send an LS to R1 to ask some questions. - comeback, to check if agreement is possible for parts that we don’t ask R1 about, Offline 24, Draft LS to R1 in R2-1916496 (Nokia). - Nokia suggest to approve the parts not related to the LS to R1, i.e. the part for codebookParameters - MTK think we anyway need to wait, but MTK would be ok with email approval. QC agrees with MTK - Postpone |
| CATT |  | There seem to be many changes proposed, which needs further checking. |  |
| Telecom Italia | Yes (in principle) | We are fine with the intention to introduce further clarifications where needed. We are ok with the proposal from Intel related to the codebookParameters |  |
| Huawei, Hisilicion | Further discussion is needed | We understand the motivation, but we think the changes are mixing default values definition and values to be mandated reported. We largely share MTK’s view. |  |
| Ericsson | Yes, but | We can simplify the changes based on the RAN1 LS response, i.e. since “RAN1’s understanding is that the UE is expected to provide this IE with its capability signalling” we do not need the sentence “If this field is not included…”; for the change in modes for type I single panel codebook, since either mode 1 or both mode 1 and 2 are supported, we would not need to further specify that at least mode 1 is supported. |  |
| Deutsche Telekom | Yes, in principle | We agree with introducing clarifications where needed. At least, this should be done for the codebookParameters based on the RAN1 reply LS. |  |

## 3 Conclusions

[To be updated]

## 4 References

[1] R2-2002552	Reply LS on default codebook parameters  (R1-2001307; contact: Nokia)	RAN1	LS in	Rel-15	NR_newRAT-Core	To:RAN2

[2] R2-2002990	TS 38.306 Clarifying consequences if not supported	Nokia, Nokia Shanghai Bell, NTT Docomo Inc.	CR	Rel-15	38.306	15.9.0	0176	4	F	NR_newRAT-Core

[3] R2-2003456	Discussion on the capability of Basic CSI feedback (2-32)	Huawei, HiSilicon, Orange, Telecom Italia S.p.A., Vodafone, CMCC, China Unicom, China Telecom	discussion	Rel-15	NR_newRAT-Core

[4] R2-2003816	CR on the capability of Basic CSI feedback (2-32)	Huawei, HiSilicon, Orange, Telecom Italia S.p.A., Vodafone, CMCC, China Unicom, China Telecom, Ericsson	CR	Rel-15	38.306	15.9.0	0283	2	F	NR_newRAT-Core	R2-2003457	Late

[5] R2-2003817	CR on the capability of Basic CSI feedback (2-32)	Huawei, HiSilicon, Orange, Telecom Italia S.p.A., Vodafone, CMCC, China Unicom, China Telecom, Ericsson	CR	Rel-16	38.306	16.0.0	0284	2	A	NR_newRAT-Core	R2-2003458	Late

[6] R2-2003457	CR on the capability of Basic CSI feedback (2-23)	Huawei, HiSilicon, Orange, Telecom Italia S.p.A., Vodafone, CMCC, China Unicom, China Telecom	CR	Rel-15	38.306	15.9.0	0283	-	F	NR_newRAT-Core	Revised

[7] R2-2003458	CR on the capability of Basic CSI feedback (2-23)	Huawei, HiSilicon, Orange, Telecom Italia S.p.A., Vodafone, CMCC, China Unicom, China Telecom	CR	Rel-16	38.306	16.0.0	0284	-	A	NR_newRAT-Core	Revised