3GPP TSG-RAN WG2 Meeting #109bis electronic	R2-20xxxxx

20 April – 30 April 2020

Agenda Item:	6.1.3

Source: 	Huawei, HiSilicon

Title:	Summary of email discussion [AT109bis-e][019][IAB] BAP

Document for:	Discussion and Decision

# Introduction

This paper aims at capturing the summary of the following email discussion:

- [AT109bis-e][019][IAB] BAP (Huawei)

Scope: Treat BAP issues corrections and CR.

Part 1: R2-2003011 (and other non-controversial corrections if any), R2-2003561 P1 and P2

Part 2: Potential additions after on-line session, or other forgotten things (TBD)

Deadline: April 23 0700 UTC

Part 3: Update of CR, e.g. to include all agreements this meeting

Summary

R2-2003561	Summary of 6.1.3 for BAP open issues and corrections	Huawei, HiSilicon	discussion	Rel-16	NR_IAB-Core	Late

Discussion proposals treated on-line, the rest by email.

DISCUSSION

P3

- 	QC think we already concluded we leave this to implementation.

- 	ZTE think we didn’t agree to leave to impl. ZTE think option 1 is preferable, e.g. for GBR.

- 	Samsung think option 2 is simpler. Intel also think we have assumed/agreed Option 2. LG think that if default channel is used it need to support all QoS channels, so option 2 is for sure simpler.

- 	Ericsson want to keep both option 1 and 2. Samsung wonder how this would work. Ericsson think they wuld not be used at the same time. Ericsson are not completely sure, and if option 1 is similar to P2 then agree with Samsung.

- 	QC think option 2 is more general.

- 	Chair proposes to agree option 2. Huawei think option 1 can still be discussed. IAB-MT doesn’t know the QoS.

- 	Nokia think indeed we can just agree option 2 as we also have P1 and P2.

- The Donor CU can configure the 1:1 or N:1 mapping to BH RLC Channel on the backup egress link of IAB-node before BH RLF. Whether it is configured is up to CU implementation.

- If the regular mapping to BH RLC Channel in the backup egress link is configured by donor CU, IAB node follows the configured BH RLC channel mapping for re-routed packets.

- If the regular mapping to BH RLC Channel in the backup egress link is NOT configured by donor CU, IAB node: uses any BH RLC channel on the backup egress link for re-routed packets by implementation.

# Handling of each Tdoc in AI 6.1.3

After careful review on the Tdoc submitted to AI 6.1.3 BAP functionality, rapporteur gives following notes on the handling of the proposals, before the discussion of each issue.

| Tdoc | Comments by the summary rapporteur | Handling |
| --- | --- | --- |
| R2-2003304 Backhaul RLC Channel Remapping for IAB node(s) Ericsson | This issue is left as FFS in last meeting | To discuss as the outcome of the summary document. |
| R2-2002851 Further consideration on bearer mapping ZTE, Sanechips |  |  |
| R2-2003015 The bearer mapping configuration on the backup link in RLF Huawei, HiSilicon |  |  |
| R2-2002889 Remaining issues of DL HbH FC vivo | P1,2,3,4: The so-called event trigger of flow control has been discussed and agreed as IAB implementation. There is no clear agreement on the need of delay timer for this trigger. | To discuss as the outcome of the summary document. |
|  | P5: This has been discussed and agreed last meeting: “The BH RLC channels to be reported by the polled IAB node is up to the polled IAB node implementation. The routing IDs to be reported by the polled IAB node is up to the polled IAB node implementation.” So, the same principle can apply to the event-triggered case. Also, please note this has been implicitly implemented in the BAP spec, i.e. no normative text to specify the H RLC CH to be reported. | Already implemented in the current BAP spec. |
| R2-2003002 TP on clarifying a condition and aligning a terminology in BAP specification LG Electronics Inc. | O1: For the terminology of BAP path ID, in the BAP running CR, we have already aligned the terminology as “BAP path identity”. O2: cannot find the change. LG can continue comment in the running CR review, if needed. O3: This is addressed in the running CR. | Already/To be captured in the updated BAP running CR. |
| R2-2003176 Corrections to BAP Nokia, Nokia Shanghai Bell | For the change in 5.2.1.3, this is related to the issue IAB_1 P2 in email discussion #035 R2-2003297. For now, it is pending on the decision made in that email discussion; | Pending on the decision in email discussion #035 R2-2003297 |
|  | For the change in 5.2.2, this was addressed in the BAP running CR in 5.5 | Already/To be captured in the updated BAP running CR. |
|  | For the 1st change in section 5.3, there is no significant difference between the proposed change and the current spec. Nothing is broken for now. | Not critical or postponed. |
|  | The rest change can be captured in the BAP running CR. Appreciate the proposal. | Already/To be captured in the updated BAP running CR. |
| R2-2003438 On Release of BAP Entities CATT | P1: This is related to the support of inactive for IAB MT, which is pending on the email discussion of Issue IAB_2 #035 R2-2003297 | Pending on the decision in email discussion #035 R2-2003297 |
|  | P2: No spec change is needed. Skip this one | Skip |
|  | P3: To be discussed | To discuss as the outcome of the summary document. |
|  | P4: The proposal seems straight forward. The proposal will be implemented in the BAP running CR as “When upper layers request release of a BAP entity, the node shall: - release the BAP entity and the related BAP configurations.” | Already/To be captured in the updated BAP running CR. |

Based on the above analyses, following issues are summarized and discussed

# BAP open issues

- Issue 0: Bearer mapping at the backup link in RLF

This was discussed during online discussion, which will be skipped.

- Issue 1: BAP entity release at IAB-DU

This issue is discussed by P3 in R2-2003438 (CATT).

| Q2: Does DU release its BAP entity when MT BAP entity is released? | For orderly release, there are then separate procedures such as - F1 removal, and - BAP release When the control plane connection to Donor CU is available, these two procedures can be done separately. We therefore do not discuss more on the two procedures’ potential implications to each other. If we focus on BAP operations, Q2 itself results from the previous agreement on having BAP entity for DU and MT. a) Implementation limitation There was also agreement that - The intention of modelling into entities is to clarify the functionality. It is not intended to be an implementation restriction. It is then reasonable not to introduce possible implementation restriction, e.g., when one of MT or DU entity is released, it shall be possible to keep or release the other. b) Use case In case of orderly release of IAB node, it is very likely that Donor CU initiates F1-removal toward IAB DU, and RRC initiates connection release for IAB MT. For the use case, it is reasonable that IAB DU also releases its BAP entity. It should be possible implementation based on a). Alternatively, it seems also possible to mandate that IAB DU shall release its BAP entity if MT does so. But whether this is necessary needs further checking. In the RRC procedure, IAB MT might enter RRC IDLE for either orderly or disorderly release, such mandated linkage between IAB DU and MT release may not be necessary for all the cases. Furthermore, considering IAB is a network node it is preferable not to add too much restriction to its configuration. | IAB DU may release its BAP entity in case of F1-removal. FFS if any stage 2 clarifications is necessary. |
| --- | --- | --- |

Rapporteur proposes followings for R2 to discuss. If this cannot be easily agreed, it is suggest to discuss this in R3 instead.

Potential Proposal 1: R2 to confirm the BAP entity at DU can be released in case of F1-removal. And leave the details up to R3 if any stage 2 clarifications is necessary.

Question 1: Do you agree the above potential proposal 1? Or any other comments?

| Companies | Agree or not? | Comments |
| --- | --- | --- |
| Ericsson |  | In our understanding, this is something to be discussed in RAN3 and probably RAN3 will have a CB on this issue during RAN3-107bis-e. |

- Issue 2: The need of delay timer for event triggered BAP flow control

This issue is discussed by P1-4 in R2-2002889 (vivo).

The motivation to introduce a delay timer or prohibit timer is to avoid the frequent flow control feedback from child node to parent node. This only applies to the BAP flow control triggered when the buffer load exceeds a certain level by implementation, rather than the polling triggered flow control feedback.

The example approach is following:

An FC delay timer is introduced to avoid frequent FC feedback transmissions. When event-triggered FC feedback is transmitted, the IAB node starts the FC delay timer. When the FC delay timer is running, no event-triggered FC feedback can be triggered. Only in the case that the buffer load exceeds a certain threshold while the FC delay timer is not running, event-triggered FC feedback can be triggered.

Rapporteur proposes to first discuss the need of introducing delay timer or prohibit timer for the event triggered BAP flow control.

Question 2: Do you agree to specify delay timer for event triggered BAP flow control?

| Companies | Agree or not? | Comments |
| --- | --- | --- |
| Ericsson | Agree |  |

# Conclusion and proposals

Based on the above summary, following proposals are given.

TBD

# Reference

- R2-2003304	Backhaul RLC Channel Remapping for IAB node(s)	Ericsson

- R2-2002851	Further consideration on bearer mapping	ZTE, Sanechips

- R2-2003015	The bearer mapping configuration on the backup link in RLF	Huawei, HiSilicon

- R2-2002889	Remaining issues of DL HbH FC	vivo

- R2-2003002	TP on clarifying a condition and aligning a terminology in BAP specification	LG Electronics Inc.

- R2-2003176	Corrections to BAP	Nokia, Nokia Shanghai Bell

- R2-2003438	On Release of BAP Entities	CATT