3GPP TSG-RAN WG2 Meeting #109bis electronic	R2-20xxxx

20 April – 30 April 2020

Agenda Item:	6.1.9

Source: 	Huawei, HiSilicon

Title:	Summary of [AT109bis-e][024] 3X.304 CRs and IAB supporting in NPN

Document for:	Discussion and Decision

# Introduction

This paper aims at capturing the summary of the following email discussion:

- [AT109bis-e][024][IAB] 38304 36304 (Huawei)

Scope: Treat 36304 38304: Issues, corrections and CRs

Specifically: R2-2003012, R2-2003013, R2-2003179, R2-2003346

Part 1: Treat meeting input and comments. If more time is needed, e.g. for R2-2003346, gather initial comments and suggest way forward for decisions next meeting.

Deadline: April 24 0700 UTC

Part 2: Update of CRs, e.g. to include agreements this meeting

R2-2003012	Miscellaneous correction to 38.304 for IAB	Huawei, HiSilicon 	CR	Rel-16	38.304	16.0.0	0153	-	F	NR_IAB_enh-Core

R2-2003013	Miscellaneous correction to 36.304 for IAB	Huawei, HiSilicon 	CR	Rel-16	36.304	16.0.0	0786	-	F	NR_IAB_enh-Core

R2-2003179	Cell re-selection handling for IAB-MT	Nokia, Nokia Shanghai Bell	discussion	Rel-16	NR_IAB-Core

Moved here from 6.1.5:

R2-2003346	IAB support in NPN deployment 	Kyocera	discussion

# Discussion

- Issue 1: UAC clarification in 3X.304

The agreement “IAB-MTs are not under UAC control” should be implemented in 38.304 and 36.304.

Followings are the proposed changes for 3x.304 CRs

TS 38.304

### 5.3.0 Introduction There are two mechanisms which allow an operator to impose cell reservations or access restrictions. The first mechanism uses indication of cell status and special reservations for control of cell selection and reselection procedures. The second mechanism, referred to as Unified Access Control as specified in TS 38.331 [3], shall allow preventing selected access categories or access identities from sending initial access messages for load control reasons. IAB-MT does not apply the unified access control.

TS 36.304

- 5.3 Cell Reservations and Access Restrictions There are two mechanisms which allow an operator to impose cell reservations or access restrictions. The first mechanism uses indication of cell status and special reservations for control of cell selection and reselection procedures. The second mechanism, referred to as Access Control, shall allow preventing selected classes of users or ACDC categories from sending initial access messages for load control reasons. For Access Control based on Access Classes, at subscription, one or more Access Classes are allocated to the subscriber and stored in the USIM TS 22.011 [4]. For Access Control based on ACDC categories, at subscription at least four ACDC categories are allocated to the subscriber and stored in the ACDC MO TS 24.105 [31] or USIM TS 31.102 [32]. IAB-MT does not apply the access control.

Question 1: Do you agree with the above changes to 3x.304 for the clarification of UAC?

| Company | Agree or not | Comments |
| --- | --- | --- |
| Huawei | Agree | Please note that in 36.304, it was called as “access control” rather than UAC. |
| Ericsson | Agree | In the spec, it is missing that the legacy cell restrictions do not apply to IAB nodes. Currently it is missing. - cellReservedForOperatorUse (IE type: "reserved" or "not reserved") Indicated in SIB1 message. In case of multiple PLMNs indicated in SIB1, this field is specified per PLMN. This field is ignored by IAB-MT nodes. - cellReservedForOtherUse (IE type: "true") Indicated in SIB1 message. In case of multiple PLMNs indicated in SIB1, this field is common for all PLMNs. This field is ignored by IAB-MT nodes. Similar for the LTE. |

- Issue 2: Barred cell exclusion for IAB-MT

This issue was discussed in last meeting and left as FFS.

When the IAB-MT is barred by the network, IAB-MT “shall exclude the barred cell as a candidate for cell selection/reselection for 300 seconds” in the currect specification. However, it is not necessary/mandatory for MT to “not consider the cell for 300 seconds”, since IAB is the network node. IAB-MT implementation should be allowed to re-check the cell frequently to reduce the access latency. The similar change was proposed in R2-2003012 and R2-2003179.

Option 1: As in current spec, the UE (IAB-MT) shall exclude the barred cell as a candidate for cell selection/reselection for 300 seconds.

When cell status "barred" is indicated or to be treated as if the cell status is "barred", - The UE is not permitted to select/reselect this cell, not even for emergency calls. - The UE shall select another cell according to the following rule: - If the cell is to be treated as if the cell status is "barred" due to being unable to acquire the MIB: - the UE may exclude the barred cell as a candidate for cell selection/reselection for up to 300 seconds. - the UE may select another cell on the same frequency if the selection criteria are fulfilled. - else: - If the cell is to be treated as if the cell status is "barred" due to being unable to acquire the SIB1 or due to trackingAreaCode being absent in SIB1 as specified in TS 38.331 [3]: - The UE may exclude the barred cell as a candidate for cell selection/reselection for up to 300 seconds. - If the field intraFreqReselection in MIB message is set to "allowed", the UE may select another cell on the same frequency if re-selection criteria are fulfilled; - The UE shall exclude the barred cell as a candidate for cell selection/reselection for 300 seconds. - If the field intraFreqReselection in MIB message is set to "not allowed": - If the cell operates in licensed spectrum or if this cell belongs to a PLMN which is indicated as being equivalent to the registered PLMN: - the UE shall not re-select a cell on the same frequency as the barred cell; - else: - the UE may select to another cell on the same frequency if reselection criteria are fulfilled. - The UE shall exclude the barred cell and, if the cell operates in licensed spectrum or if this cell belongs to a PLMN which is indicated as being equivalent to the registered PLMN, also the cells on the same frequency as a candidate for cell selection/reselection for 300 seconds.

Option 2: With the proposed changes, the UE (IAB-MT) may exclude the barred cell as a candidate for cell selection/reselection for up to 300 seconds.

When cell status "barred" is indicated or to be treated as if the cell status is "barred", - The UE is not permitted to select/reselect this cell, not even for emergency calls. - The UE shall select another cell according to the following rule: - If the cell is to be treated as if the cell status is "barred" due to being unable to acquire the MIB: - the UE may exclude the barred cell as a candidate for cell selection/reselection for up to 300 seconds. - the UE may select another cell on the same frequency if the selection criteria are fulfilled. - If operating as an IAB-MT and the cell is to be treated as if the cell status is "barred" due to iab-Support being absent in SIB1: - The IAB-MT may exclude the barred cell as a candidate for cell selection/reselection for up to 300 seconds; - The IAB-MT may select another cell on the same frequency if the selection criteria are fulfilled. - else: - If the cell is to be treated as if the cell status is "barred" due to being unable to acquire the SIB1 or due to trackingAreaCode being absent in SIB1 as specified in TS 38.331 [3]: - The UE may exclude the barred cell as a candidate for cell selection/reselection for up to 300 seconds. - If the field intraFreqReselection in MIB message is set to "allowed", the UE may select another cell on the same frequency if re-selection criteria are fulfilled; - The UE shall exclude the barred cell as a candidate for cell selection/reselection for 300 seconds. - If the field intraFreqReselection in MIB message is set to "not allowed": - If the cell operates in licensed spectrum or if this cell belongs to a PLMN which is indicated as being equivalent to the registered PLMN: - the UE shall not re-select a cell on the same frequency as the barred cell; - else: - the UE may select to another cell on the same frequency if reselection criteria are fulfilled. - The UE shall exclude the barred cell and, if the cell operates in licensed spectrum or if this cell belongs to a PLMN which is indicated as being equivalent to the registered PLMN, also the cells on the same frequency as a candidate for cell selection/reselection for 300 seconds.

Based on the rapporteur’s interpretation, the difference, if we agree option 2, is that UE is allowed to consider the barred cell as a candidate for cell selection/reselection within 300 seconds, instead of only allowed to consider the barred cell as candidate after 300 seconds in option 1.

Question 2: Which option do you prefer as the IAB-MT behaviors, if the cell status is "barred" due to iab-Support being absent? Or do you have other interpretation?

| Company | Option 1 or 2? | Comments |
| --- | --- | --- |
| Ericsson | Option 1 | If we go for Option 2 with a “may”, we can well skip the IE as it really means that the MT is allowed to do that, but it does not mean it can do something else. We think shall is appropriate in this case. |

Another issue to be clarified is whether “intraFreqReselection” in MIB should be considered by IAB-MT.  Since IAB-MT should ignore the “cellBarred” in MIB, it is straight forward to also ignore the “intraFreqReselection” in MIB. That means “The IAB-MT may select another cell on the same frequency if the selection criteria are fulfilled”, i.e. same as if intraFreqReselection is set to allowed.

Question 3: Do you agree IAB-MT can ignore intraFreqReselection (i.e. as if intraFreqReselection is set to allowed)?

| Company | Agree or not | Comments |
| --- | --- | --- |
| Ericsson | Disagree | An MT may try to camp in the most suitable cell, but this cell might not be IAB capable. It makes sense that the MT tries to find another suitable cell, if allowed in the signaling, which fulfills the corresponding criteria and is IAB capable. |

If the Question 3 can be agreed, we need to decide how to clarify this.

Option 1: Clarify this in RRC spec, which is commonly used in the RRC spec.

3> if iab-Support is not provided for the selected PLMN nor the registered PLMN nor PLMN of the equivalent PLMN list: 4> consider the cell as barred for IAB-MT in accordance with TS 38.304 [20]; 4> perform barring as if intraFreqReselection is set to allowed;

Option 2: Clarify this in 304 spec

NOTE: For IAB node, it ignores the cellBarred, cellReservedForOperatorUse, and cellReservedForOtherUse as defined in TS 38.331 [3]. IAB-MT ignores the intraFreqReselection (as if intraFreqReselection is set to allowed).

Question 4: Which option(s) do you prefer to clarify the question 3, if agreed?

| Company | Option 1 or 2 or both? | Comments |
| --- | --- | --- |

In addition, companies are welcome to comment any need of other changes to 3x.304.

Question 5: Any other issues needs to be addressed as 3x.304 CR?

| Company | Comments |
| --- | --- |
| Ericsson | See comments in Issue 1 |

- Issue 3: IAB supporting in NPN

This issue is discussed in R2-2003346.

Since R2 has not discussed the supporting of IAB in NPN in R16, we should first try the very initial discussion.

Some potential spec impacts are mentioned in R2-2003346. For example, PRN WI adds npn-IdentityInfoList to support SNPN (Standalone NPN) and PNI-NPN (Public Network Integrated NPN). But it’s outside the legacy plmn-IdentityInfoList as RAN2 agreed that “NPN information is outside PLMN-IdentityInfoList as a new Rel-16 IE for NPN-only cell and PLMN+NPN cell (the total number of network IDs is still 12)”. Therefore, we may need to add iab-Support IE within npn-IdentityInfoList.

Sure, there may be other more specification impacts which are not foreseen yet. We may need to consider the feasibility to complete the supporting of NPN for IAB in the very last stage of R16 IAB.

As proposed in R2-2003346, “RAN2 should agree to ensure IAB functionality is also allowed in non-public network deployments”, rapporteur would like to ask the very initial question.

Question 6: Do you agree “R2 intends to ensure IAB functionality is also allowed in non-public network deployments in R16”?

| Company | Agree or not | Comments |
| --- | --- | --- |
| Ericsson | Disagree | In general, we would not like to start adding functionality here and there which has not been properly discussed, and only brought at the last moment. |

# Conclusion and proposals

Based on the above summary, following proposals are given.

TBD.

# Reference

- R2-2003012	Miscellaneous correction to 38.304 for IAB	Huawei, HiSilicon

- R2-2003013	Miscellaneous correction to 36.304 for IAB	Huawei, HiSilicon

- R2-2003179	Cell re-selection handling for IAB-MT	Nokia, Nokia Shanghai Bell

- R2-2003346	IAB support in NPN deployment 	Kyocera