3GPP TSG-RAN WG2 #109b-e	R2-200xxxx

Electronic, 20 Apr – 30 Apr 2020

Source: 	vivo

Title: 	Report of [AT109bis-e][025][IIOT] Accurate Reference Timing (vivo)

Agenda Item:	6.7.2.1

Document for:	Discussion and decision

# Introduction

This is the report of the following offline discussion:

- [AT109bis-e][025][IIOT] Accurate Reference Timing (Vivo)

Status: Not yet Started, will be started after on-line session April 21

Scope: Treat topics in 6.7.2.1, open issues and corrections, in particular parts of R2-2003809 that are not treated on-line.

Part 1: Determine which issues that need resolution, find agreeable proposals. Deadline: April 24 0700 UTC

Based on the following RAN2 agreements, the offline discussion is to define the signalling details of the UE request of the reference time information and to resolve some open issues of R2-2003809 which are not treated on-line.

- The CONNECTED UE can request the reference time information. - Offline : the signaling solution, (iron out what are the two proposals and their fundamental difference)

# Discussion

## Issue 1: UE request of reference time

According to the summary paper [13], companies proposed two options for the request of the reference time:

- Option 1: The request of the reference time information is sent via the UEAssistanceInformation message

- Option 2: The request of the reference time information is sent via the DedicatedSIBRequest message

The text proposals for Option 1 is given in Annex A, and the text proposals for Option 2 is represented as Option 2-1 in Annex B (by adding reference SFN for UTC time) and Option 2-2 in Annex C (by explicitly indicating the request of reference time information).

For Option 1, it seems that the only change is to add the request of the reference time information in the UEAssistanceInformation message, and the reference time information is sent via the DLInformationTransfer message as the current specification.

For Option 2, the request of the SIB9 is added in the DedicatedSIBRequest message. As the SIB9 includes both the legacy time (i.e. the Rel-15 timeInfo of UTC) and the IIOT time (i.e. the referenceTimeInfo-r16 of IIOT), if the UE only indicates the request of SIB9 as Option 2-1 in Annex B, the gNB is not able to know which time information is requested by the UE. Thus the gNB (e.g. by implementation) would send the RRCReconfiguration message containing the whole SIB9 to the UE. One extra change in the specification would be to add the reference SFN also for the Rel-15 timeInfo of UTC. If the UE explicitly indicates the request of the reference time information as Option 2-2 in Annex C, the gNB can send the reference time information via the DLInformationTransfer message. The request of the Rel-15 timeInfo of UTC would still be via the RACH procedure of SI request.

Question 1: Which message is used to send the request of the reference time information?

- Option 1: The request of the reference time information is sent via the UEAssistanceInformation message

- Option 2: The request of the reference time information is sent via the DedicatedSIBRequest message

| Company | Answer (Option 1 or Option 2) | Comments |
| --- | --- | --- |
| vivo | Option 1 or Option 2 | We think that both Options can work, but we should probably avoid the impact on the reception of the legacy UTC time. The Option 2-2 of explicitly indicating the request of reference time via the DedicatedSIBRequest message seems to be the simplest one from the specification. |
| OPPO | Option 2 | Both Option 1 or Option 2 are Ok. But still slightly prefer Option 2 since it is close to the end of standardization. Option 1 works fine, but standardization of requesting for reference time information needs to be done from the scratch. |
| Ericsson | Option 1 | We are not okay with option 2. In addition to the complexities discussed online and summarized above, option 2 also creates inter WI-feature dependencies. We don’t think this is good for IIoT eco-system if we couple it with another specific and somewhat unrelated feature. For option 1, we can follow similar procedures as other messages sent in UEAssistantInformation. This option also comes with the benefits of extensibility without combability issue with other on-demand SI features. |

Question 2: If the Answer for Question 1 is Option 2, should the reference SFN be added for the Rel-15 timeInfo of UTC as given in the Option 2-1 of Annex B?

| Company | Answer (Yes/No) | Comments |
| --- | --- | --- |
| vivo | No | This depends on how the CONNECTED UE indicates its request of the reference time. If the DedicatedSIBRequest message includes the explicit indication of “the request on the IIOT referenceTimeInfo”, the gNB can send the reference time information to the connected UE via the DLInformationTransfer message without adding the reference SFN for the UTC time. For the request of the Rel-15 timeInfo of UTC, the CONNECTED UE can still trigger RACH-based SI request. |
| OPPO | No | We need to say UTC time info is not needed for the TSN synchronization purpose. We should pursue that the response of OSI request from the network for the SIB 9 is solely for the achieving 5G internal timing synchronization for TSC usages. |
| Ericsson |  | The question is whether the proposal is to support Rel-15 version of SIB9, Rel-16 version of SIB9 or a specific field in the Rel-16 version of SIB9. |

Question 3: If the Answer for Question 1 is Option 2, should the UE explicitly indicates the request of reference time information as given in the Option 2-2 of Annex C?

| Company | Answer (Yes/No) | Comments |
| --- | --- | --- |
| vivo | Yes | Both Option 1 and Option 2-2 provide the explicit request of the reference time information. This can avoid the impact on the reception of the legacy UTC time. |
| OPPO | No | The only reason why SIB9 could be requested by taking advantage of OSI mechanism is that there is a new IE ReferencetimeInfo introduced in R16. So we should choose option 2-1. |
| Ericsson |  | The same as above. The question is whether the proposal is to support Rel-15 version of SIB9, Rel-16 version of SIB9, or a specific field in the Rel-16 version of SIB9. |

In [8], company proposes that the UE can indicate the delivery periodicity of the reference time, as the TSC service period is generally long and the UE would need to be calibrated periodically due to the clock drifting. Thus the gNB can send the updated reference time information to the UE without the frequent/periodic report of the reference time information from the UE. From the rapporteur’s understanding, this assistance information is common for the above two Options.

Question 4: Can the UE indicate the delivery periodicity of the reference time?

| Company | Answer (Yes/No) | Comments |
| --- | --- | --- |
| vivo | No strong view | This seems to be an enhancement on the request signalling. From our understanding, the UE can send a new request of the reference time information when its internal clock drifts. |
| OPPO | No | Agree with vivo that UE can request the reference time information when needed. Unnecessary Optimization should be avoided. |
| Ericsson | No | From a practical point of view, the periodicity of the clock update depends on the clock stratum level performance of the UE. As discussed previously in this work item, all UEs should be equipped with adequately-well clock for this feature to make any sense. gNB can assume a worst case and use a corresponding periodicity for reference time transmissions in broadcast message, which we don’t expect to be very frequent. In unicast message, it might help to reducing the RRC unicast message delivery to a particular UE, but we don’t think it is crucial. Note also that UE tracks the frame boundary to perform UL transmission. In other words, it is somehow synced to the gNB’s internal clock. With this, UE might continuously deduct the reference time (considering propagation delay compensation updates) even without receiving an update. |

## Issue 2: Reference time encoding

The paper [9] proposes that the Rel-16 10ms resolution of the GPS reference time is not provided when the Rel-15 10ms resolution of the GPS time in SIB9 is provided. The papers [2] [7] consider that the Rel-16 GPS time should be independently configured. In [2], the reasons for the independent configuration would be as follows:

- The high accuracy time is from GPS receivers and and there might be some complexity (resulting in loss of accuracy) in the translation between GPS time and UTC time at both gNB and UE

- UTC contains leap seconds (i.e. the UTC time is advanced by 1 second about once every 2 years to account for anomalies in the earth’s rotation).

- UTC has the year 2038 problem (wrap around occurs at 03:14:07 UTC on 19 January 2038).

Question 5: Can companies confirm that the GPS time of the Rel-16 reference time information is provided independently without using the Rel-15 GPS 10ms resolution of SIB9, as the current specification?

| Company | Answer (Yes/No) | Comments |
| --- | --- | --- |
| vivo | Yes | The signalling saving enhancement proposed by [9] seems not essential, and could cause more issues as observed by [2] and [7]. |
| OPPO | Yes | Agree with the reasons given by [2]. |
| Ericsson | Yes |  |

The paper [2] proposes to use a single field to represent the Rel-16 reference time “to simplify the encoding of the accurate reference timing”. The paper [7] considers that “the issue with single information element with 10ns steps is that it would exceed the range of 64bit unsigned integer numbers, which may introduce unnecessary computational complexity”. The one field encoding of the reference time in [2] is quoted as follows:

ReferenceTimeInfo-r16 ::= SEQUENCE { time-r16 INTEGER (0.. 576460752303423486), uncertainty-r16 INTEGER (0..32767) OPTIONAL, -- Need R timeInfoType-r16 ENUMERATED {localClock} OPTIONAL, -- Need R referenceSFN-r16 INTEGER (0..1023) OPTIONAL -- Cond RefTime }

Question 6: Do you agree to simplify the reference time encoding by using one field?

| Company | Answer (Yes/No) | Comments |
| --- | --- | --- |
| vivo | No | Firstly we consider that the UE complexity of supporting 64bit unsigned integer numbers as observed by [7] is a valid technical issue. And we don’t think that the one field encoding simplifies the specification. |
| OPPO | No | No strong reason to change the current agreed encoding format of ReferenceTimeInfo-16. |
| Ericsson | Yes | Since all fields in IE ReferenceTime are mandatory, the proposal is to simplify the encoding that can save a couple of bits. The reason is that the individual mandatory field within the IE ReferenceTime is not a power of two. In addition, we can align a bit better with the UTC time encoding. The new field has 59 bits (2^59-1 = 576460752303423486), if they are encoded as un-signed integers. It is smaller than 64 bits. Maybe we’ve misunderstood the concerns. Note that UTC time has 39 bits and so 32 bits are already exceeded. If there are no technical concerns, we propose to adopt the simplification. Otherwise, we are fine to leave as it is. |

# Conclusion

TBD

# References

- R2-2002705	On UE need for time synch	Ericsson	discussion	NR_IIOT-Core

- R2-2002706	On encoding of reference time information	Ericsson	discussion	NR_IIOT-Core

- R2-2002752	Remaining Issues on Accurate Reference Timing	CATT	discussion	NR_IIOT-Core

- R2-2002772	UE report of the reference time interest	vivo	discussion	R2-2000489

- R2-2002940	Reference Timing Delivery of gNB	Samsung	discussion	Rel-16	NR_IIOT-Core

- R2-2002976	On-demand SI requesting for reference time information by connected UE	OPPO	discussion	Rel-16	NR_IIOT-Core

- R2-2003167	Remaining issues for accurate reference time delivery	Nokia, Nokia Shanghai Bell	discussion	Rel-16	NR_IIOT

- R2-2003294	FFS on accurate reference timing request	ZTE Corporation, Sanechips, China Southern Power Grid Co., Ltd	discussion	Rel-16	NR_IIOT-Core

- R2-2003397	ASN.1 improvements for saving 32 bits in reference time in SIB9	Qualcomm Incorporated	discussion

- R2-2003404	Draft CR 1 for On-demand SI request for RRC connected UEs	Huawei, HiSilicon	draftCR	Rel-16	38.331	16.0.0	NR_IIOT-Core	Late

- R2-2003505	Remaining Issues for Accurate reference timing	CMCC	discussion	Rel-16	NR_IIOT-Core

- R2-2003738	On-demand SI request for RRC connected UEs	Huawei, HiSilicon	discussion	Rel-16	NR_IIOT-Core	R2-2002993

- R2-2003809	Summary of 6.7.2.1 Accurate reference timing	vivo	discussion	Rel-16	NR_IIOT-Core

# Annex A: Option 1: UE assistance information [7]

### 5.7.4	UE Assistance Information

#### 5.7.4.1	General

Figure 5.7.4.1-1: UE Assistance Information

The purpose of this procedure is to inform the network of the UE's delay budget report carrying desired increment/decrement in the connected mode DRX cycle length, or overheating assistance information.

#### 5.7.4.2	Initiation

A UE capable of providing delay budget report in RRC_CONNECTED may initiate the procedure in several cases, including upon being configured to provide delay budget report, upon change of delay budget preference and upon change of interest in reference time information.

A UE capable of providing overheating assistance information in RRC_CONNECTED may initiate the procedure if it was configured to do so, upon detecting internal overheating, or upon detecting that it is no longer experiencing an overheating condition.

Upon initiating the procedure, the UE shall:

1>	if configured to provide delay budget report:

2>	if the UE did not transmit a UEAssistanceInformation message with delayBudgetReport since it was configured to provide delay budget report; or

2>	if the current delay budget is different from the one indicated in the last transmission of the UEAssistanceInformation message including delayBudgetReport and timer T342 is not running:

3>	start or restart timer T342 with the timer value set to the delayBudgetReportingProhibitTimer;

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide a delay budget report;

1>	if configured to provide overheating assistance information:

2>	if the overheating condition has been detected and T345 is not running; or

2>	if the current overheating assistance information is different from the one indicated in the last transmission of the UEAssistanceInformation message including overheatingAssistance and timer T345 is not running:

3>	start timer T345 with the timer value set to the overheatingIndicationProhibitTimer;

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide overheating assistance information;

1>	if configured to provide reference time assistance information:

2>	if the UE did not transmit a UEAssistanceInformation message with referenceTimeInfoRequired since it was configured to provide reference time assistance information; or

2>	if the UE preference to receive reference time information changed from the last time UE initiated transmission of the UEAssistanceInformation message including referenceTimeInfoRequired:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide reference time assistance information;

5.7.4.3	Actions related to transmission of UEAssistanceInformation message

The UE shall set the contents of the UEAssistanceInformation message as follows:

1>	if transmission of the UEAssistanceInformation message is initiated to provide a delay budget report according to 5.7.4.2;

2>	set delayBudgetReport to type1 according to a desired value;

1>	if transmission of the UEAssistanceInformation message is initiated to provide overheating assistance information according to 5.7.4.2;

2>	if the UE experiences internal overheating:

3>	if the UE prefers to temporarily reduce the number of maximum secondary component carriers:

4>	include reducedMaxCCs in the OverheatingAssistance IE;

4>	set reducedCCsDL to the number of maximum SCells the UE prefers to be temporarily configured in downlink;

4>	set reducedCCsUL to the number of maximum SCells the UE prefers to be temporarily configured in uplink;

3>	if the UE prefers to temporarily reduce maximum aggregated bandwidth of FR1:

4>	include reducedMaxBW-FR1 in the OverheatingAssistance IE;

4>	set reducedBW-FR1-DL to the maximum aggregated bandwidth the UE prefers to be temporarily configured across all downlink carriers of FR1;

4>	set reducedBW-FR1-UL to the maximum aggregated bandwidth the UE prefers to be temporarily configured across all uplink carriers of FR1;

3>	if the UE prefers to temporarily reduce maximum aggregated bandwidth of FR2:

4>	include reducedMaxBW-FR2 in the OverheatingAssistance IE;

4>	set reducedBW-FR2-DL to the maximum aggregated bandwidth the UE prefers to be temporarily configured across all downlink carriers of FR2;

4>	set reducedBW-FR2-UL to the maximum aggregated bandwidth the UE prefers to be temporarily configured across all uplink carriers of FR2;

3>	if the UE prefers to temporarily reduce the number of maximum MIMO layers of each serving cell operating on FR1:

4>	include reducedMaxMIMO-LayersFR1 in the OverheatingAssistance IE;

4>	set reducedMIMO-LayersFR1-DL to the number of maximum MIMO layers of each serving cell operating on FR1 the UE prefers to be temporarily configured in downlink;

4>	set reducedMIMO-LayersFR1-UL to the number of maximum MIMO layers of each serving cell operating on FR1 the UE prefers to be temporarily configured in uplink;

3>	if the UE prefers to temporarily reduce the number of maximum MIMO layers of each serving cell operating on FR2:

4>	include reducedMaxMIMO-LayersFR2 in the OverheatingAssistance IE;

4>	set reducedMIMO-LayersFR2-DL to the number of maximum MIMO layers of each serving cell operating on FR2 the UE prefers to be temporarily configured in downlink;

4>	set reducedMIMO-LayersFR2-UL to the number of maximum MIMO layers of each serving cell operating on FR2 the UE prefers to be temporarily configured in uplink;

2>	else (if the UE no longer experiences an overheating condition):

3>	do not include reducedMaxCCs, reducedMaxBW-FR1, reducedMaxBW-FR2, reducedMaxMIMO-LayersFR1 and reducedMaxMIMO-LayersFR2 in OverheatingAssistance IE;

1>	if transmission of the UEAssistanceInformation message is initiated to provide a reference time information according to 5.7.4.2:

2>	if the UE requires to be provisioned with reference time information:

3>	set referenceTimeInfoRequired to true;

2>	else:

3>	set referenceTimeInfoRequired to false;

#### –	OtherConfig

The IE OtherConfig contains configuration related to miscellaneous other configurations.

OtherConfig information element

-- ASN1START

-- TAG-OTHERCONFIG-START

OtherConfig ::=                 SEQUENCE {

delayBudgetReportingConfig  CHOICE{

release                 NULL,

setup                   SEQUENCE{

delayBudgetReportingProhibitTimer   ENUMERATED {s0, s0dot4, s0dot8, s1dot6, s3, s6, s12, s30}

}

}                                                                                                     OPTIONAL        -- Need M

}

OtherConfig-v1540 ::=           SEQUENCE {

overheatingAssistanceConfig     SetupRelease {OverheatingAssistanceConfig}                            OPTIONAL, -- Need M

...

}

OtherConfig-r16 ::=           SEQUENCE {

referenceTimeNeedReporting-r16     ENUMERATED {true}                            OPTIONAL, -- Need M

...

}

OverheatingAssistanceConfig ::= SEQUENCE {

overheatingIndicationProhibitTimer    ENUMERATED {s0, s0dot5, s1, s2, s5, s10, s20, s30,

s60, s90, s120, s300, s600, spare3, spare2, spare1}

}

-- TAG-OTHERCONFIG-STOP

-- ASN1STOP

OtherConfig field descriptions

delayBudgetReportingProhibitTimer Prohibit timer for delay budget reporting. Value in seconds. Value s0 means prohibit timer is set to 0 seconds, value s0dot4 means prohibit timer is set to 0.4 seconds, and so on.

overheatingAssistanceConfig Configuration for the UE to report assistance information to inform the gNB about UE detected internal overheating.

overheatingIndicationProhibitTimer Prohibit timer for overheating assistance information reporting. Value in seconds. Value s0 means prohibit timer is set to 0 seconds, value s0dot5 means prohibit timer is set to 0.5 seconds, value s1 means prohibit timer is set to 1 second and so on.

referenceTimeNeedReporting If present, the field indicates the UE is configured to provide reference time assistance information.

#### –	UEAssistanceInformation

The UEAssistanceInformation message is used for the indication of UE assistance information to the network.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

UEAssistanceInformation message

-- ASN1START

-- TAG-UEASSISTANCEINFORMATION-START

UEAssistanceInformation ::=         SEQUENCE {

criticalExtensions                  CHOICE {

ueAssistanceInformation             UEAssistanceInformation-IEs,

criticalExtensionsFuture            SEQUENCE {}

}

}

UEAssistanceInformation-IEs ::=     SEQUENCE {

delayBudgetReport                   DelayBudgetReport                   OPTIONAL,

lateNonCriticalExtension            OCTET STRING                        OPTIONAL,

nonCriticalExtension                UEAssistanceInformation-v1540-IEs   OPTIONAL

}

DelayBudgetReport::=                CHOICE {

type1                               ENUMERATED {

msMinus1280, msMinus640, msMinus320, msMinus160,msMinus80, msMinus60, msMinus40,

msMinus20, ms0, ms20,ms40, ms60, ms80, ms160, ms320, ms640, ms1280},

...

}

UEAssistanceInformation-v1540-IEs ::= SEQUENCE {

overheatingAssistance               OverheatingAssistance               OPTIONAL,

nonCriticalExtension                UEAssistanceInformation-v16xy-IEs   OPTIONAL

}

UEAssistanceInformation-v16xy-IEs :: = SEQUENCE {

referenceTimeInfoRequired-r16			ENUMERATED {true, false}	OPTIONAL,

nonCriticalExtension               		SEQUENCE {}		OPTIONAL

}

OverheatingAssistance ::=           SEQUENCE {

reducedMaxCCs                       SEQUENCE {

reducedCCsDL                        INTEGER (0..31),

reducedCCsUL                        INTEGER (0..31)

} OPTIONAL,

reducedMaxBW-FR1                    SEQUENCE {

reducedBW-FR1-DL                    ReducedAggregatedBandwidth,

reducedBW-FR1-UL                    ReducedAggregatedBandwidth

} OPTIONAL,

reducedMaxBW-FR2                    SEQUENCE {

reducedBW-FR2-DL                    ReducedAggregatedBandwidth,

reducedBW-FR2-UL                    ReducedAggregatedBandwidth

} OPTIONAL,

reducedMaxMIMO-LayersFR1            SEQUENCE {

reducedMIMO-LayersFR1-DL            MIMO-LayersDL,

reducedMIMO-LayersFR1-UL            MIMO-LayersUL

} OPTIONAL,

reducedMaxMIMO-LayersFR2            SEQUENCE {

reducedMIMO-LayersFR2-DL            MIMO-LayersDL,

reducedMIMO-LayersFR2-UL            MIMO-LayersUL

} OPTIONAL

}

ReducedAggregatedBandwidth ::= ENUMERATED {mhz0, mhz10, mhz20, mhz30, mhz40, mhz50, mhz60, mhz80, mhz100, mhz200, mhz300, mhz400}

-- TAG-UEASSISTANCEINFORMATION-STOP

-- ASN1STOP

# Annex B: Option 2-1: SIB9 OSI [10]

##### 5.2.2.4.10	Actions upon reception of SIB9

Upon receiving SIB9, the UE may:

1>	if SIB9 includes the utcReferenceSFN:

2>		calculate the UTC time based on the included timeInfoUTC, dayLightSavingTime, leapSeconds, localTimeOffset and utcReferenceSFN;

2>	inform upper layers of the time information;

1>	if SIB9 includes the referenceTimeInfo;

2>	perform the related actions as specified in subclause 5.7.1.3.

#### –	DedicatedSIBRequest

The DedicatedSIBRequest message is used to request SIB(s) required by the UE in RRC_CONNECTED as specified in clause 5.2.2.3.3.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

DedicatedSIBRequest message

-- ASN1START

-- TAG-DEDICATEDSIBREQUEST-START

DedicatedSIBRequest-r16 ::=      SEQUENCE {

criticalExtensions               CHOICE {

dedicatedSIBRequest-r16          DedicatedSIBRequest-r16-IEs,

criticalExtensionsFuture         SEQUENCE {}

}

}

DedicatedSIBRequest-r16-IEs ::=  SEQUENCE {

onDemandSIB-RequestList-16       SEQUENCE {

-- Editor'a Note: The size of requestedSIB-List-r16 depends by how many SIBs will be specified in Release 16 and how to implement correctly this will be done during the ASN.1 review.

requestedSIB-List-r16            SEQUENCE (SIZE(1..ffsValue)) OF SIB-ReqInfo-16

} OPTIONAL,

lateNonCriticalExtension         OCTET STRING             OPTIONAL,

nonCriticalExtension             SEQUENCE {}              OPTIONAL

}

-- Editor's Note: How to capture that SIB specified in DCCA WI cannot be requested on-demand is done once Rel-16 specification is availabe.

SIB-ReqInfo-16 ::=                   ENUMERATED {sib9, ffsOtherValues}

-- TAG-DEDICATEDSIBREQUEST-STOP

-- ASN1STOP

DedicatedSIBRequest field descriptions

requestedSIB-List Contains a list the UE is allowed to request while in RRC_CONNECTED.

#### –	SIB9

SIB9 contains information related to GPS time and Coordinated Universal Time (UTC). The UE may use the parameters provided in this system information block to obtain the UTC, the GPS and the local time.

NOTE:	The UE may use the time information for numerous purposes, possibly involving upper layers e.g. to assist GPS initialisation, to synchronise the UE clock.

SIB9 information element

-- ASN1START

-- TAG-SIB9-START

SIB9 ::=                            SEQUENCE {

timeInfo                            SEQUENCE {

timeInfoUTC                         INTEGER (0..549755813887),

dayLightSavingTime                  BIT STRING (SIZE (2))                   OPTIONAL,   -- Need R

leapSeconds                         INTEGER (-127..128)                     OPTIONAL,   -- Need R

localTimeOffset                     INTEGER (-63..64)                       OPTIONAL    -- Need R

}                                                                               OPTIONAL,   -- Need R

lateNonCriticalExtension            OCTET STRING                                OPTIONAL,

...,

[[

referenceTimeInfo-r16           ReferenceTimeInfo-r16                           OPTIONAL    -- Need R

]],

[[

utcReferenceSFN-r16           INTEGER (0..1023)           OPTIONAL    -- Cond UTC-RefTime

]]

}

-- TAG-SIB9-STOP

-- ASN1STOP

SIB9 field descriptions

dayLightSavingTime Indicates if and how daylight-saving time (DST) is applied to obtain the local time. The semantics are the same as the semantics of the Daylight Saving Time IE in TS 24.501 [23] and TS 24.008 [38]. The first/leftmost bit of the bit string contains the b2 of octet 3 and the second bit of the bit string contains b1 of octet 3 in the value part of the Daylight Saving Time IE in TS 24.008 [38].

leapSeconds Number of leap seconds offset between GPS Time and UTC. UTC and GPS time are related i.e. GPS time -leapSeconds = UTC time.

localTimeOffset Offset between UTC and local time in units of 15 minutes. Actual value = field value * 15 minutes. Local time of the day is calculated as UTC time + localTimeOffset.

timeInfoUTC Coordinated Universal Time corresponding to the SFN boundary at or immediately after the ending boundary of the SI-window in which SIB9 is transmitted. The field counts the number of UTC seconds in 10 ms units since 00:00:00 on Gregorian calendar date 1 January, 1900 (midnight between Sunday, December 31, 1899 and Monday, January 1, 1900). See NOTE 1. This field is excluded when determining changes in system information, i.e. changes of timeInfoUTC should neither result in system information change notifications nor in a modification of valueTag in SIB1.

utcReferenceSFN This field indicates the reference SFN corresponding to the UTC time information. If timeInfo field is received in RRCReconfiguration message, this field indicates the SFN of PCell.

| Conditional Presence | Explanation |
| --- | --- |
| UTC-RefTime | The field is mandatory present if timeInfo is included in RRCReconfiguration message; otherwise the field is absent. |

NOTE 1:	The UE may use this field together with the leapSeconds field to obtain GPS time as follows: GPS Time (in seconds) = timeInfoUTC (in seconds) -  2,524,953,600 (seconds) + leapSeconds, where 2,524,953,600 is the number of seconds between 00:00:00 on Gregorian calendar date 1 January, 1900 and 00:00:00 on Gregorian calendar date 6 January, 1980 (start of GPS time).

- –	ReferenceTimeInfo

The IE ReferenceTimeInfo contains timing information for 5G internal system clock used for, e.g., time stamping, see TS 23.501 [32], clause 5.27.1.2.

ReferenceTimeInfo information element

-- ASN1START

-- TAG-REFERENCETIMEINFO-START

ReferenceTimeInfo-r16 ::= SEQUENCE {

time-r16                            ReferenceTime-r16,

uncertainty-r16                     INTEGER (0..32767)          OPTIONAL,   -- Need R

timeInfoType-r16                    ENUMERATED {localClock}     OPTIONAL,   -- Need R

referenceSFN-r16                    INTEGER (0..1023)           OPTIONAL    -- Cond RefTime

}

ReferenceTime-r16 ::=           SEQUENCE {

refDays-r16                         INTEGER (0..72999),

refSeconds-r16                      INTEGER (0..86399),

refMilliSeconds-r16                 INTEGER (0..999),

refTenNanoSeconds-r16               INTEGER (0..99999)

}

-- TAG-REFERENCETIMEINFO-STOP

-- ASN1STOP

ReferenceTimeInfo field descriptions

referenceSFN This field indicates the reference SFN corresponding to the reference time information. If referenceTimeInfo field is received in DLInformationTransfer message, this field indicates the SFN of PCell.

time This field indicates time reference with 10ns granularity. The indicated time is referenced at the network, i.e., without compensating for RF propagation delay. The indicated time in 10ns unit from the origin is refDays*86400*1000*100000 + refSeconds*1000*100000 + refMilliSeconds*100000 + refTenNanoSeconds. The refDays field specifies the sequential number of days (with day count starting at 0) from the origin of the time field. If the referenceTimeInfo field is received in DLInformationTransfer message, the time field indicates the time at the ending boundary of the system frame indicated by referenceSFN. The UE considers this frame (indicated by referenceSFN) to be the frame which is nearest to the frame where the message is received (which can be either in the past or in the future). If the referenceTimeInfo field is received in SIB9, the time field indicates the time at the SFN boundary at or immediately after the ending boundary of the SI-window in which SIB9 is transmitted. If referenceTimeInfo field is received in SIB9, this field is excluded when determining changes in system information, i.e. changes of time should neither result in system information change notifications nor in a modification of valueTag in SIB1.

timeInfoType If timeInfoType is not included, the time indicates the GPS time and the origin of the time field is 00:00:00 on Gregorian calendar date 6 January, 1980 (start of GPS time). If timeInfoType is set to localClock, the origin of the time is unspecified.

uncertainty This field indicates the uncertainty of the reference time information provided by the time field. The uncertainty is 25ns multiplied by this field. If this field is absent, the uncertainty is unspecified.

| Conditional Presence | Explanation |
| --- | --- |
| RefTime | The field is mandatory present if referenceTimeInfo is included in DLInformationTransfer or RRCReconfiguration message; otherwise the field is absent. |

# Annex C: Option 2-2: SIB9 OSI

#### –	DedicatedSIBRequest

The DedicatedSIBRequest message is used to request SIB(s) required by the UE in RRC_CONNECTED as specified in clause 5.2.2.3.3.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

DedicatedSIBRequest message

-- ASN1START

-- TAG-DEDICATEDSIBREQUEST-START

DedicatedSIBRequest-r16 ::=      SEQUENCE {

criticalExtensions               CHOICE {

dedicatedSIBRequest-r16          DedicatedSIBRequest-r16-IEs,

criticalExtensionsFuture         SEQUENCE {}

}

}

DedicatedSIBRequest-r16-IEs ::=  SEQUENCE {

onDemandSIB-RequestList-16       SEQUENCE {

-- Editor'a Note: The size of requestedSIB-List-r16 depends by how many SIBs will be specified in Release 16 and how to implement correctly this will be done during the ASN.1 review.

requestedSIB-List-r16            SEQUENCE (SIZE(1..ffsValue)) OF SIB-ReqInfo-16

} OPTIONAL,

lateNonCriticalExtension         OCTET STRING             OPTIONAL,

nonCriticalExtension             SEQUENCE {}              OPTIONAL

}

-- Editor's Note: How to capture that SIB specified in DCCA WI cannot be requested on-demand is done once Rel-16 specification is availabe.

SIB-ReqInfo-16 ::=                   ENUMERATED {referenceTime, ffsOtherValues}

-- TAG-DEDICATEDSIBREQUEST-STOP

-- ASN1STOP

DedicatedSIBRequest field descriptions

requestedSIB-List Contains a list the UE is allowed to request while in RRC_CONNECTED. The value referenceTime indicates the request of the referenceTimeInfo of SIB9.