3GPP TSG-RAN WG2 #109b-e	R2-200xxxx

Electronic, 20 Apr – 30 Apr 2020

Source: 	Huawei, HiSilicon

Title: 	Report of [AT109bis-e][034][DCCA] NR-NR DC (Huawei)

Agenda Item:	6.10.3

Document for:	Discussion and decision

# Introduction

This is the report of the following offline discussion:

- [AT109bis-e][034][DCCA] NR-NR DC (Huawei)

Scope: Treat topics in 6.10.3, Start immediately with R2-2003656 and R2-2003657. Wait for on-line discussion for others.

Part 1: Determine which issues that need resolution, find agreeable proposals. Deadline: April 24 0700 UTC

This offline discussion is to discuss the NR-DC power control coordination in case of Async CA when semi-static power control Alt 1-2 is used.

# Discussion

## Background

In R16, for FR1+FR1 NR-DC and FR2+FR2 NR-DC, semi-static power control Alt.1-2 is supported as captured in TS 38.213.

If a UE is provided NR-DC-PC-mode = Semi-static-mode2 - if at least one symbol of slot of the MCG or of the SCG that is indicated as uplink or flexible to a UE by tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated, if provided, overlaps with a symbol for any ongoing transmission overlapping with slot of the SCG or of the MCG, respectively, the UE determines a power for the transmission on the SCG or the MCG overlapping with slot as described in Clauses 7.1 through 7.5 using or , respectively, as the maximum transmission power - otherwise, the UE determines a power for the transmission on MCG or the SCG overlapping with slot , as described in [8-3, TS 38.101-3] and in Clauses 7.1 through 7.5 without considering or , respectively

In last meeting, we have already discussed and agreed for semi-static power control Alt1-2, network can exchange semi-static TDD configuration of serving cells, so that network can try to use larger power for better UL performance when there is no overlapping UL transmission between MCG and SCG.

- For NR-DC power control, need an IE to indicate the semi-static TDD pattern of MCG to SN when semi-static power control Alt 1-2 is set by MN (may already be present)

And considering RAN3 may be defining network exchange procedure on Xn for TDD pattern, we send LS in R2-2001759[1] , to ask RAN3 to support exchange of the semi-static TDD pattern for NR-DC power coordination.

ACTION: RAN2 respectfully asks RAN3 to support exchange of the semi-static TDD pattern for NR-DC power coordination. It is up to RAN3 how to support such an information exchange, i.e. by reusing the existing IE (i.e. ‘intended TDD DL-UL Configuration NR' in Rel-16) or introducing a new IE.

## Issues to be solved

In R16, Async CA is supported as well, wherein the frame boundary of serving cells within one CG is unaligned and the SFN is partially aligned. The MN/SN informs the UE of the slot offset between the PCell/PSCell and SCell(s) in ca-SlotOffset-r16.

ServingCellConfig information element

-- ASN1START

-- TAG-SERVINGCELLCONFIG-START

ServingCellConfig ::=               SEQUENCE {

tdd-UL-DL-ConfigurationDedicated    TDD-UL-DL-ConfigDedicated                                   OPTIONAL,   -- Cond TDD

…………………………

rateMatchPatternToAddModList        SEQUENCE (SIZE (1..maxNrofRateMatchPatterns)) OF RateMatchPattern       OPTIONAL,   -- Need N

rateMatchPatternToReleaseList       SEQUENCE (SIZE (1..maxNrofRateMatchPatterns)) OF RateMatchPatternId     OPTIONAL,   -- Need N

downlinkChannelBW-PerSCS-List       SEQUENCE (SIZE (1..maxSCSs)) OF SCS-SpecificCarrier                     OPTIONAL    -- Need S

]],

[[

firstWithinActiveTimeBWP-Id-r16     BWP-Id                                                        OPTIONAL,   -- Cond MultipleNonDormantBWP

firstOutsideActiveTimeBWP-Id-r16    BWP-Id                                                        OPTIONAL    -- Cond MultipleNonDormantBWP-WUS

ca-SlotOffset-r16                      CHOICE {

refSCS15kHz                                    INTEGER (-2..2),

refSCS30KHz                                    INTEGER (-5..5),

refSCS60KHz                                    INTEGER (-10..10),

refSCS120KHz                                   INTEGER (-20..20)

}                                                                                           OPTIONAL     -- Cond AsyncCA

]]

}

In case of Async CA deployed within MCG or SCG, only exchanging TDD pattern is not enough for MN/SN to determine the potential overlapping UL transmission. For example, if the MN only knows the TDD pattern of SCG serving cells, and assume the slot number is aligned between SCG serving cells as indicated in Figure 1, for MCG UL slots, it may think there is no UL overlapping in SCG; whereas in case of Asyn CA as indicated in Figure 2, due to slot offset, actually there is UL overlapping for the MCG UL slots. Considering all above, we think the MN/SN needs to be aware of the slot offset between SCell and spCell within SCG/MCG.

Figure 1. UL overlapping dertermination not considering slot offset

Figure 2. UL overlapping dertermination considering slot offset

Q1: do companies agree that in case of Async CA MN/SN needs to know slot offset within SCG/MCG for semi-static power control Alt1-2?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | No | First we need to decide whether there is really a need to support DC with async CA within each cell group. What would be the deployment scenario? Unless a strong use case is found, we propose not to support this. |

In R15 NR-DC, the SFTD can be configure with UE to measure the timing difference between PCell and PSCell/neighbour cell. In case of Async CA, the first thought coming to mind maybe to reuse SFTD method to get the slot offset within SCG. However, there are some limitations with SFTD method. The first thing is that only MN can configure SFTD to UE, and only to measure the SFTD between PCell and PSCell. So from MN side, it maybe not very handy to know the time difference between PCell and SCG SCell(s). From SN side, it can know the time difference between PSCell and PCell if the MN includes this information in CGConfigInfo, but SN may still not know the time difference between other MCG SCell and PSCell. Another thing should be noted is that UE is not mandated to support SFTD, and performing SFTD may have negative impact on UE data transmission, so it should be avoided that configuring UE to do too much SFTD measurement. Therefore, we think to introduce network information exchange on slot offset may be an easier solution.

Q2: do companies agree to introduce network information exchange of slot offset within SCG/MCG for semi-static power control Alt1-2?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | No | First we need to decide whether there is really a need to support DC with async CA within each cell group. What would be the deployment scenario? Unless a strong use case is found, we propose not to support this. |

If we agree to exchange slot offset between MN and SN, we also needs to decide this slot offset information should be UE dedicated signalling or non-UE dedicated signalling. As for different UE, the SCG serving cell(s) may be different, and the PC mode is configured per-UE, so we think this information is better exchanged at per UE level, i.e. including slot offset information of per serving cell within SCG/SCG in UE dedicated signalling.

Q3: do companies agree the slot offset information should be exchanged at per-UE level?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | No | First we need to decide whether there is really a need to support DC with async CA within each cell group. What would be the deployment scenario? Unless a strong use case is found, we propose not to support this. |

There are two options to enable the information exchange of slot offset between MN and SN.

- Option 1: RAN2 to include the slot offset information in inter-node message.

- Option 2: RAN2 to ask RAN3 to include the slot offset information in XnAP message.

We understand the reason we asked RAN3 to enable the information exchange of TDD pattern in last meeting is because RAN3 is defining it then. However, from our knowledge, RAN3 is not considering to introduce slot offset related exchange procedure from the time being. Hence, we prefer to do this effort in RAN2 spec.

Q4: do companies agree RAN2 to include the slot offset information in inter-node message?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | No | First we need to decide whether there is really a need to support DC with async CA within each cell group. What would be the deployment scenario? Unless a strong use case is found, we propose not to support this. |

# Conclusion

TBD

# References

- R2-2001759, LS on TDD pattern exchange for NR-DC power control.