3GPP TSG-RAN WG2 Meeting #109bis-e	R2-2xxxxx

Electronic, 20 April – 30 April 2020

Agenda Item:	6.10.5

Source: 	OPPO

Title:  	Email report of [AT109bis-e][036][DCCA] Fast SCell Activation (OPPO)

Document for:	Discussion and decision

# Introduction

This paper aims to trigger the email discussion for the left open issues in dormant BWP topic.

- [AT109bis-e][036][DCCA] Fast Scell Activation (OPPO)

Scope: Treat general and RRC topics in 6.10.5, based on R2-2003770 and comments. Can start discussion on non-controversial proposals immediately, if any. Wait for on-line discussion for others.

Part 1: Determine which issues that need resolution, find agreeable proposals. Deadline: April 24 0700 UTC

# Open issues for dormant BWP topic

## Issue 1: Implicit BFD-RS configuration for dormant BWP?

In last RAN2 meeting, the dormancy behaviour was discussed and made a lot of agreements. There is FFS for the implicit RS configuration for beam failure detection.

- FFS: the implicit BFD-RS configuration for dormant BWP is supported or not.

In summary of dormant BWP [1], the possible WFs are summarized as:

Option 1: do not support the implicit BFD-RS on dormant BWP.

Option 2: Revise the prior agreements that the PDCCH-config IE can be configured for the dormant BWP in order to support the implicit BFD-RS configuration for dormant BWP.

Option 2.1: Only tci-StatesPDCCH-ToAddList is applied for the dormant BWP and ignore other configurations in PDCCH-config.

Option 2.2: No search space is configured in PDCCH-Config of dormant BWP, so that the UE doesn’t monitor PDCCH in dormant BWP but can apply tci-StatesPDCCH-ToAddList included in ControlResourceSet.

In Monday online discussion, the issue is discussed in RRC open issues discussion and no consensus, then agree to wait for RAN1.

Question 1: What is the way forward in RAN2 for implicit BFD-RS on dormant BWP?

WF1: Do nothing, just wait for RAN1 agreements.

WF2: RAN2 can continue the discussion. If this WF is chosen, please provide your preferred option, i.e. option 1/2.1/2.2.

| Company | WF1/2and Option 1/2.1/2.2 | Comment, if any |
| --- | --- | --- |
| OPPO | WF1 |  |
| Qualcomm | WF2 | We have concern that RAN1 will pursue some “fancy” design which will unnecessary postpone the completion of dormant BWP. We think both Option 2.1/2.2 are just simple RRC signalling change, and RAN2 can decide. Between them, we prefer Option 2.2: - Option 2.1: - It will introduce a new UE requirement (i.e. ignore other IEs). - It is not clear whether other TCI related IE needs to be ignored by the UE (e.g. tci-PresentInDCI). Note that these new MIMO related IEs are always coming in almost each release according to LTE specification experience. - Option 2.2: - It will bring much less spec impact in RRC: we only need to clarify that searchSpacesToAddModList is absent or NW only indicates controlResourceSetToAddMod/ReleaseList in dormant BWP. For example, the below change is enough: - NW can still configure TCI related (existing and future) IEs in its CORESET, which is forward combability. |
| NEC | WF1 | as discussed online (Monday, 20th) |
| Intel | WF1 |  |
| Nokia | WF1 |  |
| CATT | WF1 |  |
| Huawei | WF1 |  |
| Apple | WF1 |  |
| Futurewei | WF1 |  |
| LG | WF1 |  |
| ZTE | WF1 |  |
| MediaTek | WF1 is acceptable | We have some sympathy on QC’s proposal but accept to wait the discussion from RAN1. |

## Issue 2: Active UL non-dormant BWP configuration when leaving dormant BWP?

In last RAN2 meeting, RAN2 agree to introduce the dormant DL BWP only and configure the BWP id via RRC to be activated BWP upon transition from dormancy to non-dormancy. Based on the agreements, it means the UE will activate last serving UL BWP before entering dormancy upon transition from dormancy to non-dormancy.

- No UL dormant BWP is defined, and the UL 3ehavior is specified in TS38.321 in case the DL BWP is switched to dormant BWP.

- Network will configure the BWP id via RRC to be activated BWP upon transition from dormancy to non-dormancy (does not reuse the firstActiveDownlinkBWP-Id in RRC).

Upon transition from dormancy to non-dormancy, it is common understanding that the first non-dormant UL BWP is the UL BWP with the same ID as the first non-dormant DL BWP in TDD case. For FDD case, two companies propose to define one/two new first active UL BWP upon transition from dormancy to non-dormancy, e.g. for suitable uplink data transmission. One company proposes that the UL BWP will be deactivated if entering dormant BWP. It also proposes that it is enough to activate UL BWP configured with legacy first-Active-Uplink-BWP, i.e. no need to introduce a new UL BWP for this [1].

Question 2: Do you agree the common understanding below?

RAN2 confirm that, for TDD, the first non-dormant UL BWP is the UL BWP with the same ID as the first non-dormant DL BWP.

| Company | Yes or NO | Comment, if any |
| --- | --- | --- |
| OPPO | Yes | For TDD, the DL and UL share the same spectrum, so the DL BWP and UL BWP is linked together. Copy text from 38.213: “For unpaired spectrum operation, a DL BWP from the set of configured DL BWPs with index provided by BWP-Id is linked with an UL BWP from the set of configured UL BWPs with index provided by BWP-Id when the DL BWP index and the UL BWP index are same. For unpaired spectrum operation, a UE does not expect to receive a configuration where the center frequency for a DL BWP is different than the center frequency for an UL BWP when the BWP-Id of the DL BWP is same as the BWP-Id of the UL BWP.” |
| Qualcomm | Yes | We understand that the highlighted part from 38.213 from OPPO also applies to dormant BWP. Specifically, our understanding on the UE behaviour of dormant BWP switch in TDD is: - Upon reception of dormancy DCI indication, the UE switches to the DL BWP which is configured as dormantDownlinkBWP-Id, and the UL BWP with same BWP ID; - And then follow the DL and UL (dormancy) behaviour specified in Section 5.15.1 of TS 38.321 |
| NEC | Yes |  |
| Intel | Yes for TDD |  |
| Nokia | Yes | We agree with OPPO comment – in TDD UL and DL BWP switching is linked together. Even in 38.321 it is said “For unpaired spectrum, a DL BWP is paired with a UL BWP, and BWP switching is common for both UL and DL.” |
| CATT | Yes | Agree with OPPO that the DL BWP and UL BWP is linked together in TDD. |
| Huawei | Yes |  |
| Apple | Yes |  |
| Futurewei | Yes | Agreed for TDD |
| LG | Yes |  |
| ZTE | Yes |  |
| MediaTek | Yes |  |

Question 3: For FDD, there are 3 options to address how to indicate first active UL BWP upon transition from dormancy to non-dormancy. Which option do you prefer?

- Option 1: Implicit rule, UE will activate last serving UL BWP before entering dormancy.

- Option 2: UE will activate UL BWP addressed by firstActiveUplinkBWP-Id.

- Option 3: Network will configure the new UL BWP id via RRC to be activated UL BWP (i.e. does not reuse the firstActiveUplinkBWP-Id in RRC)

| Company | Option ½/3 | Comment, if any |
| --- | --- | --- |
| OPPO | Option 1 | We can not see the benefit to change the UL BWP due to the leaving dormancy and Option 1 does not impact the spec. |
| Qualcomm | Option 1 | We prefer Option 1 because: - It is enough and without any spec impact required - The firstWithinActiveTimeBWP-Id and firstOutsideActiveTimeBWP-Id are both from RAN1, which clearly only include DL BWP according to the excel of LS. If we want to introduce first non-dormant UL BWP, we need to get RAN1 input. - It follows similar procedure specified for “BWP switch of default BWP” in 38.321 (in Rel-15). In our understanding, the current UE behaviour of dormant BWP switch in FDD (based on current 38.321 running CR) is: - Upon reception of dormancy DCI indication, the UE switches to the DL BWP which is configured as dormantDownlinkBWP-Id, and keeps in the last UL BWP before dormancy transition; - Then follow the DL and UL (dormancy) behaviour specified in Section 5.15.1 of TS 38.321 Please note that in above dormant BWP switching procedure, the UE always maintains one active DL BWP, and one active UL BWP (although UL transmission is not allowed) in an activated SCell. And the NW knows which UL BWP is active during this procedure same as legacy BWP switch procedure. Then, we don’t see any ambiguity in BWP mismatch between gNB and UE. Thus, we don’t see benefit of Option 2 and Option 3. |
| NEC | Option 1 | Given it was agreed not to define UL dormant BWP, there is no need to define something extra for UL. Also, considering an expected data volume, the selection of UL BWP upon SCell activation would not so big issue as it is anyway SCell and PCell is always available. |
| Intel | Option1 | This is the simplest and also the practical one. We should not list DL with UL for FDD. |
| Nokia | Option 1 | In fact we do not see any problem here: Existing MAC CR (and also proposed update) handles this well. UE does not change UL BWP ID for paired spectrum when DL goes to dormant BWP ID – anyway there is no activity in UL BWP at all so it does not matter which UL BWP ID is active at this point of time. Then when DL BWP ID changes to not dormant BWP – UL operation continues in the already active UL BWP. |
| CATT | Option 1 | If the network wants to switch the active UL BWP, a DCI can always be sent. |
| Huawei | Option 1 | We also think option1 is clear without extra spec change. |
| Apple | Option 1 | Option 1 can be supported without spec change. |
| Futurewei | Option 1 | Suggest making the Option 1 more clear for proposal: Option 1: Implicit rule, UE will activate last serving UL BWP associated with the dormant DL BWP before entering dormancy. |
| LG | Option 1 | We cannot see the benefit to introduce UL BWP for leaving dormancy. |
| ZTE | Option 3 | We have one basic question: --If back to the last UL BWP before entering dormancy is sufficient, why don’t we use the same mechanism for DL BWP? In our understanding, the motivation is the same for DL and UL, In case NW wants to configure the first non-dromant DL BWP with a large BW, NW would also expect to configure a compatible UL BWP (with large BW) for the first non-dormant UL BWP. Using another DCI to switch UL BWP will cause additional latency and inefficient. So similar to DL BWP, we hope to have the flexibility to explicitly indicate the first active UL BWP for transmission from dormancy to non-dormancy. And it can be decoupled with firstActiveUplinkBWP-Id. |
| MediaTek | Prefer option 3 and option 2 is acceptable | We are surprised that so many companies want to use implicit rule for UL BWP. This is not a big change and it more like a clarification rather than enhancement in our opinion. It is also OK to consult with RAN1 on this. |

## Issue 3: PHR generation due to dormant BWP

In LTE, the SCell state includes deactivated SCell state, activated SCell state and dormant SCell state. The LTE PHR generation will only include the activated SCells’ PHR, i.e. not include the SCell’s PHR which is dormant SCell state in LTE.

In NR, the SCell state includes deactivated SCell state and activated SCell state. The dormancy behavior of SCell is one kind of activated SCell state. Following the LTE rules for PHR generation, the activated SCell on which the active BWP is dormant BWP should not be included in PHR report.

Question 4: Do you agree that activated SCell on which the active BWP is dormant BWP should not be included in PHR report as LTE?

| Company | Yes or NO | Comment, if any |
| --- | --- | --- |
| OPPO | yes | It aligns with LTE did. |
| Qualcomm | Yes |  |
| NEC | Yes | Considering no UL transmission, it is natural not to report PH for SCell using dormant BWP. |
| Intel | Yes |  |
| Nokia | yes | Alignment with LTE sounds OK to avoid 1byte overhead. This would also prevent us needing to consult RAN1 if calculation of PHR would work in this scenario. |
| CATT | Yes |  |
| Huawei | No strong view | We understand the intention is to avoid PHR related operation after switched to dormancy DL BWP, as there is no PUSCH transmission in UL. We do not have strong opinion on this. However, if we do this change in MAC spec, the other specs may also need the corresponding change, e.g. in TS 38.300. We may need careful check. Power headroom reports (PHR) are needed to provide support for power-aware packet scheduling. In NR, three types of reporting are supported: a first one for PUSCH transmission, a second one for PUSCH and PUCCH transmission in an LTE Cell Group in EN-DC (see TS 37.340 [21]) and a third one for SRS transmission on SCells configured with SRS only. In case of CA, when no transmission takes place on an activated Scell, a reference power is used to provide a virtual report. Power headroom reports are transmitted using MAC signalling.. |
| Apple | Maybe No. | As indicated by HW, current spec has support the PHR reporting for the active SCell with dormant BWP. It can work well without any spec change. |
| Futurewei | No (not making decision now) | PHR is important for efficient scheduling for data transmission. It is not clear without PHR report for the dormant BWP to get network prepared what impact would be on delaying the efficient scheduling of initial data transmission upon dormancy to non-dormancy transition. NR is slightly different from LTE since dormancy is in the activated state. The latency performance requirement should be consistent with that in activated state of NR. It would be prudent to check with RAN1 before we make decision in RAN2 |
| LG | Yes | No strong view. |
| ZTE | Yes |  |
| MediaTek | Yes |  |

# RRC changes for dormant BWP topic

## Change 1 – [R2-2002983]:

For the first part changes in [2] is make the filed descriptions under CellGroupConfig IE clearer.

Question 7: Do you agree to capture the changes of filed descriptions under CellGroupConfig IE [2] in 38.331?

| Company | Yes or NO | Comment, if any |
| --- | --- | --- |
| OPPO | Yes |  |
| Qualcomm |  | It seems the latest RRC CR (R2-2003382) has removed all the Ies related to dormant BWP within CellGroupConfig. Then we think we don’t need to discuss this question, right? |
| NEC |  | After checking CR indicated by QC, we understand some parts are already reflected, while some editorials may need to be discussed, e.g. change from Scell group to Scell dormancy group in ServingCellConfig IE |
| Nokia | Yes | Seems to be covered mostly in R2-2003382 |
| CATT |  | There are changes proposed in the latest RRC CR (R2-2003382). |
| Huawei |  | Should be cross checked together with RRC CR. |
| Futurewei |  | Likely. No need to decide now. |
| LG |  | We have same understanding with Qualcomm. |
| ZTE |  | Similar view with Qualcomm. |
| MediaTek |  | Similar view as Qualcomm. |

For the second part changes in [2], the dormant BWP configuration is omitted in 38.331 running CR. RAN1 agreed that the dormant BWP can be configured only when UE is configured with at least two BWPs for an SCell in RAN1#98bis.

- For the L1 based Scell dormancy indication sent on primary cell within active time - UE is configured with at least two BWPs for an Scell

So, the dormant BWP configuration should be based on condition that UE is configured with at least two BWPs for an Scell.

The common understanding is that the dormant BWP can be configured in Scell addition. It is not clear whether the dormant BWP can be configured via Scell modification or only via Scell release+add.

Question 8: Do you agree the following principles for dormant BWP configuration?

- Dormant BWP configuration should be based on condition that UE is configured with at least two BWPs for an Scell.

- Dormant BWP configuration can be configured in Scell addition and Scell modification procedure.

| Company | Yes or NO | Comment, if any |
| --- | --- | --- |
| OPPO | Yes |  |
| Qualcomm | Yes |  |
| NEC | Yes | For the first point, this is the fundamental assumption for dormant BWP. No need to capture anything in RRC. It is network responsibility to ensure this condition. |
| Intel | Yes |  |
| Nokia | Yes |  |
| CATT | Yes | However we wonder how the NW release the dormant BWP? Via the withinActiveTimeToReleaseList or outsideActiveTimeToReleaseList? Whether a clarification is needed that if the SCell is not included in any dormancy group, the UE will not consider that the configuration is an error if the dormant BWP referred by the dormantDownlinkBWP-Id have the normal BWP configuration e.g. PDCCH and SPS configuration. |
| Huawei | Yes |  |
| Apple | Yes |  |
| Futurewei | Yes |  |
| LG | Yes |  |
| ZTE | Yes |  |
| MediaTek | Yes |  |

For the changes for firstWithinActiveTimeBWP-Id-r16/ firstOutsideActiveTimeBWP-Id-r16, the condition is removed, and use need code “Need S” instead.

The firstWithinActiveTimeBWP-Id-r16 IE is only can be configured when the dormant BWP is configured and the firstOutsideActiveTimeBWP-Id-r16 IE can only be configured when both dormant BWP and WUS are configured.

Question 8: Do you agree the following principles for firstWithinActiveTimeBWP-Id-r16/ firstOutsideActiveTimeBWP-Id-r16 configuration?

- The firstWithinActiveTimeBWP-Id-r16 IE configuration should be based on condition when the dormant BWP is configured.

- The firstOutsideActiveTimeBWP-Id-r16 IE configuration should be based on condition when both dormant BWP and WUS are configured.

| Company | Yes or NO | Comment, if any |
| --- | --- | --- |
| OPPO | Yes |  |
| Qualcomm | No for both principle list by rapporteur and 2nd change in CR R2-2002983 | For the principle list by rapporteur: First, whether these two separate IEs are needed pending on RAN1 LS. Secondly, if only one dormant BWP and only one non-dormant BWP in one SCell, we don’t think first non-dormant BWP is needed to be configured by RRC. Thus, it is required to be configured only when dormant BWP is configured and more than 1 non-dormant BWPs are configured. For the 2nd change in CR R2-2002983: We don’t understand why the change of removing presence condition of MultipleNonDormantBWP and MultipleNonDormantBWP-WUS is required. We prefer to keep these two presence conditions in latest running CR (R2-2003382) , with below minor changes: MultipleNonDormantBWP The field is mandatory present when the SCell is configured with a dormant bandwidth part and with more than one BWP-DownlinkDedicated with pdcch-Config present non-dormant BWPs, otherwise it is absent. MultipleNonDormantBWP-WUS The field is mandatory present when the Scell is configured with WUS, with a dormant bandwidth part, and with more than one BWP-DownlinkDedicated with pdcch-Config present non-dormant BWPs, otherwise it is absent. |
| NEC | No (preference) | we prefer to capture the corresponding condition, especially for the second one (firstOutsideActiveTimeBWP-Id-r16 IE) somehow, because this is not just based on Scell dormancy function, but the combination with other function (WUS). |
| Nokia | Wait for RAN1 input whether we need even these two parameters separately |  |
| CATT | Yes |  |
| Huawei | See comments | We understand the intention of this change, but as Nokia’s comments, RAN1 may discuss this issue based on our LS, we could wait for RAN1’s reply. |
| Apple | Yes for the intention |  |
| Futurewei |  | Wait for RAN1 feedback to RAN2 LS, then take action accordingly. |
| LG | No | We think this change is not needed since current condition in the running CR is clear. |
| ZTE | No | We prefer to keep the two conditions. Further, the original text gives a wrong impression that a dormant BWP can be configured without “more than one non-dormant BWP”. We think the first non-dormant BWP must be configured if dormant BWP is configured. Otherwise UE cannot perform transition to non-doramncy. It is up to NW to ensure more than one non-dormant BWP is configured. Thus we prefer the following changes: MultipleNonDormantBWP The field is mandatory present when the SCell is configured with a dormant bandwidth part and with more than one BWP-DownlinkDedicated with pdcch-Config present , otherwise it is absent. MultipleNonDormantBWP-WUS The field is mandatory present when the SCell is configured with WUS, with a dormant bandwidth part, and with more than one BWP-DownlinkDedicated with pdcch-Config present , otherwise it is absent. |
| MediaTek | Wait for RAN1 input whether we need even these two parameters separately | Same as Nokia’s comment |

## Change 2 – [R2-2003313]:

In last RAN2 meeting, RAN2 agreed that:

- To support beam management in dormancy SCell: 
-       The tci-StatesToAddModList in pdsch-Config IE can be configured for the dormant BWP.
-       if PDSCH-Config is configured in dormant BWP, the UE only applies the TCI state, and doesn’t apply other configurations.
-       pdsch-ConfigCommon IE are not configured for dormant BWP;

However, rbg-Size field in PDSCH-Config IE is mandatory IE, the CR is proposed to be captured in 38.331CR [1].

Rbg-Size Selection between config 1 and config 2 for RBG size for PDSCH. The UE ignores this field if resourceAllocation is set to resourceAllocationType1 (see TS 38.214 [19], clause 5.1.2.2.1) or if PDSCH-Config is configured for dormant BWP.

Question 5: do you agree to capture the change above in 38.331?

| Company | Yes or NO | Comment, if any |
| --- | --- | --- |
| OPPO | Yes |  |
| Qualcomm | No | In latest running CR (R2-2003382), it has captured: pdsch-Config UE specific PDSCH configuration for one BWP. For a dormant BWP, if PDSCH-Config is configured, the UE applies only the TCI state, and does not apply the other configurations. We think it is enough |
| NEC | No | same view as Qualcomm. General description should cover this, rather than adding exceptions for contents (where necessary). |
| Nokia | No | If there is no UE behaviour described for the field in case of BWP is dormant we don’t need to write explicitly UE ignores the field. |
| CATT |  | Capturing under pdsch-Config is sufficient. |
| Huawei | See comments | Tend to agree to capture the limitation in pdsch-Config. |
| Apple | No | We share Qualcomm’s view. |
| Futurewei | No | Capture either what is applied and the rest is not applied (is done), or what is not be applied and the rest is applied should be good enough. |
| LG |  | After checking the latest running CR(R2-2003382) indicated by Qualcomm, we are ok with the CR and the change that we suggested is not need. |
| ZTE | No | Agree with Qualcomm. |
| MediaTek | No | Agree with Qualcomm. |

## Change 3 – [R2-2002789]:

In summary [1]: “The second change in [7] for the condition description of sCellState, i.e. add “Need N” to make sCellState configuration as one-shot configuration. It makes sense and can be included in the 38.331 running CR.

The first change in [7] is due to the sCellState configuration in SCell modification procedure. The default of the SCell state is deactivated and the sCellState configuration is one-shot configuration, no need to specify the deactivated state again. This change can be captured in the 38.331 running CR.”

| SCellAddSync | The field is optional present in case of SCell addition, reconfiguration with sync, and resuming an RRC connection. It is absent otherwise, need N. |
| --- | --- |

##### 5.3.5.5.9 SCell Addition/Modification The UE shall: 1> for each sCellIndex value included in the sCellToAddModList that is not part of the current UE configuration (SCell addition): 2> add the SCell, corresponding to the sCellIndex, in accordance with the sCellConfigCommon and sCellConfigDedicated; 2> if the sCellState is included and set to activated: 3> configure lower layers to consider the SCell to be in activated state; 2> else: 3> configure lower layers to consider the SCell to be in deactivated state; 2> for each measId included in the measIdList within VarMeasConfig: 3> if SCells are not applicable for the associated measurement; and 3> if the concerned SCell is included in cellsTriggeredList defined within the VarMeasReportList for this measId: 4> remove the concerned SCell from cellsTriggeredList defined within the VarMeasReportList for this measId; 1> for each sCellIndex value included in the sCellToAddModList that is part of the current UE configuration (SCell modification): 2> modify the SCell configuration in accordance with the sCellConfigDedicated; 2> if the sCellState is included and set to activated: 3> configure lower layers to consider the SCell to be in activated state;

Question 6: do you agree to capture the two changes above in 38.331?

| Company | Yes or NO | Comment, if any |
| --- | --- | --- |
| OPPO | Yes |  |
| Qualcomm | Yes |  |
| NEC | Yes |  |
| Nokia | No strong view | Change is OK |
| CATT | Yes |  |
| Huawei | For the first change, Yes but see comments; For the second change, No | For the need code, we are fine with need N for absent case, meanwhile for the present case, we feel the need code also should be added as need N. For the second part to delete the sentence “ ” , we do NOT agree. The reason is that in 38331g00 5.3.5.5.2 Reconfiguration with sync, the UE behaviour is captured as “2> consider the SCell(s) of this cell group, if configured, that are not included in the SCellsToAddModList in the RRCReconfiguration message, to be in deactivated state;”, which means for the SCells included in SCellsToAddModList, the UE will not take them as deactivated state, it can only set the SCell state during SCell Addition/Modification procedure in 5.3.5.5.9. Therefore we should not delete the corresponding UE behaviour as proposed here. |
| Apple | Yes |  |
| Futurewei | No to the second change | Have similar view as Huawei. The “else” including two cases: 1) absence of sCellState, 2) the sCellState is included and set to deactivated. If the suggested change is made, in current spec. the SCell state of case 1) is not well defined. |
| LG | Yes |  |
| ZTE | For the first change, Yes For the second change, needs further update | We are ok with the first change. For the second change, we agree with the intention of the CR, but the change is incomplete as indicated by Huawei. For reconfigurationWithSync case, the state of SCells which included in ScellsToAddModList but not configured with sCellState becomes unclear, and those SCells should be deactivated. To take into account all cases, we propose the following two options: Option 1: set all SCell as deactivated in the procedure reconfiguration with sync. clause 5.3.5.5.2 Reconfiguration with sync: 1> else: 2> reset the MAC entity of this cell group; 2> consider the SCell(s) of this cell group, if configured, that are not included in the SCellsToAddModList in the RRCReconfiguration message, to be in deactivated state; Clause 5.3.5.5.9 SCell Addition/Modification 1> for each sCellIndex value included in the sCellToAddModList that is part of the current UE configuration (SCell modification): 2> modify the SCell configuration in accordance with the sCellConfigDedicated; 2> if the sCellState is included and set to activated: 3> configure lower layers to consider the SCell to be in activated state;. Option 2: (as in 36.331) keep procedure text in reconfiguration with sync as it is, and add condition to 5.3.5.5.9 SCell Addition/Modification as following: clause 5.3.5.5.2 Reconfiguration with sync: 1> else: 2> reset the MAC entity of this cell group; 2> consider the SCell(s) of this cell group, if configured, that are not included in the SCellsToAddModList in the RRCReconfiguration message, to be in deactivated state; Clause 5.3.5.5.9 SCell Addition/Modification 1> for each sCellIndex value included in the sCellToAddModList that is part of the current UE configuration (SCell modification): 2> modify the SCell configuration in accordance with the sCellConfigDedicated; 2> If the SCellsToaddModList is received within an RRCResume, an RRCConnectionResume, or an RRCReconfiguration message with ReconfigWithSync: 32> if the sCellState is included and set to activated: 43>configure lower layers to consider the SCell to be in activated state; 32> else: 43>configure lower layers to consider the SCell to be in deactivated state.. |
| MediaTek | For the first change, Yes but see comments; For the second change, No | We have similar view as Huawei. For the option 1 and option 2 suggested by ZTE, we need more time to think about it. |

# Conclusions

# Reference

[1] R2-2003770		Summary of Fast SCell activation (OPPO)

[2]	R2-2002983	RRC Dormant cleanup	Nokia, Nokia Shanghai Bell	CR	Rel-16	38.331	16.0.0	1537	-	F	LTE_NR_DC_CA_enh-Core