3GPP TSG-RAN WG2 Meeting #109bis-e					       R2-20xxxxx

Electronic, 20 April – 30 April 2020

Agenda Item:		6.19

Source:			Huawei, HiSilicon

Title:	[AT109bis-e][043][NR16 Other] Bandwidth combination set to asymmetric bandwidths (Huawei)

WI code(s):			NR_n66_BW

Document for:		Discussion and Decision

## 1	Introduction

This document is to kick off the below offline discussion:

- Bandwidth combination set to asymmetric bandwidths

R2-2002533	LS to RAN2 on introduction of channel bandwidth combination set to asymmetric channel bandwidths (R4-2002852; contact: Huawei)	RAN4	LS in	Rel-16	NR_n66_BW	To:RAN2

R2-2003469	CR on introduction of BCS to asymmetric channel bandwidths (38.331)	Huawei, HiSilicon, Telus	CR	Rel-16	38.331	16.0.0	1563	-	B	NR_n66_BW

R2-2003470	CR on introduction of BCS to asymmetric channel bandwidths (38.306)	Huawei, HiSilicon, Telus	CR	Rel-16	38.306	16.0.0	0289	-	B	NR_n66_BW

R2-2002631	Introduction of asymmetric BW BCS 1	OPPO	CR	Rel-16	38.331	16.0.0	1509	-	B	NR_n66_BW

R2-2002632	Introduction of asymmetric BW BCS 1	OPPO	CR	Rel-16	38.306	16.0.0	0267	-	B	NR_n66_BW

R2-2002633	Introduction of asymmetric BW BCS 0	OPPO	CR	Rel-15	38.306	15.9.0	0268	-	B	NR_n66_BW

R2-2002634	Introduction of asymmetric BW BCS 0	OPPO	CR	Rel-16	38.306	16.0.0	0269	-	A	NR_n66_BW

[AT109bis-e][043][NR16 Other] Bandwidth combination set to asymmetric bandwidths (Huawei)

Scope: Treat papers above on Bandwidth combination set to asymmetric bandwidths

Wanted Outcome: Agreed-in-principle CRs

## 2	Discussion

Based on RAN4 LS R4-2002852, RAN4 agreed to introduce channel bandwidth combination set to asymmetric channel bandwidths defined in clause 5.3.63.6, in TS 38.101-1 with following agreements:

- UE shall support asymmetric channel bandwidth combination set 0 which was defined in Rel-15 by default.

- For n66, support of asymmetric channel bandwidth combination set 1 is optional in Rel16.

### 2.1	Do companies agree to add a new field (BIT STRING) in BandNR to indicate the asymmetric channel bandwidth combination set 1? For example:

[[

asymmetricBandwidthCombinationSet    BIT STRING (SIZE (1..32))                   OPTIONAL

]]

| Company | Yes/No | Comments |
| --- | --- | --- |

R2-2003470 provides the description in TS 38.306 for the new field indicating asymmetric channel bandwidth combination set 1:

| asymmetricBandwidthCombinationSet Defines the supported asymmetric channel bandwidth combination for the band as defined in the TS 38.101-1 [2]. Field encoded as a bit map, where bit N is set to "1" if UE support Asymmetric channel bandwidth combination set N for this band as defined in the TS 38.101-1 [2]. The leading / leftmost bit (bit 0) corresponds to the Asymmetric channel bandwidth combination set 1, the next bit corresponds to the Asymmetric channel bandwidth combination set 2 and so on. UE shall support Asymmetric channel bandwidth combination set 0. | Band | No | No | No |
| --- | --- | --- | --- | --- |

R2-2002632 provides the description in TS 38.306 for the new field indicating asymmetric channel bandwidth combination set 1:

| supportedAsymmetricBandwidthCombinationSet Defines the supported asymmetric channel bandwidth combination set for a band supporting asymmetric channel bandwidth as defined in clause 5.3.6 of TS 38.101-1 [2]. Field encoded as a bit map, where bit N is set to "1" if UE support Bandwidth Combination Set N for this band combination as defined in clause 5.3.6 of TS 38.101-1 [2]. The leading / leftmost bit (bit 0) corresponds to the bandwidth combination set 1. | Band | No | No | No |
| --- | --- | --- | --- | --- |

### 2.2 Companies are encouraged to provide the comments for the descriptions above.

| Company | Comments |
| --- | --- |

R2-2002632 provides the updates for channelBWs-DL/channelBWs-UL in TS 38.306 since it refers to the new field.

channelBWs-DL Indicates for each subcarrier spacing the UE supported channel bandwidths. Absence of the channelBWs-DL (without suffix) for a band or absence of specific scs-XXkHz entry for a supported subcarrier spacing means that the UE supports the channel bandwidths among [5, 10, 15, 20, 25, 30, 40, 50, 60, 80, 100] and [50, 100, 200] that were defined in clause 5.3.5 of TS 38.101-1 version 15.7.0 [2] and TS 38.101-2 version 15.7.0 [3] for the given band or the specific SCS entry. For FR1, the bits in channelBWs-DL (without suffix) starting from the leading / leftmost bit indicate 5, 10, 15, 20, 25, 30, 40, 50, 60 and 80MHz. For FR2, the bits in channelBWs-DL (without suffix) starting from the leading / leftmost bit indicate 50, 100 and 200MHz. The third / rightmost bit (for 200MHz) shall be set to 1. For FR1, the leading/leftmost bit in channelBWs-DL-v1590 indicates 70MHz, and all the remaining bits in channelBWs-DL-v1590 shall be set to 0. NOTE: To determine whether the UE supports a specific SCS for a given band, the network validates the supportedSubCarrierSpacingDL and the scs-60kHz. To determine whether the UE supports a channel bandwidth of 90 MHz, the network may ignore this capability for and validate instead the channelBW-90mhz and the supportedBandwidthCombinationSet. For serving cells with other channel bandwidths the network validates the channelBWs-DL, the supportedBandwidthCombinationSet, the supportedAsymmetricBandwidthCombinationSet (for a band supporting asymmetric channel bandwidth and asymmetric channel bandwidth Combination Set 1 as defined in clause 5.3.6 of TS 38.101-1 [2]) and supportedBandwidthDL.

channelBWs-UL Indicates for each subcarrier spacing the UE supported channel bandwidths. Absence of the channelBWs-UL (without suffix) for a band or absence of specific scs-XXkHz entry for a supported subcarrier spacing means that the UE supports the channel bandwidths among [5, 10, 15, 20, 25, 30, 40, 50, 60, 80, 100] and [50, 100, 200] that were defined in clause 5.3.5 of TS 38.101-1 version 15.7.0 [2] and TS 38.101-2 version 15.7.0 [3] for the given band or the specific SCS entry. For FR1, the bits in channelBWs-UL (without suffix) starting from the leading / leftmost bit indicate 5, 10, 15, 20, 25, 30, 40, 50, 60 and 80MHz. For FR2, the bits in channelBWs-UL (without suffix) starting from the leading / leftmost bit indicate 50, 100 and 200MHz. The third / rightmost bit (for 200MHz) shall be set to 1. For FR1, the leading/leftmost bit in channelBWs-UL-v1590 indicates 70 MHz, and all the remaining bits in channelBWs-UL-v1590 shall be set to 0. NOTE: To determine whether the UE supports a specific SCS for a given band, the network validates the supportedSubCarrierSpacingUL and the scs-60kHz. To determine whether the UE supports a channel bandwidth of 90 MHz the network may ignore this capability for and validate instead the channelBW-90mhz and the supportedBandwidthCombiantionSet. For serving cells with other channel bandwidths the network validates the channelBWs-UL, the supportedBandwidthCombinationSet, the supportedAsymmetricBandwidthCombinationSet (for a band supporting asymmetric channel bandwidth and asymmetric channel bandwidth Combination Set 1 as defined in clause 5.3.6 of TS 38.101-1 [2]) and supportedBandwidthUL.

### 2.3 Companies are encouraged to provide the comments for the descriptions above.

| Company | Comments |
| --- | --- |

In the context of RAN4 LS R4-2002852, RAN4 does not mention whether the new introduced asymmetric channel bandwidth combination set needs to be applied to Rel-15. Technically, we understand that it should be decided by RAN4.

### 2.4	Do companies agree that a LS to RAN4 is needed to ask whether the new field indicating asymmetric channel bandwidth combination set is applied to Rel-15? If there are some other considerations, please feel free to add in the Comments column.

| Company | Yes/No | Comments |
| --- | --- | --- |

## 3 Conclusion

[To be updated]