3GPP TSG-RAN WG2 Meeting #109bis-e					       R2-20xxxxx

Electronic, 20 April – 30 April 2020

Agenda Item:		6.19

Source:			Huawei, HiSilicon

Title:	[AT109bis-e][043][NR16 Other] Bandwidth combination set to asymmetric bandwidths (Huawei)

WI code(s):			NR_n66_BW

Document for:		Discussion and Decision

## 1	Introduction

This document is to kick off the below offline discussion:

- Bandwidth combination set to asymmetric bandwidths

R2-2002533	LS to RAN2 on introduction of channel bandwidth combination set to asymmetric channel bandwidths (R4-2002852; contact: Huawei)	RAN4	LS in	Rel-16	NR_n66_BW	To:RAN2

R2-2003469	CR on introduction of BCS to asymmetric channel bandwidths (38.331)	Huawei, HiSilicon, Telus	CR	Rel-16	38.331	16.0.0	1563	-	B	NR_n66_BW

R2-2003470	CR on introduction of BCS to asymmetric channel bandwidths (38.306)	Huawei, HiSilicon, Telus	CR	Rel-16	38.306	16.0.0	0289	-	B	NR_n66_BW

R2-2002631	Introduction of asymmetric BW BCS 1	OPPO	CR	Rel-16	38.331	16.0.0	1509	-	B	NR_n66_BW

R2-2002632	Introduction of asymmetric BW BCS 1	OPPO	CR	Rel-16	38.306	16.0.0	0267	-	B	NR_n66_BW

R2-2002633	Introduction of asymmetric BW BCS 0	OPPO	CR	Rel-15	38.306	15.9.0	0268	-	B	NR_n66_BW

R2-2002634	Introduction of asymmetric BW BCS 0	OPPO	CR	Rel-16	38.306	16.0.0	0269	-	A	NR_n66_BW

[AT109bis-e][043][NR16 Other] Bandwidth combination set to asymmetric bandwidths (Huawei)

Scope: Treat papers above on Bandwidth combination set to asymmetric bandwidths

Wanted Outcome: Agreed-in-principle CRs

## 2	Discussion

Based on RAN4 LS R4-2002852, RAN4 agreed to introduce channel bandwidth combination set to asymmetric channel bandwidths defined in clause 5.3.63.6, in TS 38.101-1 with following agreements:

- UE shall support asymmetric channel bandwidth combination set 0 which was defined in Rel-15 by default.

- For n66, support of asymmetric channel bandwidth combination set 1 is optional in Rel16.

### 2.1	Do companies agree to add a new field (BIT STRING) in BandNR to indicate the asymmetric channel bandwidth combination set 1? For example:

[[

asymmetricBandwidthCombinationSet    BIT STRING (SIZE (1..32))                   OPTIONAL

]]

| Company | Yes/No | Comments |
| --- | --- | --- |
| OPPO | Yes |  |
| Qualcomm Incorporated | Yes |  |
| Nokia | Yes |  |
| Huawei | Yes |  |

R2-2003470 provides the description in TS 38.306 for the new field indicating asymmetric channel bandwidth combination set 1:

| asymmetricBandwidthCombinationSet Defines the supported asymmetric channel bandwidth combination for the band as defined in the TS 38.101-1 [2]. Field encoded as a bit map, where bit N is set to "1" if UE support Asymmetric channel bandwidth combination set N for this band as defined in the TS 38.101-1 [2]. The leading / leftmost bit (bit 0) corresponds to the Asymmetric channel bandwidth combination set 1, the next bit corresponds to the Asymmetric channel bandwidth combination set 2 and so on. UE shall support Asymmetric channel bandwidth combination set 0. | Band | No | No | No |
| --- | --- | --- | --- | --- |

R2-2002632 provides the description in TS 38.306 for the new field indicating asymmetric channel bandwidth combination set 1:

| supportedAsymmetricBandwidthCombinationSet Defines the supported asymmetric channel bandwidth combination set for a band supporting asymmetric channel bandwidth as defined in clause 5.3.6 of TS 38.101-1 [2]. Field encoded as a bit map, where bit N is set to "1" if UE support Bandwidth Combination Set N for this band combination as defined in clause 5.3.6 of TS 38.101-1 [2]. The leading / leftmost bit (bit 0) corresponds to the bandwidth combination set 1. | Band | No | No | No |
| --- | --- | --- | --- | --- |

### 2.2 Companies are encouraged to provide the comments for the descriptions above.

| Company | Comments |
| --- | --- |
| OPPO | We see quite some commonality of the two proposals, but would like to highlight two things missing in 3470: - The target band should be limited to “band supporting asymmetric channel bandwidth as defined in clause 5.3.6 of TS 38.101-1”, so it is good to clarify; - “the next bit corresponds to the Asymmetric channel bandwidth combination set 2”, we do not have set 2 defined yet. For “UE shall support Asymmetric channel bandwidth combination set 0”, the statement should not be limited to R16 UE only, so we need to find a way to capture that for R15 UE as well (for which, there is no such field of supportedAsymmetricBandwidthCombinationSet - ) |
| Qualcomm Incorporated | Agree to the key points mentioned by OPPO above. It is also good to clarify the absence of the UE capability parameter indicates the UE supports set 0, since the signalling only supports the range from 1 to 32. Such sentence will also cover the default release-15 UE behaviour as seen by the network. |
| Nokia | 3470 looks quite good to us. And we think that OPPOs 2nd comment is not valid. We need to indicate how the field is mapped to 38.101 regardless if such a value is not yet used in RAN4. Regarding first comment from OPPO we consider that text “channel bandwidth combination for the band as defined in the TS 38.101-1 [2].” limits the applicability to any band for which this is defined in 38.101 i.e. no need to clarify that band needs to support asymmetric channel BW as it is clear by referring to 38.101. Anyway one needs to read all the specs to get full picture. |
| Huawei | For the comments from OPPO, we share the same view with Nokia. For 1st comment, the RAN4 spec is referred so it means the target band is the band defined in RAN4 table. For 2nd comment, I agree there is no Asymmetric channel bandwidth combination set 2 yet but it is just explains the logic. For the comment from QC, it is ok to clarify that “If the field is absent, the UE supports asymmetric channel bandwidth combination set 0.” in TS 38.306. |

R2-2002632 provides the updates for channelBWs-DL/channelBWs-UL in TS 38.306 since it refers to the new field.

channelBWs-DL Indicates for each subcarrier spacing the UE supported channel bandwidths. Absence of the channelBWs-DL (without suffix) for a band or absence of specific scs-XXkHz entry for a supported subcarrier spacing means that the UE supports the channel bandwidths among [5, 10, 15, 20, 25, 30, 40, 50, 60, 80, 100] and [50, 100, 200] that were defined in clause 5.3.5 of TS 38.101-1 version 15.7.0 [2] and TS 38.101-2 version 15.7.0 [3] for the given band or the specific SCS entry. For FR1, the bits in channelBWs-DL (without suffix) starting from the leading / leftmost bit indicate 5, 10, 15, 20, 25, 30, 40, 50, 60 and 80MHz. For FR2, the bits in channelBWs-DL (without suffix) starting from the leading / leftmost bit indicate 50, 100 and 200MHz. The third / rightmost bit (for 200MHz) shall be set to 1. For FR1, the leading/leftmost bit in channelBWs-DL-v1590 indicates 70MHz, and all the remaining bits in channelBWs-DL-v1590 shall be set to 0. NOTE: To determine whether the UE supports a specific SCS for a given band, the network validates the supportedSubCarrierSpacingDL and the scs-60kHz. To determine whether the UE supports a channel bandwidth of 90 MHz, the network may ignore this capability for and validate instead the channelBW-90mhz and the supportedBandwidthCombinationSet. For serving cells with other channel bandwidths the network validates the channelBWs-DL, the supportedBandwidthCombinationSet, the supportedAsymmetricBandwidthCombinationSet (for a band supporting asymmetric channel bandwidth and asymmetric channel bandwidth Combination Set 1 as defined in clause 5.3.6 of TS 38.101-1 [2]) and supportedBandwidthDL.

channelBWs-UL Indicates for each subcarrier spacing the UE supported channel bandwidths. Absence of the channelBWs-UL (without suffix) for a band or absence of specific scs-XXkHz entry for a supported subcarrier spacing means that the UE supports the channel bandwidths among [5, 10, 15, 20, 25, 30, 40, 50, 60, 80, 100] and [50, 100, 200] that were defined in clause 5.3.5 of TS 38.101-1 version 15.7.0 [2] and TS 38.101-2 version 15.7.0 [3] for the given band or the specific SCS entry. For FR1, the bits in channelBWs-UL (without suffix) starting from the leading / leftmost bit indicate 5, 10, 15, 20, 25, 30, 40, 50, 60 and 80MHz. For FR2, the bits in channelBWs-UL (without suffix) starting from the leading / leftmost bit indicate 50, 100 and 200MHz. The third / rightmost bit (for 200MHz) shall be set to 1. For FR1, the leading/leftmost bit in channelBWs-UL-v1590 indicates 70 MHz, and all the remaining bits in channelBWs-UL-v1590 shall be set to 0. NOTE: To determine whether the UE supports a specific SCS for a given band, the network validates the supportedSubCarrierSpacingUL and the scs-60kHz. To determine whether the UE supports a channel bandwidth of 90 MHz the network may ignore this capability for and validate instead the channelBW-90mhz and the supportedBandwidthCombiantionSet. For serving cells with other channel bandwidths the network validates the channelBWs-UL, the supportedBandwidthCombinationSet, the supportedAsymmetricBandwidthCombinationSet (for a band supporting asymmetric channel bandwidth and asymmetric channel bandwidth Combination Set 1 as defined in clause 5.3.6 of TS 38.101-1 [2]) and supportedBandwidthUL.

### 2.3 Companies are encouraged to provide the comments for the descriptions above.

| Company | Comments |
| --- | --- |
| OPPO | The effect due to asymmetric bandwidth combination set on channel bandwidth has to be reflected together with the other key factors in the NOTE, so good to clarify. |
| Qualcomm Incorporated | OK to add the new UE capability parameter in the list, but the explanation in parentheses is not needed. |
| Nokia | Ok to have parameter listed in the NOTE but no need for parentheses. Field description of field should make the use of this field clear and no need to repeat. |
| Huawei | OK to add, also agree that parentheses is not needed. |

In the context of RAN4 LS R4-2002852, RAN4 does not mention whether the new introduced asymmetric channel bandwidth combination set needs to be applied to Rel-15. Technically, we understand that it should be decided by RAN4.

### 2.4	Do companies agree that a LS to RAN4 is needed to ask whether the new field indicating asymmetric channel bandwidth combination set is applied to Rel-15? If there are some other considerations, please feel free to add in the Comments column.

| Company | Yes/No | Comments |
| --- | --- | --- |
| OPPO | No | In the incoming LS, it mentioned “• UE shall support asymmetric channel bandwidth combination set 0 which was defined in Rel-15 by default.”, so we understand this set 0 is something existing in R15, and thus does not need to be further confirmed by RAN4. In R16 ran4 spec In R15 RAN4 spec So we believe this set 0 should be supported by Rel-15 UE already. |
| Qualcomm Incorporated | Yes | Good to clarify with RAN4. The question is if we disallow release-15 UEs to support more than set 0. |
| Nokia | Yes and No | We think that release 15 UE cano also support asymmetric bandwidths of future release? Band related capabiliites are regularly release independent. If not all the companies share this view then probably LS to RAN4 is needed. |
| Huawei | Yes | It would be better to ask RAN4 to confirm. |

## 3 Conclusion

[To be updated]