3GPP TSG-RAN WG2 Meeting #109bis-e					R2-20xxxxx

Electronic, 20 April – 30 April 2020

Agenda Item:		6.20.3.1		Open / ongoing proposals

Source:			Huawei

Title:				Summary on eCall over NR

WI code(s):			TEI16

Document for:		Discussion and Decision

## 1	Introduction

In section 6.20.3.1 Open/ongoing proposals, there is an email discussion as below:

- [AT109bis-e][055][TEI16] eCall over NR (Huawei)

Scope: Treat papers above on eCall over NR.

Wanted Outcome: Agreed solution, if possible Agreed-in-principle CR(s)

Deadline: April 28 0700 UTC

The relevant papers are listed in [1] ~ [6]. This paper is to progress on the topic eCall over NR.

## 2	Discussion

Among all papers, [1] is the incoming LS on eCall over NR, and [2] is a discussion paper for potential RAN2 impacts. It is proposed to use [2] as a basline for collecting companies’ opinions.

In [2], the following observations and proposals are provided:

Observation 1: In NR specifications, the eCall over IMS has been defined in stage-2 and stage-3 (signaling aspect), but the parameter eCallOverIMS-Support is not used.

Observation 2: TS 38.304 does not capture anything on eCall only mode like TS 36.304.

Proposal 1: It is proposed to add a clarification for eCall over IMS in TS 38.300:

UEs that are not in limited service state need to only consider eCallOverIMS to determine if eCall over IMS is possible.

Proposal 2: It is proposed to keep the parameter eCallOverIMS-Support in SIB1 and change the condition from “Cond Absent” to “Need R”. The UE should forward the parameter to its upper layer if received in SIB1.

Proposal 3: It is proposed to add a clarification on eCall only mode to TS 38.304, i.e. control and restrict registration for a UE in eCall only mode.

Regarding the above observations and proposals, please companies provide comments into the following table.

| Company | Comments |
| --- | --- |

## 3	Conclusions

[To be updated]

## 4	References

eCall over NR

Moved from AI 3:

[1] R2-2002549	Reply LS on support for eCall over NR (SP-200287; contact: Qualcomm)	SA	LS in	Rel-16	EIEI, 5GS_Ph1	To:SA2, SA5, RAN2, CT1, RAN5	Cc:SA1, SA4, RAN, CT

[2] R2-2003564	Discussion on eCall over IMS for NR	Huawei, HiSilicon	discussion	Rel-16	TEI16

[3] R2-2003565	Introduction of eCall over IMS for NR	Huawei, HiSilicon	draftCR	Rel-16	38.300	16.1.0	C	TEI16

[4] R2-2003566	Introduction of eCall over IMS for NR	Huawei, HiSilicon	draftCR	Rel-16	38.304	16.0.0	C	TEI16

[5] R2-2003567	Introduction of eCall over IMS for NR	Huawei, HiSilicon	draftCR	Rel-16	38.331	16.0.0	C	TEI16

[6] R2-2003568	Draft reply LS on support for eCall over NR	Huawei	discussion	Rel-16	TEI16