3GPP TSG-RAN WG2 Meeting #109bis-e					R2-20xxxxx

Electronic, 20 April – 30 April 2020

Agenda Item:		5.4.3

Source:			Huawei, HiSilicon

Title:				[AT109bis-e][063][NR15] SRS capability for SRS-only SCell (Huawei)

WI code(s):			NR_newRAT-Core

Document for:		Discussion and Decision

## 1	Introduction

This document is to kick off the below offline discussion:

SRS Capability for SRS only SCell

- [AT109bis-e][063][NR15] SRS capability for SRS-only SCell (Huawei)

Scope: Treat R2-2003443, R2-2003444, R2-2003445, R2-2002574

Intended outcome: Agreed-in-principle CRs

Deadline: April 29 0700 UTC

## 2	Discussion

### 2.1	SRS capability for SRS carrier switching in case of SRS-only SCell (R2-2003443, R2-2003444, R2-2003445)

R2-2003443 is the summary of email discussion after RAN2#109-e meeting.

R2-2003444 and R2-2003445 are the corresponding CRs.

As the potential options have already been discussed during the post RAN2#109-e email discussion, it is encouraged by the moderator not to repeat the discussion again unless companies do have strong concern on the current proposal. It is rather realistic to focus on the contents of the proposed CRs.

Q1: Do companies agree with the proposal in the above papers?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Qualcomm Incorporated | Yes |  |

Q2: Do companies have specific comments on the CRs?

| Company | Yes/No | Comments |
| --- | --- | --- |

### 2.2 UE capability signalling for simultaneous SRS antenna and carrier switching (R2-2002574)

R2-2002574 is to address the case of simultaneous SRS antenna switching and SRS carrier switching. For LTE, RAN2 received LS from RAN1 in R2-1900008/R1-1813779 indicating when operating with SRS carrier switching, the antenna switching capability is the one for the switched band combination. It is proposed to apply this also to NR.

Q3: Do companies agree with the proposal in the CR?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Qualcomm Incorporated | Yes (proponent) | Sorry we did not bring release-16 shadow, but the intention is to do Cat.A change in release-16. |

Q3: Do companies have specific comments on the CR?

| Company | Yes/No | Comments |
| --- | --- | --- |

## 3 Conclusions

[To be updated]

## 4 References

[1] R2-2003443	[Post109e#25][NR15] SRS Capability report for SRS only Scell summary    Huawei, HiSilicon    discussion    Rel-15    NR_newRAT-Core

[2] R2-2003444	SRS Capability report for SRS only Scell    Huawei, HiSilicon    CR    Rel-15    38.331    15.9.0    1559    -    F    NR_newRAT-Core

[3]R2-2003445	SRS Capability report for SRS only Scell    Huawei, HiSilicon    CR    Rel-16    38.331    16.0.0    1560    -    A    NR_newRAT-Core

[4] R2-2002574	Correction on UE capability signalling for simultaneous SRS antenna and carrier switching    Qualcomm Incorporated    CR    Rel-15    38.306    15.9.0    0265    -    F    NR_newRAT-Core