3GPP TSG-RAN WG2 #109 E-meeting BIS	R2-200xxxx

Electronic Meeting, 20th – 30th April, 2020

Agenda item:       6.0.1

Source:	ZTE , Sanechips, Samsung, Intel Corporation

Title:	[AT109bis-e][069][NR RIL] DiscMail5 and DiscMail6 (ZTE)

Document for:	Discussion and Decision

# Introduction

This is the email discussion as below:

* [AT109bis-e][069][NR RIL] DiscMail5 + DiscMail6 (ZTE)

Scope: Discussion and implementation of review issues.

Wanted outcome: a) Agreed RIL Status update in the email discussion report b) Agreed ASN.1/procedure text proposal included in the email discussion report.
After email discussion report is agreed, the TPs will be included in the ASN.1 Review file, for the continued ASN.1 review.

Deadline:  Email discussion Stop at EOM, April 30

# Discussions

## Z132/Z134

| Z132 |  | Z(DF) |  | Seems this IE is actually not a list. So, the word “List” in the name can be deleted. Also, there is no need for setup release since there is already release list in this IE. | Modify the definition as follows: sps-Config-r16 SPS-ConfigList-r16 OPTIONAL, -- Need M |
| --- | --- | --- | --- | --- | --- |
| Z134 | Z(DF) |  |  | Since the IE configuredGrantConfigList-r16 is not a list, we may remove the word’list’ from the name , in addition, since there is a list configured into this IE, maybe the setupRelease is not suitable. | configuredGrantConfig-r16 ConfiguredGrantConfigList-r16 OPTIONAL -- Need M |

For the first change, the naming change: According to the naming rule, the parameter named as “list” is often used for a list. However, the IE sps-ConfigList-r16 and configuredGrantConfiglist-r16 seems not a list. We suggest to change the name of  sps-ConfigList-r16 and configuredGrantConfigList-r16 to sps-Config-r16 and configuredGrantConfig-r16 accordingly.

Q1-1: Do companies agree the issue about IE names raised in Z132/134?

| Company | Yes/No | Reason |
| --- | --- | --- |
| ZTE | Yes |  |

Q1-2: For companies who have positive attitudes on Q1-1, do you agree with the suggestion from Z132/134? If not, please provide your suggestion

| Company | Yes/No | Suggestion |
| --- | --- | --- |
| ZTE | Yes |  |

For the change of parameter type, we have the following two reasons:

Since release list SPS-ConfigToReleaseList-r16 and ConfiguredGrantConfigToReleaseList-r16 are included in the SPS-ConfigList and ConfiguredGrantConfigList accordingly, it seems the setup/release structure is not needed for the SPS-ConfigList  and ConfiguredGrantConfigList. NW can release SPS list and CG list via SPS-ConfigToReleaseList-r16 and ConfiguredGrantConfigToReleaseList-r16 accordingly.

In addition,  in the IE BWP-UplinkDedicated and BWP-DownlinkDedicated, the configuredGrantConfig and sps-Config is configured with the type of setup/release as well, it will be difficult to understand the case that configuredGrantConfig and sps-Config are configured as release but the SPS-ConfigList and ConfiguredGrantConfigList are configured as setup.

From above reasons, we would like to change the setup/release type to the normal type.

Q 1-3: Do companies agree the type shall be changed according to Z132/134?

| Company | Yes/No | Reason |
| --- | --- | --- |
| ZTE | Yes |  |

Q1-4: For companies who have positive attitudes on Q1-1, do you agree with the suggestion from Z132/134? If not, please provide your suggestion

| Company | Yes/No | Suggestion |
| --- | --- | --- |
| ZTE | Yes |  |

## S201/S202/Z133/Z135

| S201 | Samsung (Sangkyu) |  | In the field description of SPS-ConfigList-r16, a restriction "Except for reconfiguration with sync, the NW does not reconfigure a SPS configuration when it is active (see TS 38.321 [3]). However, the NW may release a SPS configuration at any time." is not for SPS-ConfigList but for SPS-configToAddModList. | Move “Except for reconfiguration with sync, the NW does not reconfigure a SPS configuration when it is active (see TS 38.321 [3]). However, the NW may release a SPS configuration at any time.” To SPS-ConfigToAddModList in SPS-ConfigList IE. |
| --- | --- | --- | --- | --- |

| S202 | Samsung (Sangkyu) |  | In the field description of configuredGrantConfigList, a restriction "Except for reconfiguration with sync, the NW does not reconfigure a Type 2 configured grant configuration when it is active (see TS 38.321 [3]). However, the NW may release a configured grant configuration at any time." is not for configuredGrantConfigList for configuredGrantConfigToAddModList in ConfiguredGrantConfigList. | Move “Except for reconfiguration with sync, the NW does not reconfigure a Type 2 configured grant configuration when it is active (see TS 38.321 [3]). However, the NW may release a configured grant configuration at any time." to field description of configuredGrantConfigToAddModeList in ConfiguredGrantConfigList. |
| --- | --- | --- | --- | --- |

| Z133 | Z(DF) |  | Since there is release list in this IE, it seems the network can add or remove one or more specific SPS. So, the field description seems inaccurate? | UE specific multiple SPS (Semi-Persistent Scheduling) configurations for one BWP. Except for reconfiguration with sync, the NW does not reconfigure a SPS configuration other than sps-ConfigDeactivationStateList-r16 when it is active (see TS 38.321 [3]). However, the NW may release a SPS configuration at any time. |
| --- | --- | --- | --- | --- |

| Z135 | Z(DF) |  | Since there is release list in this IE, it seems the network can add or remove this. So, the field description seems inaccurate? | Except for reconfiguration with sync, the NW does not reconfigure a Type 2 configured grant configuration other than configuredGrantConfigType2DeactivationStateList-r16 when it is active (see TS 38.321 [3]). |
| --- | --- | --- | --- | --- |

Above RILs state the similar issue,  the sentence “Except for reconfiguration with sync, the NW does not reconfigure a Type 2 configured grant configuration/SPS when it is active (see TS 38.321 [3]). However, the NW may release a configured grant configuration at any time.” in the current field description of SPS-ConfigList-r16 and configuredGrantConfigList-r16 seems not appropriate.

Q2-1: Do companies agree the issue raised in S201/S202 and Z133/Z135?

| Company | Yes/No | Reason |
| --- | --- | --- |
| ZTE | Yes | We also think the current field description of the configuredGrantConfigList and SPS-ConfigList is not appropriate. |

According to the suggestion from S201-S202, the sentence  “Except for reconfiguration with sync, the NW does not reconfigure a Type 2 configured grant configuration/SPS configuration when it is active (see TS 38.321 [3]). However, the NW may release a configured grant configuration/SPS configuration  at any time.” shall be moved to the IE configuredGrantConfigToAddModList in ConfiguredGrantConfigList and to SPS-ConfigToAddModList in SPS-ConfigList IE accordingly.

Q2-2: For companies who have positive attitudes on Q2-1, do you agree with the suggestion from S201/S202? If not, please provide your suggestion

| Company | Yes/No | Suggestion |
| --- | --- | --- |
| ZTE | Yes |  |

## Z139

| Z139 | Z(DF) |  | sps-ConfigIndex is only needed in case of multiple SPS configuration being present. So, we can make this a conditional field | make the field conditional field (e.g. Cond Multiple-SPS) |
| --- | --- | --- | --- | --- |

In the current IE sps-Config, the sps-ConfigIndex is configured for identifying the specific sps-config in the case that multiple SPS configuration are configured within one given BWP. In a natural way , This IE need be absent  if only one SPS configuration is configured for this BWP, otherwise, this IE shall be present.

Q3-1: Do companies agree the issue raised in Z139?

| Company | Yes/No | Reason |
| --- | --- | --- |
| ZTE | No | For the case that only one SPS configuration is configured, it is simpler from allocation of an ID if NW would like to add other SPS configurations to this BWP, otherwise, NW need to remove this SPS configuration first and then add other SPS configuration with allocated ID. Thus we think there is no need to have a condition for this IE. |

For this change, the suggestion from z139 is shown as following:

SPS-Config information element

-- ASN1START

-- TAG-SPS-CONFIG-START

SPS-Config ::=                  SEQUENCE {

periodicity                     ENUMERATED {ms10, ms20, ms32, ms40, ms64, ms80, ms128, ms160, ms320, ms640,

spare6, spare5, spare4, spare3, spare2, spare1},

nrofHARQ-Processes              INTEGER (1..8),

n1PUCCH-AN                      PUCCH-ResourceId                                                            OPTIONAL,   -- Need M

mcs-Table                       ENUMERATED {qam64LowSE}                                                     OPTIONAL,   -- Need S

...,

[[

sps-ConfigIndex-r16         SPS-ConfigIndex-r16                                                             OPTIONAL,   -- Cond Multiple-SPS

harq-ProcID-Offset-r16      INTEGER (0..15)                                                                 OPTIONAL,   -- Need N

periodicityExt-r16          INTEGER (1..5120)                                                               OPTIONAL,   -- Need N

harq-CodebookID-r16         INTEGER (1..2)                                                                  OPTIONAL    -- Need N

]]

}

-- TAG-SPS-CONFIG-STOP

-- ASN1STOP

| Conditional Presence | Explanation |
| --- | --- |
| Multiple-SPS | This fields is present if more than 1 SPS configuration are configured to the associated DL BWP. It is absent otherwise. |

Q3-2 : For companies who have positive attitudes to Q3-1, do you agree with the suggestion from Z139? If not, please provide your suggestion.

| Company | Yes/No | Suggestion |
| --- | --- | --- |

I665~I667

| I665 | Intel (Sudeep) |  | Looks like stored configuration and hence can’t be Need N. | Change to M/R depending on how to release the field. Same for the other fields here. |
| --- | --- | --- | --- | --- |

According to the I665, it indicates the sps-ConfigIndex-r16 included in SPS-config is optional with Need-N however, the sps-ConfigIndex-r16 seems a stored information element , and then it shall be corrected to Need M or R

Q4-1: Do companies agree the issue raised in I665?

| Company | Yes/No | Reason |
| --- | --- | --- |
| ZTE | Yes |  |

Q4-2: For the companies who have positive attitude on Q4-1, pleas provide your suggestion on the optional type: Need R, Need M, etc.

| Company | Type | Reason |
| --- | --- | --- |
| ZTE | R or S | Either Need -R or Need S seems fine for us. We have slight preference on Need R with the description that “if not configured, the value 0 shall be applied” |

| I666 | Intel (Sudeep) |  | Can’t be Need N as it looks like a stored configuration. | Change to Need M/R as appropriate. Same for sps-PUCCH-AN-ListPerCodebook-r16 |
| --- | --- | --- | --- | --- |

According to the I666, it points out the sps-ConfigDeactivationStateList-r16 included in SPS-config is optional with Need-N, however, the sps-ConfigDeactivationStateList-r16 seems a stored information element , and then it shall be corrected to Need M or R

Q5-1: Do companies agree the issue raised in I666?

| Company | Yes/No | Reason |
| --- | --- | --- |
| ZTE | Yes | Agree with above that sps-ConfigDeactivationStateList-r16 shall be stored by UE. |

Q5-2 For the companies who have positive attitudes on Q5, pleas provide your suggestion on the optional type: Need R, Need M, or etc.

| Company | Type | Reason |
| --- | --- | --- |
| ZTE | R or M | Even though Need R or M can be accepted by us, we still have the concern: What’s the UE behavior in the case that this IE is absent. |

| I667 | Intel (Sudeep) |  | Can’t be Need N as it looks like a stored configuration. | Change to Need M/R as appropriate. Could also be mandatory as there are no other fields besides the ID. |
| --- | --- | --- | --- | --- |

According to the I666, it points out the  maxPayloadSize-r16 included in SPS-PUCCH-AN is optional with Need-N, however, the maxPayloadSize-r16  seems a stored information element , and it shall be corrected to Need M or R

Q6-1: Do companies agree the issue raised in I667?

| Company | Yes/No | Reason |
| --- | --- | --- |
| ZTE | Yes | Similar with the maxPayloadSize defined in PUCCH-ResourceSet, it is defined for UE to determine to use which resource to send ACK/NACK information for SPS DL transmisson, the detail can be shown as following: If the UE is provided SPS-PUCCH-AN-List and transmits UCI information bits that include only HARQ-ACK information bits in response to one or more SPS PDSCH receptions, the UE determines a PUCCH resource to be - a PUCCH resource with SPS-PUCCH-AN-ResourceID = 0 if , or - a PUCCH resource with SPS-PUCCH-AN-ResourceID = 1, if provided, if where is either provided by maxPayloadSize in SPS-PUCCH-AN-List for SPS-PUCCH-AN-ResourceID = 1 or is otherwise equal to 1706, or - a PUCCH resource with SPS-PUCCH-AN-ResourceID = 2, if provided, if where is either provided by maxPayloadSize in SPS-PUCCH-AN-List for SPS-PUCCH-AN-ResourceID = 2 or is otherwise equal to 1706, or - a PUCCH resource with SPS-PUCCH-AN-ResourceID = 3, if provided, if where is equal to 1706. Thus we think it shall be aligned with the current need code of maxPayloadsize defined in PUCCH-ResourceSet. |

Q6-2: For the companies who have positive attitude on Q5, please provide your suggestion on the optional type: Need R, Need M, or etc.

| Company | Type | Reason |
| --- | --- | --- |
| ZTE | Need-R | See above reason. |

# Conclustion

Added after email discussion