3GPP RAN WG2 Meeting #109bis-e	DRAFT_R2-2003894
Online, 20 – 30 April 2020

Agenda Item:	6.16.3

Source:	Nokia

Title:	Offline 104: Timer based BFR MAC CE Transmission

Document for:	Discussion, Decision

# Introduction

This is the summary report for TDocs submitted to the eMIMO agenda item for the following topic:

- [AT109bis-e][104][EMIMO] Timer based BFR MAC CE Transmission (Nokia) Scope: Discuss the proposals for timer based BFR MAC CE Transmission based on R2-2002796, R2-2003589 and R2-2003712 Initial intended outcome: summary of the offline discussion with list of proposals Initial deadline (for companies' feedback): Thursday 2020-04-23 07:00 UTC Initial deadline (for rapporteur's summary in R2-2003894): Thursday 2020-04-23 16:00 UTC

# SCell BFR MAC CE transmission

## Frequent SCell BFR triggering

The current SCell Beam Failure Dectection (BFD) and Recovery (BFR) procedure is captured in the TS 38.321 as follows:

- 5.17 Beam Failure Detection and Recovery procedure (…) The MAC entity shall for each Serving Cell configured for beam failure detection: 1> if beam failure instance indication has been received from lower layers: 2> start or restart the beamFailureDetectionTimer; 2> increment BFI_COUNTER by 1; 2> if BFI_COUNTER >= beamFailureInstanceMaxCount: 3> if the Serving Cell is SCell: 4> trigger a BFR for this Serving Cell; 3> else: 4> initiate a Random Access procedure (see clause 5.1) on the SpCell. 1> if the beamFailureDetectionTimer expires; or 1> if beamFailureDetectionTimer, beamFailureInstanceMaxCount, or any of the reference signals used for beam failure detection is reconfigured by upper layers associated with this Serving Cell: 2> set BFI_COUNTER to 0. 1> if the Serving Cell is SpCell and the Random Access procedure is successfully completed (see clause 5.1): 2> set BFI_COUNTER to 0; 2> stop the beamFailureRecoveryTimer, if configured; 2> consider the Beam Failure Recovery procedure successfully completed. 1> else if the Serving Cell is SCell, and a PDCCH addressed to C-RNTI indicating uplink grant for a new transmission is received for the HARQ process used for the transmission of the SCell BFR MAC CE or truncated SCell BFR MAC CE which contains beam failure recovery information of this Serving Cell; or 1> if the SCell is deactivated as specified in clause 5.9: 2> set BFI_COUNTER to 0; 2> consider the Beam Failure Recovery procedure successfully completed and cancel all the triggered BFRs for this Serving Cell. The MAC entity shall: 1> if the Beam Failure Recovery procedure determines that at least one BFR has been triggered and not cancelled: 2> if UL-SCH resources are available for a new transmission: 3> if the UL-SCH resources can accommodate the SCell BFR MAC CE plus its subheader as a result of LCP: 4> instruct the Multiplexing and Assembly procedure to generate the SCell BFR MAC CE. 3> else if the UL-SCH resources can accommodate the truncated SCell BFR MAC CE plus its subheader as a result of LCP: 4> instruct the Multiplexing and Assembly procedure to generate the truncated SCell BFR MAC CE. 2> else: 3> trigger the SR for SCell beam failure recovery.

BFD is performed in the yellow highlighted part above, BFR request in the turquoise part, and successful BFR with BFR termination in the green part.

TDocs [1,2,3] all point out the same issue with the current procedure: when a BFR is triggered by the BFD and SCell BFR MAC CE is transmitted, whenever the next beam failure instance indication is received from lower layers, the BFR is triggered again in case NW didn’t provide UE with ACK to the transmitted SCell BFR MAC CE or deactivate the given SCell before that occasion. Hence, the UE attempts to send the SCell BFR MAC CE again – it should be noted that the beam failure instance indication may be provided with the minimum periodicity of 2ms by lower layers which would lead to very frequent BFR triggers. If there is no UL resources available, UE would trigger SR procedure for SCell BFR.

Furthermore, if multiple SCells failed, the SCell BFR MAC CE/BFR SR transmission would be triggered each time any of these SCells got beam failure instance indication next time from lower layers. This can be problematic to the NW since it would not know if there were other SCell(s) triggering new beam failure event(s) or whether the BFR SR was due to the already failed SCell(s).

In fact, currently, the BFR remains pending even after transmitting the SCell BFR MAC CE and UE would immediately attempt to transmit the SCell BFR MAC CE again. This is proposed to be fixed by [4], however, the above problem remains even if the BFR was cancelled after SCell BFR MAC CE transmission.

Question 1: Do you agree the current behavior may lead to frequent and redundant SCell BFR MAC CE and/or SCell BFR SR transmissions by the UE?

| Company | Reply (y/n) | Additional comments |
| --- | --- | --- |
| vivo | Y |  |
| Samsung | yes |  |
| Lenovo | Yes |  |
| Apple | Yes |  |
| APT | Yes | Since the network is not required to provide a BFR response (i.e., indicating an UL grant for new transmission) within a defined period, the period between MAC CE transmission and the network’s BFR response could be long. As a result, the UE is likely to frequently trigger and/or transmit BFR MAC CE/SR during this period. |
| CATT | Maybe | We are not sure how serious the issue of repeated BFR trigger or BFR for multiple SCell is. For the 2nd issue in [4], it seems a valid issue for us. |
| Ericsson | Maybe | Depends how often the MAC entity checks/executes the second line in the turquoise part and this is not specified. |

Since NW may not be able to provide UE with response to the SCell BFR request before the next occasion of the SCell BFR MAC CE transmission, for instance, due to HARQ retransmissions required to transmit the first TB carrying the SCell BFR MAC CE, it seems desirable to prevent UE from transmitting SCell BFR MAC CE or SCell BFR SR in vain.

Question 2: If you agree with Question 1, do you agree to introduce a mechanism to prevent UE from transmitting the frequent and redundant SCell BFR MAC CE and/or SCell BFR SR?

| Company | Reply (y/n) | Additional comments |
| --- | --- | --- |
| vivo |  | No strong view, but ok to have a simply solution to avoid redundant report. |
| Samsung |  | No strong view. |
| Lenovo | Yes |  |
| Apple | Yes |  |
| APT | Yes |  |
| CATT |  | No strong view. Please see our comments to Q1. |
| Ericsson |  | No strong view. |

Question 3: If you do not agree with Question 2, how should the issue presented in Question 1 be solved?

| Company | Additional comments |
| --- | --- |
| Samsung | One simple option could be to just reset the BFI_COUNTER |
| Ericsson | Samsung proposal could work. Another could be to reuse the BFR timer. |

## Solution alternatives

TDocs [1,2,3] all propose similar solution to the issue presented in section 2.1, the common part of the proposals can be summarized into Solution alternative 1:

Solution alternative 1: A timer is introduced to prevent UE from triggering new SCell BFR procedure when the BFR procedure is already ongoing for the given SCell – the timer runs per Serving Cell and works as follows.

- Started upon transmission of SCell BFR MAC CE consisting of BFR information for the given SCell;

- When the timer is running, new SCell BFR procedure cannot be triggered;

- Stopped upon:

  - receiving a PDCCH addressed to C-RNTI indicating uplink grant for a new transmission for the HARQ process used for the transmission of the SCell BFR MAC CE or truncated SCell BFR MAC CE which contains beam failure recovery information of this Serving Cell; or

  - the SCell deactivation.

- Upon expiry, SCell BFR procedure is triggered for the given SCell.

Question 4: Do you agree to support the timer in Solution alternative 1 to solve the problem in Question 1?

| Company | Reply (y/n) | Additional comments |
| --- | --- | --- |
| vivo | N | We are ok to prevent the redundant report via a prohibit timer, but don’t agree with the UE behaviors regarding the expiry of the timer. If we want to introduce a timer, the timer should not be used for the retransmission of the BFR MAC CE, as the intention of the timer is to prevent redundant BFR report. The trigger of the SCell BFR can still be kept as it is when the timer expires. |
| Samsung | N | One simple option could be to just reset the BFI_COUNTER |
| Lenovo | Yes | We would prefer to have a similar solution as for Rel-15 BFR, where UE monitors during some time window for a response message the BFR from network to. A timer is a simple and efficient mechanism to achieve such functionality. It will avoid frequent and redundant SCell BFR MAC CE and/or SCell BFR SR and also at the same avoids a situation where the SCell BFR procedure is stuck when no ACK is received from the NW. Resetting the BFI_COUNTER for example will not provide a complete solution as it doesn’t address the case when no ACK is received from NW, therefore we would prefer the timer based solution. |
| Apple | Yes | For the UE operation upon timer expiry, the new SCell BFR should be triggered if the SCell is still enduring the beam failure problem, otherwise, there is no way to recover the problematic SCell. |
| APT | Yes, but… | In general, we support the timer-based solution, but we consider the UE should keep monitoring PDCCH while the timer is running. The concept is similar to RAR window of SpCell BFR case. This behavior was also proposed in [2] and [3], but it seems not being captured in the solution alternative 1. |
| CATT |  | We are not sure if a timer is really necessary. But we are fine to discuss the changes proposed in [4]. |
| Ericsson |  | Why is it one timer per serving cell? The UE would then anyway be able to send/trigger one MAC CE/SR per failed SCell. Samsung proposal is probably good enough. If the signaling is important, use RRC. |

Additionally, [1] proposes a recovery mechanism if NW does not provide response to the SCell BFR request even after number of SCell BFR MAC CE transmissions, for instance, if all the transmissions happened over failed SCell.

The proposal is to increase a counter by 1 for each transmission of SCell BFR MAC CE without NW response and upon the counter hitting a configured maximum number threshold, RA procedure is triggered over SpCell.

Question 5: If you agree to Question 4, do you agree additionally to support recovery mechanism via RA procedure after number of failed SCell BFR MAC CE transmissions?

| Company | Reply (y/n) | Additional comments |
| --- | --- | --- |
| vivo | No | If the transmission of the SCell BFR MAC CE keeps failing, the UE would be expected to trigger RLF due to the very bad radio condition. No additional handling is needed. Actually, it is not clear to us how the RACH procedure (without triggering RRC reestablishment) can ensure the transmission of the SCell BFR MAC CE. For example, the SCell BFR MAC CE could be transmitted via a configured grant (i.e. before the RAR reception) even though the RACH is triggered. |
| Samsung | No |  |
| Lenovo | Yes | As replied to Q4 we prefer to have a complete solution |
| Apple | Yes | Since RLM/RLF is not supported on SCell, there is no way for UE to indicate its problem. If RACH procedure can be used as the last try for SCell BFR MAC CE transmission, RACH success means the BFR success, and RACH failure would trigger RLF. |
| APT | No | The BFR MAC CE could be transmitted on any available UL resource of any serving cells (including SpCell). In the case that BFR MAC CE failed to be transmitted for a number of times may reflect that the UE is suffering from poor radio condition. The legacy mechanisms to recover the bad channel condition, e.g., measurement report, SpCell BFR, RLF, etc. may be enough. |
| CATT | No | Previously we discussed a lot on possible handling of sending BFR MAC CE from failed CC, but then no optimization agreed. In this late stage we do not see a strong need to open the door again. We can consider those in a later release if issue identified in field. |
| Ericsson | No |  |

Naturally, there is possibility for other solution alternatives or tweaks for the solution presented above. Those can be provided below.

Question 6: Do you have another solution alternative, please specify it below?

| Company | Additional comments |
| --- | --- |

### Solution alternative X

Specify solution here and indicate in the table above.

# Conclusion

[TBA]

# Text proposal

[TBA]

# References

- R2-2002796, Timer based BFR MAC CE Transmission, Apple, Nokia, Nokia Shanghai Bell

- R2-2003589, Remaining issues on BFR on SCell, ZTE, Sanechips

- R2-2003712, Remaining issues on SCell BFR, Huawei, HiSilicon

- R2-2003052, Draft CR on Corrections for SCell BFR procedure, Nokia, Nokia Shanghai Bell, Apple