3GPP TSG-RAN2 Meeting #109bis E-meeting	    R2- 20xxxxx

20 – 30 Apr 2020

Agenda item:	6.15.2

Source	LG Electronics Inc.

Title	Discussion on CLI specific RILs

Document for	Discussion and Decision

# Introduction

In this contribution, we discuss some CLI specific RILs ([I000], [I001], [S601], [S602], [E122] and [E123]) to reflect the results in the output of below discussion.

[AT109bis-e][109][CLI] RRC CR (LG)

Scope: discuss the 38.331 CRs based on R2-2002911

Intended outcome: In-principle agreed 38.331 CR

Deadline for companies' feedback:  Tuesday 2020-04-2810:00 UTC

Deadline for rapporteur's version for agreement:  Wednesday 2020-04-29 10:00 UTC

Status: Ongoing

# Discussion

## I001

| ID | Class | Subclause/IE | Description and Proposed change |
| --- | --- | --- | --- |
| I001 | 2 | 5.5.3.1 | Does it include CSI-RSSI? If yes, we need to add ‘except for RSSI and CSI-RSSI'. If not, we may need to clarify CSI-RSSI still required layer 3 filtering. |

Companies are requested to provide input on I001:

| Company | Comment and proposed change |
| --- | --- |
| LG | The change is not needed. L3 filtering is applied to the CLI-RSSI measurements. |

## I000

| ID | Class | Subclause/IE | Description and Proposed change |
| --- | --- | --- | --- |
| I000 | 2 | 5.5.4.1 | Upon expiry of the periodical reporting timer’ should be replaced with ‘upon expiry of the periodical reporting timer or CLI periodical reporting timer’. Because the regular periodically timer is different than CLI periodically timer. |

Companies are requested to provide input on I000:

| Company | Comment and proposed change |
| --- | --- |
| LG | For a measurement id only one periodical timer can be configured, so it is clear that the periodical timer mapped to the CLI measurement object is the periodical timer for CLI. We prefer to keep the current wording. |

## E122

| ID | Class | Subclause/IE | Description and Proposed change |
| --- | --- | --- | --- |
| E122 | 3 | SRS-Config | SRS-Config not used by CLI. MeasObjectCLI uses SRS-Resource, not SRS-Config. “…or to configure sounding reference signal measurements for CLI” should be deleted. |

Companies are requested to provide input on E122:

| Company | Comment and proposed change |
| --- | --- |
| LG | Agree with Ericsson `s suggestion. |

## E123

| ID | Class | Subclause/IE | Description and Proposed change |
| --- | --- | --- | --- |
| E123 | 2 | maxNrofSRS-Resources-16 | Missing “CLI” in maxNrofSRS-Resources-16. should rename to maxNrofSRS-ResourcesCLI-r16, to avoid confusion. |

Companies are requested to provide input on E123:

| Company | Comment and proposed change |
| --- | --- |
| LG | Agree with Ericsson `s suggestion. |

## S601, S602

These are already in rapporteur CR. Please provide your comment in the CR directly.

# Conclusion