3GPP TSG-RAN WG2 #109b-e	R2-200xxxx

Electronic, 20 Apr – 30 Apr 2020

Source: 	Huawei, HiSilicon

Title: 	Report of [AT109bis-e][205][MOB] Flagging and discussion of DAPS UP open issues for PDCP/RLC/MAC (Huawei)

Agenda Item:	7.3.2.1

Document for:	Discussion and decision

# Introduction

This is the report of the following offline discussion:

- [AT109bis-e][205][MOB] Flagging and discussion of DAPS UP open issues for PDCP/RLC/MAC (Huawei) Scope: - Companies flagging critical DAPS UP issues requiring Web conference discussion - Discuss the remaining UP open issues identified in email discussion report of Post109#11 in R2-2003371. Intended outcome: - Discussion summary document in R2-2003845, including resolutions to open issues and identification of non-critical issues that should no longer be pursued in Rel-16 Deadlines for flagging issues for Web conference discussion: - Flagging of issues for the Web conference: Tuesday 2020-04-21 10:00 UTC - Rapporteur summary: Tuesday 2020-04-21 11:30 UTC Deadlines for providing comments and for rapporteur inputs: - Initial deadline (for companies' feedback): Thursday 2020-04-23 10:00 UTC - Initial deadline (for rapporteur's summary in R2-2003845): Friday 2020-04-24 08:00 UTC - Proposed agreements in R2-2003845 indicated for email agreement and not challenged until Tuesday 2020-04-28 12:00 UTC will be declared as agreed by the session chair.

This offline discussion can be divided into two phases:

First phase: flagging phase.

Since we already have several proposals for DAPS UP issues in email discussion Post109#11 [1], firstly companies can flag any proposal that you think may benefit from further online discussion.

And for the issues that are identified for further discussion in Post109#11, companies can also pick which ones are suitable for online discussion.

Second phase: resolving open issues.

For open issues identified in Post109#11, we can try to make more progress here if no corresponding agreements are made online.

# Discussion

## First phase: flagging phase

### For issues with proposals in Post109#11:

##### MAC part:

- Stopping RACH in source:

Proposal S2.1-1: All the functions in Figure 4.2.2-1 will be supported by the source and target MAC entity in DAPS HO.

Proposal S3.9: Follow proposal S2.1-1, RACH is allowed to source after RACH towards target is successful.

2. 2-step RACH applied to DAPS:

Proposal S2.1-2: UE switches the UL PDCP data transmission upon successful RACH procedure (i.e. Msg.B for 2-step RACH).

| Company | Is there any issue/proposal that need to be flagged for online discussion? (Y or N) | Comments (including which one/ones) |
| --- | --- | --- |
| Ericsson | N |  |

##### PDCP part:

- PDCP status report for DAPS UM DRB:

Proposal S2.2-1-1: The PDCP status report for DL UM DRBs is needed for DAPS HO.

Proposal S2.2-2-1: The second PDCP status report for DL UM DRBs is not needed for DAPS HO.

Proposal S3.2: PDCP status report for UM is mandatory for DAPS capable UE.

- ROHC handling in case of DAPS HO without key change:

Proposal S2.3-5-1: For DAPS DRBs, keep original agreements,i.e. separate RoHC context shall be applied for the source and target link even if DAPS handover is performed without key change

- PDCP SN number continuity for UM DRB:

Proposal S2.6-2: Keep original agreement that RLC UM (UL/DL) with PDCP SN number continuity is supported for DAPS.

- Discarding indication from PDCP to RLC:

Proposal S2.6-3: Do not introduce discard indication in source from PDCP to RLC upon UL switching.

- Discard timer:

Proposal S3.7-3: Discard timer is maintained during DAPS HO.

- PDCP status report to reflect holes from Source NB before DAPS HO:

Proposal S3.12: Do not introduce special handling on PDCP status report to support DAPS HO.

- LTE specific PDCP handling:

Proposal S2.6-5-1: Reordering_PDCP_RX_COUNT used for AM DRB reordering is needed for DAPS DRB.

Proposal S2.6-5-2: Last_Submitted_PDCP_RX_SN and Reordering_PDCP_RX_COUNT used for AM DRB reordering are needed for DAPS DRB.

Proposal S2.6-5-3: Reordering_PDCP_RX_COUNT is set to the COUNT value associated to RX_HFN and Next_PDCP_RX_SN upon PDCP reconfiguration for LTE UM DRB and LTE AM DRB without reordering from normal PDCP to DAPS PDCP.

Proposal S2.6-5-4: Last_Submitted_PDCP_RX_SN is set to [(Next_PDCP_RX_SN-1) modulo (Maximum_PDCP_SN+1)] upon PDCP reconfiguration for LTE UM DRB from normal PDCP to DAPS PDCP.

Proposal S2.6-5-5: For the change from DAPS PDCP to the normal PDCP upon the source release, the reordering function is still maintained.

| Company | Is there any issue/proposal that need to be flagged for online discussion? (Y or N) | Comments (including which one/ones) |
| --- | --- | --- |
| Ericsson | Y | Proposal S2.3-5-1 (reason: resetting the RoHC context but maintaining the security key during handover may lead to keystream reuse if a DL/UL packet is retransmitted/duplicated in the target cell) |

### For issues identified for further discussion

PDCP part:

- Downlink ROHC IR packets

Disc S2.2-3-1: To be discussed whether to capture in the PDCP specification that “the target cell maintain the IR state in U-mode and O-Mode during DAPS handover”

Disc S2.2-3-2: Do not capture in the PDCP specification that “the source cell maintain the IR state in U-mode and O-Mode during DAPS handover”

MAC part:

- PHR reporting in another node

Option 1: reuse LTE and NR PHR MAC CE (NR: Multiple Entry PHR MAC CE in Figure 6.1.3.9-1; LTE: DC PHR MAC CE in Figure 6.1.3.6b-1;) 8 companies

Option 2: new MAC CE to support PHR reporting in another node;

Option 3: do not support PHR reporting in another node; 7 companies

Disc S2.3-7: To be discussed whether to support PHR reporting in another node;

As for the issues above Post109#11 Rapporteur has suggested to conclude them in the meeting, do companies think they can benefit from online discussion or offline discussion?

| Company | Downlink ROHC IR packets (online or offline) | PHR reporting in another node (online or offline) | Comments |
| --- | --- | --- | --- |
| Ericsson | Online | Offline | The PHR can benefit from further technical discussion which is best carried out offline. The RoHC IR issue has already been extensively discussed and I think we should try to resolve this issue during the online session. |

## Second phase: resolving open issues

# Conclusion

# References

- R2-2003371, Report of [Post109e#11][MOB] Resolving open issues for DAPS (Intel), Intel Corporation