3GPP TSG-RAN WG2 #109b-e	R2-200xxxx

Electronic, 20 Apr – 30 Apr 2020

Source: 	Huawei, HiSilicon

Title: 	Report of [AT109bis-e][205][MOB] Flagging and discussion of DAPS UP open issues for PDCP/RLC/MAC (Huawei)

Agenda Item:	7.3.2.1

Document for:	Discussion and decision

# Introduction

This is the report of the following offline discussion:

- [AT109bis-e][205][MOB] Flagging and discussion of DAPS UP open issues for PDCP/RLC/MAC (Huawei) Scope: - Companies flagging critical DAPS UP issues requiring Web conference discussion - Discuss the remaining UP open issues identified in email discussion report of Post109#11 in R2-2003371. Intended outcome: - Discussion summary document in R2-2003845, including resolutions to open issues and identification of non-critical issues that should no longer be pursued in Rel-16 Deadlines for flagging issues for Web conference discussion: - Flagging of issues for the Web conference: Tuesday 2020-04-21 10:00 UTC - Rapporteur summary: Tuesday 2020-04-21 11:30 UTC Deadlines for providing comments and for rapporteur inputs: - Initial deadline (for companies' feedback): Thursday 2020-04-23 10:00 UTC - Initial deadline (for rapporteur's summary in R2-2003845): Friday 2020-04-24 08:00 UTC - Proposed agreements in R2-2003845 indicated for email agreement and not challenged until Tuesday 2020-04-28 12:00 UTC will be declared as agreed by the session chair.

This offline discussion can be divided into two phases:

First phase: flagging phase.

Since we already have several proposals for DAPS UP issues in email discussion Post109#11 [1], firstly companies can flag any proposal that you think may benefit from further online discussion.

And for the issues that are identified for further discussion in Post109#11, companies can also pick which ones are suitable for online discussion.

Second phase: resolving open issues.

For open issues identified in Post109#11, we can try to make more progress here if no corresponding agreements are made online.

# Discussion

## First phase: flagging phase

### For issues with proposals in Post109#11:

##### MAC part:

- Stopping RACH in source:

Proposal S2.1-1: All the functions in Figure 4.2.2-1 will be supported by the source and target MAC entity in DAPS HO.

Proposal S3.9: Follow proposal S2.1-1, RACH is allowed to source after RACH towards target is successful.

2. 2-step RACH applied to DAPS:

Proposal S2.1-2: UE switches the UL PDCP data transmission upon successful RACH procedure (i.e. Msg.B for 2-step RACH).

3. Data transmission in MSG3:

Proposal S3.7-2: Forbid data transmission of non-DAPS DRBs in MSG3:

| Company | Is there any issue/proposal that need to be flagged for online discussion? (Y or N) | Comments (including which one/ones) |
| --- | --- | --- |
| Ericsson | N |  |
| OPPO | N |  |
| QC | Yes | Needs discussion about Proposal S3.9/S2.1-2 (reason: no need for UE to send any RACH to source cell. For example if UE fails source UL sync,RLC max transmission failure, beam failure etc, UE can just stop transmitting/receiving data from source cell) |

##### PDCP part:

- PDCP status report for DAPS UM DRB:

Proposal S2.2-1-1: The PDCP status report for DL UM DRBs is needed for DAPS HO.

Proposal S2.2-2-1: The second PDCP status report for DL UM DRBs is not needed for DAPS HO.

Proposal S3.2: PDCP status report for UM is mandatory for DAPS capable UE.

- ROHC handling in case of DAPS HO without key change:

Proposal S2.3-5-1: For DAPS DRBs, keep original agreements,i.e. separate RoHC context shall be applied for the source and target link even if DAPS handover is performed without key change

- PDCP SN number continuity for UM DRB:

Proposal S2.6-2: Keep original agreement that RLC UM (UL/DL) with PDCP SN number continuity is supported for DAPS.

- Discarding indication from PDCP to RLC:

Proposal S2.6-3: Do not introduce discard indication in source from PDCP to RLC upon UL switching.

- Discard timer:

Proposal S3.7-3: Discard timer is maintained during DAPS HO.

- PDCP status report to reflect holes from Source NB before DAPS HO:

Proposal S3.12: Do not introduce special handling on PDCP status report to support DAPS HO.

- LTE specific PDCP handling:

Proposal S2.6-5-1: Reordering_PDCP_RX_COUNT used for AM DRB reordering is needed for DAPS DRB.

Proposal S2.6-5-2: Last_Submitted_PDCP_RX_SN and Reordering_PDCP_RX_COUNT used for AM DRB reordering are needed for DAPS DRB.

Proposal S2.6-5-3: Reordering_PDCP_RX_COUNT is set to the COUNT value associated to RX_HFN and Next_PDCP_RX_SN upon PDCP reconfiguration for LTE UM DRB and LTE AM DRB without reordering from normal PDCP to DAPS PDCP.

Proposal S2.6-5-4: Last_Submitted_PDCP_RX_SN is set to [(Next_PDCP_RX_SN-1) modulo (Maximum_PDCP_SN+1)] upon PDCP reconfiguration for LTE UM DRB from normal PDCP to DAPS PDCP.

Proposal S2.6-5-5: For the change from DAPS PDCP to the normal PDCP upon the source release, the reordering function is still maintained.

- PDCP: Others

Proposal S2.3-5-2: For DAPS HO, capture PDCP handling for SRB in PDCP specification, the detailed text can be further discussed when capture it in PDCP specification.

Proposal S2.6-1: Leave the issue on uplink duplicated PDCP SDUs to RAN3.

Proposal S2.6-4: Leave the disucssion on PDCP anchor relocation in DAPS to RAN3.

Proposal S3.7-2: Forbid data transmission of non-DAPS DRBs in MSG3:

| Company | Is there any issue/proposal that need to be flagged for online discussion? (Y or N) | Comments (including which one/ones) |
| --- | --- | --- |
| Ericsson | Y | Proposal S2.3-5-1 (reason: resetting the RoHC context but maintaining the security key during handover may lead to keystream reuse if a DL/UL packet is retransmitted/duplicated in the target cell) |
| OPPO | Y | Proposal S2.2-1-1 (reason: PDCP status report is like providing feedback for UM DRBs and will introduce additional delay for DL transmission of UM DRBs, which is not good for real-time services) |
| QC | Yes | Proposal S2.3-5-1, same comment as Ericsson. More importantly as there can be large number of outstanding packets in UL window, which need to be retransmitted after the HO to target cell, this becomes burden on UE to re-generate the PDUs with “New ROHC Context + Old Ciphering Context” and retransmit especially when the outstanding packets to be retransmitted are too large. This might cause additional interruption. Proposal S3.12, in PDCP status report UE receiver sends FMS, NACKs for PDCP SDUs which failed compression and any missing SDUs. Target cell compresses all data packets from FMS with target cell ROHC and re-transmitted only NACKed SDUs. At UE receiver, after ROHC reset, UE does not get all PDCP SDUs in sequence from target cell and which may fail to decompress PDCP SDUs due to missing SNs. This can be fixed by enhancing PDCP Status Reporting. |

### For issues identified for further discussion

PDCP part:

- Downlink ROHC IR packets

Disc S2.2-3-1: To be discussed whether to capture in the PDCP specification that “the target cell maintain the IR state in U-mode and O-Mode during DAPS handover”

Disc S2.2-3-2: Do not capture in the PDCP specification that “the source cell maintain the IR state in U-mode and O-Mode during DAPS handover”

MAC part:

- PHR reporting in another node

Option 1: reuse LTE and NR PHR MAC CE (NR: Multiple Entry PHR MAC CE in Figure 6.1.3.9-1; LTE: DC PHR MAC CE in Figure 6.1.3.6b-1;) 8 companies

Option 2: new MAC CE to support PHR reporting in another node;

Option 3: do not support PHR reporting in another node; 7 companies

Disc S2.3-7: To be discussed whether to support PHR reporting in another node;

As for the issues above Post109#11 Rapporteur has suggested to conclude them in the meeting, do companies think they can benefit from online discussion or offline discussion?

| Company | Downlink ROHC IR packets (online or offline) | PHR reporting in another node (online or offline) | Comments |
| --- | --- | --- | --- |
| Ericsson | Online | Offline | The PHR can benefit from further technical discussion which is best carried out offline. The RoHC IR issue has already been extensively discussed and I think we should try to resolve this issue during the online session. |
| OPPO | Offline | Online | Whether enabling PHR reporting in another node or not is not critical for DAPS HO as the DAPS mode will not last for a long time. We should make decision in online session. For the RoHC IR issue, companies are already fully aware of the consequences. We should not waste online discussion time and simply follow majority views from offline discussion. |
| QC | Online | Offline |  |

## Second phase: resolving open issues

# Conclusion

# References

- R2-2003371, Report of [Post109e#11][MOB] Resolving open issues for DAPS (Intel), Intel Corporation