3GPP TSG-RAN WG2 Meeting#109bis-e	Draft_R2-2004046

Online, 20th - 30th Apr, 2020

Agenda Item:	7.2.3

Source:	Huawei

Title:	Draft_Report of offline discussion [AT109bis-e][311] on PUR open issues (Huawei)

Document for:	Discussion and Decision

# Introduction

The report of email discussion [Post109e#46][NBIOT/EMTC] [1] has been discussed and the following agreements on PUR were made:

RRC aspects: - For both NB-IoT and eMTC, the value range of pur-TimeAlignmentTimer-r16 is INTEGER (1..8), i.e. 1~8 * PUR periodicity. - All PUR parameters are stored in the eNB. RAN2 has not identified any parameters that must be stored in the MME. MAC aspects: - Remove the Editor’s Note “FFS whether restarting the window is indended” from 36.321. - Remove the Editor’s Note “FFS what is the impact of PUR in this section” from 36.321. RRC-MAC Interactions - No further MAC-RRC interaction on TA validation is needed. Remove the Editor’s Note “How RRC indicates to MAC that TA is valid or instructs MAC to use PUR” from 36.321. - Remove the references to PUR TA timer validation in section 5.4.7.1 from 36.321. - PUR release due to RACH initiation on a new cell is captured in RRC. - PUR configuration is released when the UE initiates RA procedure on a new cell for all purposes.

This document is for the following offline discussion on the remaining PUR open issues:

- [AT109bis-e][311][NBIOT] PUR open issues (Huawei)

Scope: Remaining open issues on PUR

Intended outcome: Finalise the open issues, report in R2-2004046

Deadline: 22-04-2020, 16:00 UTC

# Discussion

The following proposals in the report of email discussion [Post109e#46][NBIOT/EMTC] were not discussed/concluded in the online session:

- RRC aspects

Proposal 1-1:	For PUR TBS in eMTC, the current TBS values captured in eMTC RRC CR are supported, i.e. {b328, b408, b504, b600, b712, b808, b936, b1000, b1352, b1544, b1736, b1992, b2152, b2344, b2792, b2984}. (7/7)

Proposal 1-2:	[FFS] For PUR TBS in eMTC, TBS values larger than b2984 can be supported, FFS exact values and how many code points. (4/7)

Proposal 1-3:	For PUR TBS in NB-IoT, TBS values {b328, b408, b504, b584, b680, b808, b936, b1000, b1128, b1256, b1384, b1608, b1800, b2024, b2280, b2536} are supported. (5/7)

Proposal 1-4:	[FFS] For pur-Periodicity-r16 and requestedPeriodicity-r16, FFS whether to support hsf16384, hsf32768 and hsf65536 for both NB-IoT and eMTC (4/8).

Proposal 1-5:	[FFS] For both NB-IoT and eMTC, pur-StartTime-r16 is a 2-level start offset (5/8)

-	Level 1: startHSF: {hsf128, hsf256, hsf512, hsf1024, hsf2048, hsf4096, hsf8192, spare} (7/8)

-	Level 2: startSubframe: FFS value range (8/8)

Proposal 1-6:	[FFS] For both NB-IoT and eMTC, the granularity of requestedTimeOffset-r16 is H-SF level, FFS exact values. (4/8)

Proposal 1-8:	PUR-RNTI is used as the name of RNTI used for PUR. (6/8)

Proposal 2-2:	The eNB links CP-PUR configuration to each UE in RRC_IDLE according to PUR resource by implementation. (5/7)

Proposal 2-3:	[FFS] PUR (re-)configuration can be provided to the UE for the CP solution without AS security enabled (4/7).

- MAC aspects

Proposal 5: [FFS] No additional change on implicitReleaseAfter is needed in MAC specification. (4/7)

- RRC-MAC interactions

Proposal 6-0:	RAN2 to discuss whether to confirm or revert the working assumption that MAC calculates the PUR grant for each PUR occasion.

The following proposals 6-1 to 8 are conditional. If RAN2 confirms the working assumption:

Proposal 6-1:	RRC is aware of PUR grant. How RRC is aware is up to UE implementation. (5/7)

Proposal 6-2:	RRC can decide not to use the PUR grant for NAS signalling and no MAC-RRC interaction is needed. (6/7)

Proposal 6-3:	pur-NumOccasion is handed in MAC layer. (5/7)

Proposal 7:	MAC is aware of RRC state. How MAC is aware is up to UE implementation. (4/6)

Proposal 8:	MAC is aware of CP transmission using PUR. How MAC is aware is up to UE implementation. (7/7)

## RRC aspects

### requestedTBS-r16

requestedTBS-r16 in eMTC

The following proposals were made in [1] for the value range of requestedTBS-r16 in eMTC:

Proposal 1-1: For PUR TBS in eMTC, the current TBS values captured in eMTC RRC CR are supported, i.e. {b328, b408, b504, b600, b712, b808, b936, b1000, b1352, b1544, b1736, b1992, b2152, b2344, b2792, b2984}. (7/7)

Proposal 1-2: [FFS] For PUR TBS in eMTC, TBS values larger than b2984 can be supported, FFS exact values and how many code points. (4/7)

In the email discussion, 4 companies think that TBS larger than b2984 should be supported for eMTC.

Question 1. Do you agree to support TBS value larger than b2984 for eMTC? If yes, what is the maximum TBS?

| Company name | Yes, TBS? No, Why | Comments |
| --- | --- | --- |
| ZTE | Yes | Since PUR has the benefit of avoiding RA access, it should be used if the TB Size can be supported by the UE. Maximum value of b4008 or even b7224 for eMTC, maximum value of b2536 for NB-IoT are acceptable. |
| Huawei, HiSilicon | No | PUR should focus on small data transmission. For data size larger than b2984, we think it is better to transmit in RRC_CONNECTED. |

Question 2. Based on the proposed maximum value according to Question 1, how many code points should be supported for the requestedTBS-r16 in eMTC?

- Full set of TBS according to RAN1 PUSCH table

- 16 code points, 32 code points, etc.

| Company name | How many code points? | Comments |
| --- | --- | --- |
| ZTE | Full set of TBS according to RAN1 PUSCH table | For Less padding bits in PUR transmission, we prefer a finer TBS granularity. Several bits saved in requestedTBS-r16 may lead to tens or hundreds of padding bits in PUR transmission, that’s unexpected. |
| Huawei, HiSilicon | 16 | In any case, we do not see the need to support full set of TBS in PUR request. Thus the values captured in eMTC RRC CR are fine for us. |

requestedTBS-r16 in NB-IoT

The following proposal was made in [1] for the value range of requestedTBS-r16 in NB-IoT:

Proposal 1-3: For PUR TBS in NB-IoT, TBS values {b328, b408, b504, b584, b680, b808, b936, b1000, b1128, b1256, b1384, b1608, b1800, b2024, b2280, b2536} are supported. (5/7)

In the email discussion, 5 companies out of 7 are fine with the proposed value range for NB-IoT.

Question 3. Can you accept the proposed value range in proposal 1-3?

| Company name | Yes/No | Comments |
| --- | --- | --- |
| ZTE | No | With same reason of less padding bits in PUR transmission, we also prefer a finer TBS granularity for NB-IoT. |
| Huawei, HiSilicon | Yes | We do not see the need to support full set of TBS in PUR request message. |

### pur-Periodicity-r16 and requestedPeriodicity-r16

The following proposal was made in [1] for PUR periodicity larger than hsf8192:

Proposal 1-4:	[FFS] For pur-Periodicity-r16 and requestedPeriodicity-r16, FFS whether to support hsf16384, hsf32768 and hsf65536 for both NB-IoT and eMTC (4/8).

There is no majority support on additional larger values for pur-Periodicity-r16 and requestedPeriodicity-r16.

Question 4. Is it acceptable to remove the FFS for pur-Periodicity-r16 and requestedPeriodicity-r16 in both NB-IoT and eMTC? I.e. PUR periodicity is {hsf8, hsf16, hsf32, hsf64, hsf128, hsf256, hsf512, hsf1024, hsf2048, hsf4096, hsf8192, spareX}

| Company name | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes | We are fine with the current values but suggest more spare bits for future extension. |
| Huawei, HiSilicon | Yes | We think the current values from hsf8 to hsf8192 are enough. |

### Start offset

pur-StartTime-r16 in PUR configuration

For pur-StartTime-r16 in PUR configuration, all companies think that the UE needs to know exact start subframe of PUR grant. 5 companies out of 8 support to introduce a 2-level offset. Thus the following proposal was made in the email discussion for pur-StartTime-r16 in PUR configuration:

Proposal 1-5: [FFS] For both NB-IoT and eMTC, pur-StartTime-r16 is a 2-level start offset (5/8)

-	Level 1: startHSF: {hsf128, hsf256, hsf512, hsf1024, hsf2048, hsf4096, hsf8192, spare} (7/8)

-	Level 2: startSubframe: FFS value range (8/8)

Question 5. Is it acceptable to introduce a 2-level offset for pur-StartTime-r16 in PUR configuration, i.e. level-1 in H-SF level and level-2 in subframe level?

| Company name | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes |  |
| Huawei, HiSilicon | Yes |  |

If 2-level offset for pur-StartTime-r16 in PUR configuration is supported, in the email discussion, there are two options to signal level-1 offset for start H-SF:

Option 1: all possible start H-SF can be signalled, e.g. offsetHSF INTEGER (0..8191)

Option 2: only some H-SF can be signalled, e.g. startHSF: {hsf128, hsf256, hsf512, hsf1024, hsf2048, hsf4096, hsf8192, spare}

Question 6. If the answer to Question 5 is yes, which option do you prefer to signal the level-1 offset for the start H-SF?

| Company name | Option? | Comments |
| --- | --- | --- |
| ZTE | Option 1 | Option 1 is more flexible to ensure that network can fulfill the PUR scheduling requirements for UEs. |
| Huawei, HiSilicon | Option 2 | We needs to consider the signalling overhead. We do not see the need for subframe granulatity for the level-1 offset. We think it should be possible to signal value ‘0’ for the level-1 offset (so that we can configure PUR start offset smaller than PUR periodicity. |

In the email discussion, there are also two options to signal the level-2 subframe offset:

Option 1: all possible subframes can be signalled, e.g. offsetSubframe INTEGER (0..1023)

Option 2: similar way as DRX start offset in NB-IoT can be used, i.e. startSubframe: INTEGER(0..2559), value is in number of sub-frames by step of (PUR periodicity / 2560)

Question 7. If the answer to Question 5 is yes, which option do you prefer to signal the level-2 offset for the start subframe?

| Company name | Option? | Comments |
| --- | --- | --- |
| ZTE | Option 1 | As mentioned before, in option 2, only a few subframes within a start HSF offset can be indicated as the start subframe offset. For example, if hsf128 is selected for the pur-Periodicity, only about 4 sub frames (with step of 512 subframes) in the start HSF offset can be indicated as the start subframe offset. Moreover, if hsf8192 is selected for the pur-Periodicity, the step will be 32768 sub frames. We assume it means there exists only one start subframe offset about every three HSFs. It’s difficult to understand such configuration and this also give much restriction on the scheduling flexibility. Taken into account that many UEs may request the PUR resource simultaneously, such restriction would make it difficult for the eNB to allocate suitable PUR resources for the UEs. Therefore, option 1 is flexible enough and preferred. As PUR configuration is provided via dedicated signaling, the signaling overhead is not big issue. |
| Huawei, HiSilicon | Option 2 | We prefer to use similar way as for NB-IoT DRX offset so that we do not need to signal all possible subframes. Similarly with DRX start offset, maybe the step can be offsetHSF/2560 then the value range of the level-2 offset can be INTEGER(0..2559). |

requestedTimeOffset-r16 in PUR request

For requestedTimeOffset-r16 in PUR request, all companies think that requestedTimeOffset-r16 should have coarser granularity than pur-StartTime-r16. But not all companies made comments on the detailed granularity. 4 companies think H-SF level granularity should be enough.

The following proposal was made in [1] for requestedTimeOffset-r16 in PUR request:

Proposal 1-6:	[FFS] For both NB-IoT and eMTC, the granularity of requestedTimeOffset-r16 is H-SF level, FFS exact values. (4/8)

Question 8. Is it acceptable that requestedTimeOffset-r16 in PUR request has H-SF level granularity?

| Company name | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes | We are fine with coarse granularity for requestedTimeOffset-r16, e.g., H-SF level granularity. |
| Huawei, HiSilicon | Yes | We think H-SF level granularity is enough for the requested offset. |

Question 9. If the answer to Question 8 is yes, is it acceptable to use the same value range as in Question 6 for the level-1 offset in PUR configuration? If not, what are the proposed values?

| Company name | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes | Also prefer option 1: all possible start H-SFN can be signalled, e.g. offsetHSF INTEGER (0..8191) |
| Huawei, HiSilicon | Yes |  |

### RNTI used for PUR

In the email discussion, 6 companies think PUR-RNTI is more appropriate, 2 companies prefer to use PUR C-RNTI. Thus the following proposal was made:

Proposal 1-8:	PUR-RNTI is used as the name of RNTI used for PUR. (6/8).

Question 10. Can you accept to use PUR-RNTI as the name of RNTI used for PUR?

| Company name | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes |  |
| Huawei, HiSilicon | Yes |  |

### Configuration for the CP solution

We have agreed that all PUR parameters are stored in the eNB. Regarding how does the eNB link CP-PUR configuration to each UE in RRC_IDLE, the following options were proposed in [1]

- 5 companies out of 7 think that the eNB can determine the UE according to PUR resource by eNB implementation.

- 1 company thinks the eNB can link CP-PUR configuration to a UE by S-TMSI.

- 2 companies think the eNB can link CP-PUR configuration to a UE by PUR RNTI.

The following proposal was made according to the majority view:

Proposal 2-2: The eNB links CP-PUR configuration to each UE in RRC_IDLE according to PUR resource by implementation. (5/7)

Question 11. Can you accept proposal 2-2?

| Company name | Yes/No | Comments |
| --- | --- | --- |
| ZTE | No | This proposal doesn’t work in the following scenarios: - For UE in RRC CONNECTED requests PUR reconfiguration/release, the eNB cannot know which PUR configuration need to be reconfigured or released. - We have agreement: ‘m’ is not increased (neither by UE nor eNB) while UE is in RRC_CONNECTED. If a UE having PUR configuration enters RRC_CONNECTED, in its following PUR occasion, there may have no PUR transmission. The eNB needs not to increase “m” for this certain PUR grant as the related UE is in the RRC_CONNECTED. But as the eNB cannot link this PUR grant with the specific UE, it’s impossible for eNB to do this. As a result, the “m” would be incorrectly increased at eNB side. For PUR-RNTI, we assume it can be multiplexed by UEs (the time/frequency resources can be differentiated), so we think it can only be used to uniquely identify a UE in a certain period, e.g., during the PUR transmission. Moreover, we don’t assume PUR-RNTI would be maintained or associated with a UE in RRC_CONNECTED. |
| Huawei, HiSilicon | Yes |  |

For the security of PUR configuration for the CP solution, the following proposal was made in the email discussion:

Proposal 2-3:	[FFS] PUR (re-)configuration can be provided to the UE for the CP solution without AS security enabled (4/7).

Question 12. Can you accept that PUR (re-)configuration can be provided to the UE for the CP solution without AS security enabled?

| Company name | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes | In legacy CP solution, radio resource configuration in Msg4 is already allowed and no security issue. |
| Huawei, HiSilicon | Yes | This is the only option for the CP solution. RRC configuration without AS security is already supported in legacy NB-IoT. Considering that the CP solution is mandatory for NB-IoT, we this this should be allowed also for PUR. |

## MAC aspects

### How to capture implicitReleaseAfter

In the email discussion [1]:

- 4 companies think the current MAC specification works and no additional change is needed.

- 3 companies think agree to take TP provided by Qualcomm as a baseline to optimise the specification.

Thus the following proposal was made:

Proposal 5: [FFS] No additional change on implicitReleaseAfter is needed in MAC specification. (4/7)

Question 13. Is it acceptable that no additional change on implicitReleaseAfter is needed in MAC specification?

| Company name | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes |  |
| Huawei, HiSilicon | Yes |  |

## RRC-MAC interactions

RAN2#109e made a working assumption that PUR grant is calculated in MAC:

- RRC provides PUR configuration to MAC once and MAC calculates the PUR grant for each PUR occasion.

According to the working assumption, the following proposals were made in [1] for MAC-RRC interactions:

Proposal 6-1: RRC is aware of PUR grant. How RRC is aware is up to UE implementation. (5/7)

Proposal 6-2: RRC can decide not to use the PUR grant for NAS signalling and no MAC-RRC interaction is needed. (6/7)

Proposal 6-3: pur-NumOccasion is handed in MAC layer. (5/7)

Proposal 7: MAC is aware of RRC state. How MAC is aware is up to UE implementation. (4/6)

Proposal 8: MAC is aware of CP transmission using PUR. How MAC is aware is up to UE implementation. (7/7)

Question 14. If the working assumption is confirmed, i.e. MAC maintains the PUR grant, do you agree with above proposals?

| Company name | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes |  |
| Huawei, HiSilicon | Maybe | In the existing modelling for RRC-MAC interactions, it is always RRC that controls the usage of radio resources and configures the lower layer accordingly. If the PUR grant is maintained in MAC, we have to define neW type of MAC-RRC interactions or leave them to UE implementation as above proposals. However, in our view, the specification should describe the expected behaviour otherwise we will have testing and maintenance issues |

According to above proposals, if the PUR grant is maintained by MAC layer, most of RRC-MAC interactions are left to UE implementation. In the email discussion, there were still concern on this working assumption, including both RRC/MAC need to calculate the UL grant and MAC needs to be aware of RRC state. There was also proposal to revert the working assumption, i.e. move the calculation of PUR grant from MAC to RRC.

Question 15. If the working assumption is reverted, i.e. RRC maintains the PUR grant, what RRC-MAC interactions are needed on the handling of PUR grant/configuration?

| Company name | Comments |
| --- | --- |
| ZTE | As mentioned before, if PUR grant is not calculated in MAC, RRC needs to send PUR configuration to MAC for each time PUR transmission. Such RRC-MAC interaction would be per-PUR transmission. That would be frequent. If PUR grant can be maintained in MAC, according to the proposals in Q14, there will be very rare RRC-MAC interaction. We only identify the case that RRC reconfigure or release the PUR configuration. In this case, RRC may need to update PUR grant in MAC. But such case may be rare. |
| Huawei, HiSilicon | For PUR grant: - RRC maintains (calculates) the PUR grant, no need to configure/deconfigure the grant to MAC upon reception of RRC release message including PUR configuration. - RRC configures the lower layer to use PUR when initiating transmission using PUR, similarly to EDT procedure. - RRC maintains the m counter also. - The UE behaviour captured in MAC related to the above can be removed. For TA, we do not see the need to change the handling of TA in the current MAC/RRC specifications, i.e. TA timer is still configured to MAC upon RRC release and maintained in MAC, TA validation is still checked by RRC. |

Question 16. Based on the answers to Questions 14 and 15, do you prefer to confirm or revert the working assumption?

| Company name | confirm/revert | Comments |
| --- | --- | --- |
| ZTE | confirm |  |
| Huawei, HiSilicon | Revert | We would like to revert the WA because based on the WA: - We have to leave many interactions to UE implementation as indicated in Question 14 - Actually both RRC and MAC need to maintain the grant - MAC needs to be aware of RRC state |

## Other

There was one issue related to PUR submitted to ASN.1 discussion as RIL [Z603]:

Description:

In RAN2#107 meeting, RAN2 has agreed “The UE may use the D-PUR resource to send RRCConnectionRequest or RRCConnectionResumeRequest to establish or resume RRC connection.” However, the transmission of RRCConnectionRequest message using PUR to establish RRC connection hasn’t been captured in 36.331.

Proposed change:

1> the establishment or resumption request is for mobile originating calls and the establishment cause is mo-Data or mo-ExceptionData or delayTolerantAccess or mt-Access or mo-Signalling;

Considering that we have not agreed to use PUR for mt-Access or mo-Signalling, we think this should be an open issue instead of an ASN.1 issue. Thus, we propose to discuss it in this offline discussion.

Question 17. Whether transmission using PUR can be used for mt-Access or mo-Signalling?

| Company name | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes | This follows the previous agreement. |
| Huawei, HiSilicon | No | The reason we agreed to support RRC connection establishment/resumption using PUR resource is for the case that the UL data is larger than the configured TBS. Please refer to Discussion point 6C on other fallback cases in R2-1910173. |

# Summary

TBD

# References

- R2-2003746, “Report of email discussion [Post109e#46][NBIOT/EMTC] PUR open issues” Huawei