3GPP TSG-RAN WG2 Meeting#109bis-e	Draft_R2-2004052

Online, 20 – 30 April

Agenda Item:	7.2.4

Source:	Huawei

Title:	Draft_Report of [AT109bis-e][315][NBIOT] UE specific DRX - FFSs

Document for:	Discussion and Decision

# Introduction

UE specific DRX in NB-IoT has been discussed and the following agreements were made:

- UE-specific DRX cycle values 320ms, 640ms, 1280ms, 2560ms, 5120ms and 10240ms are supported in NB-IoT for both EPS and 5GS. - Introduce an indication in SIB to enable/disable the use of UE specific DRX cycles in NB-IoT for 5GS (similar to EPS). FFS whether it is cell or PLMN specific. - Will clarify UE behaviour in case of CSS overlap due to large repetitions needed to decode the NPDCCH for paging. FFS how. - Send a LS to CT1 and RAN3 to inform them about the UE specific DRX cycle values introduced for NB-IoT for both EPS and 5GS. - Send a LS to RAN4 to inform them about the UE specific DRX cycle values introduced for NB-IoT for both EPS and 5GS and ask to update RRM requirements, if needed.

This document is for the following offline discussion two address the 2 highlighted FFS on UE specific DRX:

- [AT109bis-e][315][NBIOT]  UE specific DRX - FFSs (Huawei)

Scope: Address the 2 FFS on UE specific DRX

Intended outcome: Report in R2-2004052

Deadline: 28-04-2020, 10:00 UTC

# Discussion

## SIB indication for 5GS

It has been agreed to introduce an indication in SIB to enable/disable the use of UE specific DRX cycles in NB-IoT for 5GS (similar to EPS). FFS whether it is cell or PLMN specific.

Question 1. Whether the SIB indication to enable/disable the use of UE specified DRX for 5GS should be cell specific indication or PLMN specific indication? Please provide justification.

| Company name | Cell specific/ PLMN specific | Justifications |
| --- | --- | --- |
| Huawei, HiSilicon | Cell specific | We prefer to have a simple cell specific indication so that the eNB can enable/disable the use of UE specific DRX according to the deployments and use case. We do not see the need to have per-PLMN indication. There were comments online that in case different operators share the same cell, they may have different paging requirement for different PLMN. However, we do not think this scenario is possible. There is only one set of paging configuration in one cell. Even in legacy, it is not possible to have different paging configuration for different PLMN. Thus, when the eNB makes decision to enable/disable UE specific DRX in one cell, it can only make decision according to cell specific DRX configuration, not per PLMN requirement. |
| Qualcomm | Cell specific | - The reason for this control is due to ng-eNB configuration e.g. deep coverage support makes it difficult to support short UE specific DRX therefore the control should apply to all PLMNs served by the cell. - PLMN level UE specific DRX control is possible at NAS level. From system perspective the DRX cycle used by a UE configured with UE specific DRX is less than or equal to configured UE specific DRX. But if RAN2 agrees to have a cell specific control for UE specific DRX then at system level it is no longer the case that delay to paging a UE would be bounded by the UE specific DRX. Therefore, SA2 must be information of this decision. |
| MediaTek | Cell specific | If the SIB indication is PLMN specific, then a cell would be able to enable and disable UE specific DRX in the same time for different PLMN. But the SIB indication is designed for resolving the conflict of UE specific DRX and deep coverage, so it should not be enable and disable in the same time. |
| Sequans | No strong view | The Tdoc R2-2003815 promoting this indication states “SIB indication is needed so that the NW/operator can enable/disable the use of UE specific DRX according to the deployment and targeted use cases”. Still it seems impossible to have an operator granularity control in case of shared NW, according to above comments. |
| Lenovo | Cell specific | It is simple and no extra issue introduced by per-PLMN paging mechanism. |
| Vodafone | Cell Specific | with this granularity we are able to manage the DRX cycles of the UEs according to the cell traffic and activity , furthermore the last location of the UE is easily located |
| ZTE | Cell specific | Cell level indication is enough. |
| Ericsson | Cell specific | Agree with the comments from HW above. |

Summary for Question 1

7 companies out of 8 think the SIB indication to enable/disable the use of UE specified DRX for 5GS should be cell specific indication.

- 1 company thinks SA2 should be informed.

1 company does not have strong view.

- The SIB indication to enable/disable the use of UE specified DRX for 5GS is cell specific indication. (8/8)

## Clarification on CSS overlapping

CSS overlapping was discussed in the email discussion [1] and the following agreement was made:

- Will clarify UE behaviour in case of CSS overlap due to large repetitions needed to decode the NPDCCH for paging. FFS how.

In general, we think search space overlapping is not a new issue introduced due to UE specific DRX. It may happen in legacy NB-IoT, thus we need to understand search space overlapping cases in legacy NB-IoT.

Search space overlapping case has been specified in TS36.213 as following:

- 16.6 Narrowband physical downlink control channel related procedures If a NB-IoT UE is configured with higher layer parameter twoHARQ-ProcessesConfig - the UE is not required to monitor an NPDCCH candidate of an NPDCCH search space if the candidate ends in subframe n, and if the UE is configured to monitor NPDCCH candidates of another NPDCCH search space having starting subframe k0 before subframe n+5 otherwise - the UE is not required to monitor NPDCCH candidates of an NPDCCH search space if an NPDCCH candidate of the NPDCCH search space ends in subframe n, and if the UE is configured to monitor NPDCCH candidates of another NPDCCH search space having starting subframe k0 before subframe n+5.

According to above RAN1 specification, if CSS overlapping happens, i.e. the distance between the last NPDCCH candidate of the CSS n and the first NPDCCH candidate of CSS n+1 is smaller than 5 subframes, the UE is not required to monitor all NPDCCH candidates of CSS n. However, since CSS is periodic, the UE will not monitor any of NPDCCH candidates for all CSS as two adjacent CSS will always be overlapped with each other.

Observation 1: according to the current RAN1 specification, if CSS overlapping happens, the UE is not required to monitor any of NPDCCH candidates for all CSS.

Question 2. Do you agree with above observation 1 on RAN1 specification?

| Company name | Yes/No | Justifications |
| --- | --- | --- |
| Huawei, HiSilicon | Yes |  |
| Qualcomm | Yes | This means if CSS overlap happens then effectively UE cannot receive any page. |
| MediaTek | Yes |  |
| Sequans | Yes (in the context of all CSS overlap) | We agree with the explanation. The observation 1 is correct in the context where all CSS overlap, as explained by the rapporteur (e.g. DRX 320ms with Rmax 512). It means that in such configuration, the UE can completely stop monitoring the paging and be compliant to specification (even if the UE is in good coverage) |
| Lenovo | Yes |  |
| Vodafone | Yes |  |
| ZTE | Yes |  |
| Ericsson | Yes |  |

Search space overlapping is not a new issue, some existing configurations may cause search space overlapping, e.g.:

For CSS-Type1: defaultPagingCycle: rf128 + npdcch-NumRepetitionPaging: 2048

Observation 2: In theory, search space overlapping is possible in legacy in certain configurations for both USS and CSS.

Question 3. Do you think above observation 2 that in theory search space overlapping may happen in legacy with certain configurations for CSS?

| Company name | Yes/No | Justifications |
| --- | --- | --- |
| Huawei, HiSilicon | Yes | Search space overlapping will happen at least in above examples. |
| Nokia | No with proper network configuration | In Rel-15 based system, when the above situation happens the UE may choose not monitor the NPDCCH at all thus result in UE not receiving paging for such configuration. If the UE can choose to receive only the limited number of NPDCCH candidates which does not overlap with next search space, this problem can be solved. But this will limit the UE coverage to lower value than the maximum defined for the cell. Because this UE cannot receive page with maximum repetitions defined for CSS search space. This issue was discussed earlier within RAN2 in the context of fractional NB, and it is agreed that the network need to ensure the cell specific DRX cycle to avoid such overlap. With lower DRX cycle lengths which is UE specific, the problem cannot be solved for the UE configured with UE specific DRX cycle, unless until the UE behaviour is clarified for this situation. Possible options : - O1: UE and Network automatically fallback to cell specific DRX cycle when the UE is paged in cell where there is CSS overlap for the UE specific DRX configuration. (Clarification to TS36.304) - In this option : UE loses the benefit of short DRX cycle in the cells which is configured for coverage extension which results in CSS overlap. - O2:UE and network may choose to DRX cycle length to next higher value which will not result in the overlap. - O3: Above section should in TS36.213 needs to be modified to clarify the different UE behaviour for CSS overlap for paging. In our view view O1 and O2 were earlier discussed for similar issue within RAN2 based on Inputs from Docomo. As the UE behaviour changes were not prefered those options are not considered. UE specific DRX cycle is new feature in Rel-16 with specification impact, we don’t see issue in capturing the additional clarification for such specific UE in Rel-16. |
| Qualcomm | Yes | In legacy it is not an issue because this would be considered a bad cell configuration and all UEs would be impacted hence problem will be easily identified and fixed. |
| MediaTek | Yes |  |
| Sequans | Yes but | There are perfectly valid and useful configurations where such overlap would occur. So we would not say “in theory”. In practice too. Common example is when G=1.5 is used, at SFN rollover (e.g. for USS, RA CSS) We do not see the point in this email discussion. |
| Lenovo | Yes | Search space overlapping will happen at least in above cases. We think eNB should try to avoid search space overlapping for NB-IOT scenario, otherwise it is UE implementation although UE coverage level is reduced somehow. No extra clarification is needed. |
| Vodafone | Yes | Overlapping would be a common occurance |
| ZTE | Yes | In theory, search space overlapping may happen in legacy with certain configurations. But it’s feasible for eNB to avoid it by implementation. |
| Ericsson | Yes |  |

Question 4. If the answer to Question 3 is yes, do you think above configurations which may lead to search space overlapping in legacy are valid?

| Company name | Yes/No | Justifications |
| --- | --- | --- |
| Huawei, HiSilicon | No | According to our reply to Q2, if search space overlapping happens, the UE will not to monitor any of NPDCCH candidates for that search space. Thus we think above configurations are invalid configuration and should be avoided by the eNB. |
| Nokia | No | For cell specific DRX case, the above configuration is invalid and can be avoided by proper network configuration. As per the above behaviour defined for CSS search space, the UE configured with UE specific DRX cycle will not be able to receive paging in cells where the CSS is configured for duration beyond the UE specifc DRX cycles. This impact is not acceptable. |
| Qualcomm | No | It’s a bad eNB configuration that impacts all UEs and easily fixed with correct network configuration. The legacy mechanisms to ensure CSS for paging do not overlap cannot work where UEs in a cell use a varied range of DRX cycles. |
| MediaTek | No | eNB should avoid to use CSS overlapped configuration. |
| Sequans | Yes | Again, there are perfectly valid configurations with CSS overlap, and again not the point of this email discussion. |
| Lenovo | No | eNB could avoid this kind of configuration. |
| Vodafone | No | Agree with comments above the eNodeB should be designed to avoid this scenario |
| Ericsson |  | The configuration is valid, however as specified in 36.213, the UE would not monitor NPDCCH candidates of an NPDCCH search space if there is CSS overlap and thus it should be avoided by the network, i.e., bad configuration. |

Summary for Questions 2-4:

Questions 2-4 are all related to the understanding on the CSS overlapping issue in legacy NB-IoT.

The following observations on the CSS overlapping issue in legacy NB-IoT are summarised to help us to make decision on CSS overlapping issue in UE specific DRX for Rel-16 NB-IoT. Thus no proposal will be made for those questions.

For Question 2, all companies agreed that according to the current RAN1 specification, if CSS overlapping happens, the UE is not required to monitor any of NPDCCH candidates for all CSS.

- According to the current RAN1 specification, if CSS overlapping happens, the UE is not required to monitor any of NPDCCH candidates for all CSS. (8/8)

For Question 3, 7 companies out of 8 agree that in theory search space overlapping is possible in legacy in certain configurations for CSS.

- One company replies NO. According to the reply, it is commented that it is agreed that the network need to ensure the cell specific DRX cycle to avoid such overlap, i.e. CSS overlapping needs to be avoided.

- One company thinks CSS overlapping exists in practice in legacy NB-IoT.

- In theory, CSS overlapping is possible in legacy in certain configurations. (7/8)

For Question 4, 7 companies out of 8 think that configuration for paging that will lead to CSS overlapping (i.e. defaultPagingCycle: rf128 + npdcch-NumRepetitionPaging: 2048) should be avoided (one company thinks the configuration is valid but should be avoided). One company thinks configuration with CSS overlapping is perfectly valid in legacy NB-IoT.

- In legacy NB-IoT, configuration that will lead to CSS overlapping should be avoided by the network. (7/8)

Question 5. According to Questions 2, 3 and 4, do you think any clarification on UE behaviour when CSS overlapping happens is needed? If yes, how to clarify?

| Company name | Yes/No, how? | Justifications |
| --- | --- | --- |
| Huawei, HiSilicon | No | We think the current TS36.213 is very clear regarding the UE behaviour when search space overlapping happens, i.e. the UE is not required to monitor any of NPDCCH candidates of the search space. No further clarification is needed. |
| Nokia | Yes | As indicated in Q3 any of the proposed options can be considered in our view. We can discuss on other options also based on company inputs. But we don’t agree that this feature (support for UE specific DRX cycles lower than cell specific values) supported without additional clarification. |
| Qualcomm | Yes | Because this is a bad configuration and the UE behaviour is clear. With such bad configurations UEs would not be able to receive any page. But with UE specific DRX the problem is created for certain UEs even though cell configuration works fine for cell specific DRX. |
| MediaTek | No | The CSS overlapped issue has already been clarified without any ambiguity.The clarification can also apply to UE specific DRX. No need further clarification. |
| Sequans | Yes | As stated by HW: “the current TS36.213 is very clear regarding the UE behaviour when search space overlapping happens, i.e. the UE is not required to monitor any of NPDCCH candidates of the search space. ” In new scenarios introduced by short DRX cycles (e.g. T=320ms and Rmax=512), this allows the UE to stop monitoring paging – even when the UE is in good coverage! That would be the natural UE decision since one could deduce from the specification that the NW would not even try to page him…So why bother listening to paging, at least this enables power consumption gain. We agreed online to clarify UE behaviour in that case, the email discussion should focus on that. In our view, the interface with RAN1 is the PF/PO location, indicating the CSS start. In order to keep the possibility for UE/NW to keep accumulating enough repetitions, we could indicate which POs shall be “skipped” in case of overlap. We had a proposal in last meeting “R2-2001630” “For NB-IoT UEs for which UE specific DRX cycle is applicable, in case T is lower than T/nB and the UE requires more repetitions than available during T (overlapping paging CSS), the UE may skip paging occasions and keep PF such that SFN mod Text = 0, where Text is the lowest DRX cycle value for which enough repetitions are available, among possible DRX cycle values. ” |
| Lenovo | No | The same view as Huawei, no further clarification is needed. |
| Vodafone | Yes | the specification needs to tightened given the realistic scnearios that could occur in practice |
| ZTE | Yes | Different from legacy, in R16 after introduction of NB-IoT UE specific DRX, when the eNB configure the npdcch-NumRepetitionPaging, eNB cannot know the DRX cycle used by the UE (eNB can only know this when receives the paging). Then it may be difficult for eNB to set a suitable npdcch-NumRepetitionPaging which can match all the UE’s used DRX cycle. Therefore, we think some simple constraint between parameters may be useful. For example, if NB-IoT UE specific DRX is enabled (eNB may assume the network coverage is good), the npdcch-NumRepetitionPaging should not be set to a value larger than r256 (this is the maximum repetition number which can match the 320ms UE specific DRX cycle). |
| Ericsson | Yes | This depends on the intention. If RAN2 agrees that the UE behaviour already specified in 36.213 is also acceptable for this case, i.e., UE-specific DRX, there is no need for further clarification, otherwise we think a clarification is needed so that it would be possible for the network to page such UEs even when there may be a CSS overlap due to many repetitions when paging the UE. |

Summary for Questions 5:

3 companies think that the current specification is clear enough thus not further clarification is needed.

6 companies think further clarification is needed:

- 2 companies did not provide proposal on how to clarify.

- The following options are mentioned by companies:

  - Clarification on the realistic scenarios for UE specific DRX is needed, e.g. UE specific DRX cannot be enabled with certain NPDCCH repetition number for paging CSS (preferred by 2 companies).

  - UE and Network automatically fallback to cell specific DRX cycle when the UE is paged in cell where there is CSS overlap for the UE specific DRX configuration. (Clarification to TS36.304) (preferred by 1 company)

  - UE and network may choose to DRX cycle length to next higher value which will not result in the overlap (preferred by 1 company)

  - PF skipping mechanism as proposed in R2-2001630. (preferred by 1 company)

Regarding how to clarify UE behaviour, companies’ views are split, there is no consensus which way to go.

- No consensus on how to address CSS overlapping for UE specific DRX.

# Conclusion

This offline discussion focused on the 2 FFS of UE specific DRX cycle in NB-IoT.

- According to the current RAN1 specification, if CSS overlapping happens, the UE is not required to monitor any of NPDCCH candidates for all CSS. (8/8)

- In theory, CSS overlapping is possible in legacy in certain configurations. (7/8)

- In legacy NB-IoT, configuration that will lead to CSS overlapping should be avoided by the network. (7/8)

- The SIB indication to enable/disable the use of UE specified DRX for 5GS is cell specific indication. (7/7)

- No consensus on how to address CSS overlapping for UE specific DRX.

# References

- R2-2003669	Report of [Post109e#15][NBIOT] UE specific DRX DRX cycle values Sequans