3GPP TSG-RAN WG2 Meeting #109bis electronic	                                                                           R2-200xxxx

20th April – 30th April 2020

Agenda Item:	7.1.9

Source: 	ZTE (email discussion rapporteur)

Title:  	Draft report of [AT109bis-e][414][eMTC/NB-IoT]Coexistence with NR-Open issues (ZTE)

Document for:	Discussion and Decision

# Scope of the offline email discussion

This document contains the summary of the offline email discussion “[AT109bis-e][414][eMTC] Coexistence with NR - Open issues”, as indicated below:

- [AT109bis-e][414][eMTC/NB-IoT] Coexistence with NR - Open issues (ZTE)

Scope: Remaining open issues on coexistence with NR.

Intended outcome: Report including a list of proposals categorized as agreeable, need further discussion etc.. The outcome can be provided in R2-2003929.

Deadline: Friday, Apr. 24th 10:00 UTC

# Offline email discussion

## 2.1 Summary for NB-IoT

#1-1: Whether to specify broadcast signaling

In last meeting, RAN2 has agreed to specify dedicated signaling for providing carrier-specific resource reservation configuration for NB-IoT coexistence with NR. During the meeting discussion, there had suggestion to also specify broadcast signaling to provide resource reservation configuration.

In this meeting, based on some observations on NB-IoT and NR coexistence scenarios and the intention respectively for DL and UL resource reservation, the following proposal has been submitted:

| [1] | ZTE | Proposal 1: It’s suggest RAN2 to decide not to specify SIB for providing NB-IoT and NR coexistence parameters. |
| --- | --- | --- |

Q1: Companies are invited to provide comments on whether to specify broadcast signaling for providing carrier-specific resource reservation configuration for NB-IoT.

| Company | Yes, to specify SIB/No, not to specify SIB | Detailed comments |
| --- | --- | --- |

Conclusion: TBD

Proposal: TBD

Q2: If the answer for Q1 is yes, companies are invited to provide comments to the table below.

| Company | Detailed comments on how to specify SIB and what’s the relationship between the configuration in SIB and dedicated signaling |
| --- | --- |

Conclusion: TBD

Proposal: TBD

#1-2: New UE capabilities

As mentioned in [1], one RAN1 requirement is to have separate indications for subframe level and slot/symbol-level resource reservation capabilities for NB-IoT. For slot-level and symbol-level resource reservation, only one capability is enough and which can be used to indicate supporting both slot-level and symbol-level resource reservation together. The related proposal is as following:

| [1] | ZTE | Proposal 2: It’s suggest RAN2 to introduce two NB-IoT UE capabilities for separately indicating subframe level and slot/symbol-level resource reservation capabilities. |
| --- | --- | --- |

Q3: Companies are invited to provide comments to the table below, on whether to introduce two NB-IoT UE capabilities for separately indicating subframe level and slot/symbol-level resource reservation capabilities.

| Company | Yes/No | Detailed comments |
| --- | --- | --- |

Conclusion: TBD

Proposal: TBD

#1-3: Signalling optimization

In last meeting, there has some initial discussion on signaling optimization. Based on the current NB-IoT 36.331 CR, companies can further suggest optimization on the dedicated signaling, or the possible SIB signaling.

Q4: Companies are invited to provide comments to the table below, on whether to have further signaling optimization on the current dedicated signaling, or the possible SIB signaling, and how.

| Company | Yes/No | Detailed suggestions |
| --- | --- | --- |

Conclusion: TBD

Proposal: TBD

During last meeting discussion for LS out R2-2001888, company has mentioned the signaling optimization for mobility case. E.g., for NB-IoT, when resumption in a neighboring cell, it may be possible the previous configuration parameters can be reused, avoiding to signal them again?

Q5: Companies are invited to provide comments to the table below, on whether to have signaling optimization for mobility case in NB-IoT and how.

| Company | Yes/No | Detailed suggestions |
| --- | --- | --- |

Conclusion: TBD

Proposal: TBD

#1-4: Other issues:

Q6: If there has any other issues for NB-IoT, companies are invited to provide comments to the table below.

| Company | Issues or comments |
| --- | --- |

Conclusion: TBD

Proposal: TBD

## 2.2 Summary for eMTC

#2-1: Whether to specify broadcast signaling

In last meeting, RAN2 has agreed to specify dedicated signaling for providing resource reservation configuration and DL subcarrier puncturing configuration for eMTC coexistence with NR. During the meeting discussion, there had suggestion to also specify broadcast signaling to provide these configurations.

In this meeting, based on some observations on the intention respectively for DL and UL resource reservation, the following proposal has been submitted in [2]:

| [2] | ZTE | Proposal 1: It’s suggest RAN2 not to specify system information for providing resource reservation parameters for eMTC and NR coexistence. |
| --- | --- | --- |

Moreover, based on the understanding that DL subcarrier puncturing configuration can be per eMTC narrowband, the following proposal has been submitted:

| [2] | ZTE | Proposal 2: It’s feasible to provide DL subcarrier puncturing configuration via SIB message. |
| --- | --- | --- |

Q7: Companies are invited to provide comments on whether to specify broadcast signaling for providing resource reservation configuration for eMTC.

| Company | Yes, to specify SIB/No, not to specify SIB | Detailed comments |
| --- | --- | --- |

Conclusion: TBD

Proposal: TBD

Q8: Companies are invited to provide comments on whether to specify broadcast signaling for providing DL subcarrier puncturing configuration for eMTC.

| Company | Yes, to specify SIB/No, not to specify SIB | Detailed comments |
| --- | --- | --- |

Conclusion: TBD

Proposal: TBD

Q9: If the answer for Q7 and/or Q8 are yes, companies are invited to provide comments to the table below.

| Company | Detailed comments on how to specify SIB and what’s the relationship between the configuration in SIB and dedicated signaling |
| --- | --- |

Conclusion: TBD

Proposal: TBD

#2-2: New UE capabilities

As analyzed in [2], only one capability to indicate supporting both slot-level and symbol-level resource reservation may be needed for eMTC. The related proposal is as following:

| [2] | ZTE | Proposal 3: It’s suggest RAN2 to introduce an eMTC UE capability for indicating slot/symbol-level resource reservation capability |
| --- | --- | --- |

Q10: Companies are invited to provide comments to the table below, on whether to introduce eMTC UE capability(s) for indicating subframe level, slot/symbol-level resource reservation capabilities.

| Company | Yes/No | Detailed comments |
| --- | --- | --- |

Conclusion: TBD

Proposal: TBD

#2-3: Signalling optimization

In last meeting, there has some initial discussion on signaling optimization. Based on the current eMTC 36.331 CR, companies can further suggest optimization on the dedicated signaling, or the possible SIB signaling.

Q11: Companies are invited to provide comments to the table below, on whether to have further signaling optimization on the current dedicated signaling, or the possible SIB signaling, and how.

| Company | Yes/No | Detailed suggestions |
| --- | --- | --- |

Conclusion: TBD

Proposal: TBD

During last meeting discussion for LS out R2-2001888, company has mentioned the signaling optimization for mobility case. E.g., for eMTC:

- when handover, it may be possible parameters are the same in the new cell, there is no need to provide them again.

- when resumption in a neighboring cell, it may be possible the previous configuration parameters can be reused, avoiding to signal them again

Q12: Companies are invited to provide comments to the table below, on whether to have signaling optimization for mobility case in eMTC and how.

| Company | Yes/No | Detailed suggestions |
| --- | --- | --- |

Conclusion: TBD

Proposal: TBD

#2-4: Other issues:

Q13: If there has any other issues for eMTC, companies are invited to provide comments to the table below.

| Company | Issues or comments |
| --- | --- |

Conclusion: TBD

Proposal: TBD

# Summary

[TBD]

# References

- R2-2003477, Further discussion on NB-IoT coexistence with NR	ZTE Corporation, Sanechips	discussion	Rel-16	NB_IOTenh3-Core

- R2-2003478, Further discussion on eMTC coexistence with NR	ZTE Corporation, Sanechips	discussion	Rel-16	LTE_eMTC5-Core

- R2-2001888, LS to RAN1 on co-existence with NR, Qualcomm, RAN2#109 e-meeting