Agenda Item:	7.1.13

Source: 	Qualcomm Incorporated

Title:    	[AT109bis-e][417][eMTC] Interworking between Cat M and NR

Document for:		Discussion and decision

# Introduction

The purpose of this offline is to discus and conclude on the reply to LS from SA2 in [1] and, if necessary, changes to RAN2 specification.

# Discussion

SA2 has asked following question in their LS in [1]:

“SA2 kindly asks RAN2 whether a category M1/M2 device accessing WB-E-UTRA does not indicate NR support, and whether a device accessing NR does not indicate category M1/M2”

The background to above question is when 5GC is connected to both ng-eNB and gNB then can a Cat M1/M2 UE move between ng-eNB and gNB. Currently in EPC a Cat M1/M2 device is differentiated from normal LTE devices by an indicator generated by eNB. Similarly, an LTE-M indicator is added to Release 16 RRC when Cat M1/M2 device is connected to 5GC.

Note: An LTE UE supporting CE mode is not covered by the LTE-M indicator generated by the eNB nor by the LTE-M indicator sent by the UE in Release 16 RRC.

As pointed out in [2], a Cat M1/M2 is a bandwidth reduced low complexity device hence if it supports NR then this description no longer fits.

Question 1. 	Cat M1/M2 device Support or Not Support interworking with NR? Provide justification for your answer.

| Company | Support/ Not Support | Comment |
| --- | --- | --- |
| Qualcomm | Not Support | Hardware capability required for NR is significantly higher than that required for Cat M UE hence such a UE would support normal LTE functionality anyway and no need to pretend it is Cat M UE. Note, a LTE UE can support and operate in CE mode. |
| Huawei, HiSilicon | Not Support | No strong reason to support from UE perspective due to the reasons given by QC, and given it will require additional (possibly significant) work in SA2 if supported then we are fine with QC proposals.. |

As pointed out in [2], a Cat M1/M2 is a bandwidth reduced low complexity device hence if it supports NR then this description no longer fits.

Question 2. 	If your answer to Question 1 is Not Support, then do you support the reply in [3]? If no, provide alternative wording.

| Company | Yes/No | Comment |
| --- | --- | --- |
| Qualcomm | Yes |  |
| Huawei, HiSilicon | Yes | Seems fine. |

If Cat M device does not support interworking with NR then this needs to be captured in the RAN2 specifications in some way. One approach is to state this in stage 2 as proposed in [4]. Another way is to capture this via UE capability. In [5] it is proposed to add the following capabilities:

Proposal 9-6: For eMTC, introduce the following capabilities for support of connection to 5GC:

- ce-eutra-5GC

- ce-eutra-5GC-HO-ToNR-FDD-FR1

- ce-eutra-5GC-HO-ToNR-TDD-FR1

- ce-eutra-5GC-HO-ToNR-FDD-FR2

- ce-eutra-5GC-HO-ToNR-TDD-FR2

Provided the description of these fields states something like “This field is not applicable for UEs of Category M1 and Category M2” then stage 2 clarification may not be necessary

Question 3. 	If Cat M device should not support interworking with NR then should RAN2 specification state this.

| Company | Yes/No | Comment |
| --- | --- | --- |
| Qualcomm | Yes | Specification needs to be clear to avoid confusion to the reader. |
| Huawei, Hisilicon | Yes | Normally we capture applicability of capabilities, and any exceptions. |

Question 4. If you answer to question 3 is Yes then which specification should capture this restriction?

| Company | Stage 2/ Stage 3/ Both | Comment |
| --- | --- | --- |
| Qualcomm | Both | Stage 2 makes it clear that interworking is not supported in both directions. Stage 3 changes are useful for the reader to know from E-UTRA point of view which capability is not applicable to Cat M. |
| Huawei, HiSilicon | Stage 3 | It seems enough to capture the applicability in 36.306, this is what we normally do. |

- Summary and Conclusion

TBD

- References

- R2-2000056, LS on category M devices and NR, RAN2#109_e, February 2020.

- R2-2003185, Interworking between Cat M and NR, RAN2#109bis_e, April 2020.

- R2-2003186, Reply LS on category M devices and NR, RAN2#109bis_e, April 2020.

- R2-2003187, Clarification interworking with NR not supported in eMTC, RAN2#109bis_e, April 2020.

- R2-2003248, UE capabilities, TDD/FDD differentiation and 5GC applicability for NB-IoT and eMTC, RAN2#109bis_e, April 2020.