3GPP TSG-RAN WG2 Meeting #109bis electronic	R2-200xxxx

Online, 20th April – 24th April, 2020

Source:	CATT

Title:	[AT109bis-e][505][PowSav] RRM Open Issues

Agenda Item:	6.11.6

Document for:	Discussion and Decision

# Introduction

This document provides a summary of the Power Saving contributions posted at RAN2#109bis e-meeting on agenda item 6.11.6: RRM measurement relaxation [1]-[5].

An email discussion addressing identified RRM open issues so far was held before this e-meeting resulting in a three non-controversial proposals as well as one unresolved issue [7].

The contributions/proposals posted in this agenda item can be classified as addressing the following categories of issues:

1. Issues/proposals not discussed as part of the email discussion [2]p2-5, [5]

2. Issue that could not be solved during the email discussion [1], [2]p1, [3]p1-2, [6]p5

3. Issues that were solved during the email discussion [3]p3, [4], [6]p6

For category 3 issues, given the related proposals in [7] are supported by a vast majority of companies, we do not address them in this summary and only focus on contributions/proposals for above categories 1-2 issues.

# Discussion

## Frequency-specific relaxation triggers

This issue was discussed in [7] without conclusion:

The following issue could not be resolved and should be continued on-line during next e-meeting:

The configuration of the relaxation criteria is constant for all frequencies (6) or is per-frequency (or per-FR) configured (7).

Four contributions addressed this issue further:

| [1] - Sony | Proposal: Network configures additional criteria for measuring a particular frequency. These criteria could include the detection of a particular cell or frequency (higher priority) or a timer (e.g. if UE does not find this frequency whilst the timer is running then it skips measuring this frequency) |
| --- | --- |
| [2] - MediaTek Inc. | Proposal 1: Only a single value is configured for each RRM measurement relaxation threshold, and it applies to the measurements on all neighbor frequencies. |
| [3] - vivo | Proposal 1: RRM configuration should support to configure UE to relax the measurement only on some of frequencies/FR. FFS on per frequency or per FR. Proposal 2: Network indicates whether the UE is allowed to relax RRM measurements on each frequency/FR explicitly via “per frequency/FR indication” or implicitly via per-frequency/FR configuration for measurement relaxation. |
| [6] - Ericsson | Proposal 7: The same RRM relaxation is applied to all inter-frequencies the UE needs to measure (except Thigherprioritysearch measurements shall not be relaxed). |

[1] suggests addressing the frequency-specific RRM measurement relaxation by introducing a new criterion on top of those agreed so far, where “frequencies to be measured are made conditional on the detected cell/frequency or their existence in the neighbourhood”. This is a new approach that has not been discussed so far. As a result, given the late stage of the WI, we would suggest not discussing new solutions for issues for which other simple solutions have already been discussed and are close to be converged.

[2][3][6] stick to the strict scope of the email discussion question i.e. whether the relaxation criteria agreed so far should be configured as global parameters commonly applied to all frequencies, or should it be per-frequency configured (or enabled/disabled). Arguments can be summarized as a complexity vs flexibility trade-off, as follows:

| Single value | Per frequency/FR indication/configuration |
| --- | --- |
| [2] - MediaTek Inc.: It was mentioned in [1] that we may introduce carrier-specific search thresholds for measurement relaxation on inter-frequency carriers. For example, the network may configure UE to relax measurements on carrier#1 (or FR1) if serving cell RSRP drop is less than SSearchDeltaP1, and on carrier#2 (or FR2) if serving cell RSRP drop is less than SSearchDeltaP2. However, our understanding is that these thresholds consider serving cell channel conditions, and UE evaluates whether neighbor cell measurements can be relaxed. If UE is not with low mobility or at cell edge, all neighbors should be measured regardless of the carrier frequencies. Also, in the agreed RRC CR [2], we have only one s-SearchDeltaP (for low mobility criteria), one s-SearchThresholdP and one s-SearchThresholdQ (for not-at-cell-edge criteria), as shown below. This part has to be modified if we want to introduce carrier-specific configurations, but we would prefer to avoid such complexity. [6] Ericsson: In case there is a need to relax FR2 measurement more compared to FR1 measurements, we think that priorities can be used, i.e. FR2 is assigned a higher priority compared to FR1. | [3] – vivo: In the scenario of Figure1, Frequency 1 is the current camping frequency and measurement relaxation on Frequency 2 is not allowed. Hence, the network configures Frequency 2 with higher priority than Frequency 1 and unset the highPriorityMeasRelax indication. As a result, measurement relaxation on Frequency 2 is disabled. We agree that option 2 is enough in some cases, e.g. the case in Figure1. However, option 2 couples the frequency priority and per frequency/FR measurement relaxation, it reduces the flexibility of network configuration. For example, the option 2 cannot disable measurement relaxation on a low priority frequency. Besides, this highPriorityMeasRelax indication is being discussed in RAN4. There may be some possibility that there is no need to introduce such indication finally. Compared with option2, option1a and 1b are straight forward and most clean approaches. |

Rapporteur’s summary:

A global configuration is the current state of the specifications, and going to per-frequency/FR indication/configuration would increase flexibility but also complexity.

In the email discussion [7] most companies supporting a finer configuration granularity than one global configuration were also OK to limit it to per frequency range (FR1/FR2), thus also liming the complexity. And two scenarios were brought-up in contributions [3][6], as examples differentiating RRM measurement relaxation for FR1/FR2 cells:

Scenario 1: low frequency is used for coverage purpose. RRM measurement relaxation may be only applied for high frequency cells [3]

Scenario 2: load balancing typically favoring UE to reselect spotty coverage layer (FR2) vs large cells (FR1): RRM measurement relaxation may be only applied for low frequency cells [6].

Some possible solution proposed in [6] (as well as other companies in [7]) consists in using the existing “higher priority” framework to differentiate RRM measurement relaxation of some frequencies, for example FR2 frequencies. But for [3], such approach depends on how RAN4 will address RRM measurement relaxation of higher priority frequencies, and the final impact it will have on the RAN2-defined highPriorityMeasRelax indication allowing disabling relaxation on higher priority frequencies. However, from RAN4 WF on RRM measurement relaxation for inter-frequency layer with higher priority ([8] slide 4), whatever option RAN4 decide in the end, high priority frequencies can only have less or same relaxation than equal/lower-priority cells, hence it is unclear whether it impacts the above coupling, i.e. both aim at differentiating RRM measurements of two classes of frequencies.

From the above, a possible outcome could be to 1) focus on FR1/FR2 differentiation only and 2) understand better the motivation scenarios and why they cannot be addressed by existing high-priority frequency framework.

Proposal 1: Continue the discussion in an offline focusing on:

1) FR1/FR2 differentiation only and

2) understanding better the motivation scenarios and why they cannot be addressed by existing high-priority frequency framework.

## RAN4-related issues

The following contributions/proposals discuss RRM relaxation methods (not triggers), which is in RAN4 scope [8]:

| [2] - MediaTek Inc. | Proposal 2: The same scaling factor is used for RRM measurement relaxation with longer intervals in both low-mobility scenario and not-at-cell-edge scenario. Proposal 3: The same scaling factor is used for RRM measurement relaxation with longer intervals on all neighbor frequencies. Proposal 4: The scaling factor RRM measurement relaxation with longer intervals is a predefined fixed value. The feasibility needs to be confirmed by RAN4. Proposal 5: For UE in cell center but not satisfying low-mobility criteria, the measurements on high priority frequencies are not further relaxed even when highPriorityMeasRelax is set. |
| --- | --- |
| [5] - LG Electronics Inc. | Proposal 1: UE shall not perform relaxed measurement on a given frequency when the UE is configured to perform early measurement on that frequency and T331 is running. Proposal 2: After T331 expiry, the UE may perform relaxed measurement on the frequency if the UE continues to perform early measurements based on implementation. |

The above issues were not included in the email discussion [7] because they are already discussed in RAN4, and rapporteur suggests we should let RAN4 conclude on those.

Proposal 2: Scaling factor(s) for relaxed RRM measurements are left to RAN4 to decide.

Proposal 3: How/if higher priority frequencies RRM measurements are relaxed is left to RAN4 to decide.

Proposal 4: Whether RRM measurements relaxation is allowed on a frequency when the UE is configured to perform early measurement on that frequency and T331 is running is left to RAN4 to decide.

# Conclusion

The following proposals capture the outcome suggested by the rapporteur of this summary:

Proposal 1: Continue the discussion in an offline focusing on:

1) FR1/FR2 differentiation only and

2) understanding better the motivation scenarios and why they cannot be addressed by existing high-priority frequency framework.

Proposal 2: Scaling factor(s) for relaxed RRM measurements are left to RAN4 to decide.

Proposal 3: How/if higher priority frequencies RRM measurements are relaxed is left to RAN4 to decide.

Proposal 4: Whether RRM measurements relaxation is allowed on a frequency when the UE is configured to perform early measurement on that frequency and T331 is running is left to RAN4 to decide.

# Reference

- R2-2002665 UE power saving for inter frequency measurements, Sony

- R2-2002735 Configurations for RRM Measurement Relaxation, MediaTek Inc.

- R2-2002867 Configurations for RRM Measurement Relaxation, vivo

- R2-2002950 Correction of SI update of relaxed measurement parameters, Nokia, Nokia Shanghai Bell, Ericsson

- R2-2003219 EMR issue on relaxed measurement, LG Electronics Inc.

- R2-2003288 Open issues UE capability, DCP, UE assistance and RRM relaxation, Ericsson

- R2-2002791 Report of [Post109e#44][PowSav] RRM open issues, CATT

- R4-2002338 WF on RRM measurement relaxation for Power Saving, CATT, vivo