3GPP TSG-RAN WG2 Meeting #109-bis electronic	Draft_R2-2004085

20 – 30 April 2020

Agenda Item:		6.4.2.1

Source:	Huawei, HiSilicon

Title:	Proposed easy agreements on RRC for 5G V2X with NR SL

Document for:	Agreement

# Introduction

This document proposes the easy agreements as per the summary in [Offline-701], and it is submitted for approval.

- [AT109bis-e][701][V2X] RRC open issues and ASN.1 class2/3 issues (Huawei)

Scope: Discuss and conclude issues of “to be discussed” in the open issues and 38.331 ASN.1 class2/3 issues in R2-2003519 and R2-2002918 => List of proposals for easy agreements which we can approve via email.

Expected outputs: Proposals and summary in R2-2004071 => List of proposals for easy agreements for email approval in R2-2004085

Deadline: 4/30 10:00 (UTC) for rapporteur’s proposals

Status: Ongoing

# Proposed Agreements

Proposal 1: Gather the PHY-MAC-RLC related SL configurations in SL-ConfigDedicatedNR into the same IE, i.e. SL-PHY-MAC-RLC-Config-r16, which can be signalled from DU to CU.

 Supporting vs. non-supporting: 12:1

Proposal 1a: RAN2 adopt the changes in the Appendix to TS 38.331.

 Supporting vs. non-supporting: 11:2

Proposal 2: No support of delta-RSRP based event for event triggered SL RSRP reporting in this release.

Proposal 3: Remove the field of sl-HeaderCompression from RRCReconfigurationSidelink, and, as in LTE SL/V2X SL, pre-configure header compression related parameters for NR SL.

Proposal 4: Number SL-SRB configurations for SCCH, with:

- 0: SL-SRB configuration carrying PC5-S messages that are not protected

- 1: SL-SRB configuration carrying PC5-S messages "Direct Security Mode Command" and "Direct Security Mode Complete"

- 2: SL-SRB configuration carrying other PC5-S messages that are protected

- 3: SL-SRB configuration carrying PC5-RRC messages

 Supporting vs. non-supporting: 11:1

Proposal 4: In TS 38.331, specify that the UE shall release the configured sidelink grant type 1, if T311 is running.

 Supporting vs. non-supporting: 11:2

Proposal 4a: In TS 38.321, clarify the NOTE 1 in 5.22.1.1 (used to prohibit simultaneous mode-1/2) as follows:

- NOTE 1:	If the MAC entity has been configured by RRC to transmit using SL-RNTI or SLCS-RNTI but is configured by RRC to transmit using a pool of resources in a carrier as indicated in TS 38.331 [5], the MAC entity can create a configured sidelink grant ‘based on random selection or sensing’ only after releasing any configured sidelink grant ‘type 1 (if any)’.

 Supporting vs. non-supporting: 8:3

# Conclusion

It is proposed that RAN2 agree the proposed agreements in Section 2.

# Reference

- R2-2004071	Summary of offline discussion [AT109bis-e][701][V2X] RRC open issues and ASN.1 class2/3 issues	Huawei (Rapporteur)

# Appendix: Changes for Proposal 1

- –	SL-ConfigDedicatedNR

The IE SL-ConfigDedicatedNR specifies the dedicated configuration information for NR sidelink communication.

SL-ConfigDedicatedNR information element

-- ASN1START

-- TAG-SL-CONFIGDEDICATEDNR-START

SL-ConfigDedicatedNR-r16 ::=         SEQUENCE {

sl-ScheduledConfig-r16               SetupRelease { SL-ScheduledConfig-r16 }                                OPTIONAL,    -- Need M

sl-UE-SelectedConfig-r16             SetupRelease { SL-UE-SelectedConfig-r16 }                              OPTIONAL,    -- Need M

sl-FreqInfoToReleaseList-r16         SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF ARFCN-ValueNR                OPTIONAL,    -- Need N

sl-FreqInfoToAddModList-r16          SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF SL-FreqConfig-r16            OPTIONAL,    -- Need N

sl-RadioBearerToReleaseList-r16      SEQUENCE (SIZE (1..maxNrofSLRB-r16)) OF SLRB-Uu-ConfigIndex-r16        OPTIONAL,    -- Need N

sl-RadioBearerToAddModList-r16       SEQUENCE (SIZE (1..maxNrofSLRB-r16)) OF SL-RadioBearerConfig-r16       OPTIONAL,    -- Need N

sl-RLC-BearerToReleaseList-r16       SEQUENCE (SIZE (1..maxSL-LCID-r16)) OF SL-RLC-BearerConfigIndex-r16    OPTIONAL,    -- Need N

sl-RLC-BearerToAddModList-r16        SEQUENCE (SIZE (1..maxSL-LCID-r16)) OF SL-RLC-BearerConfig-r16         OPTIONAL,    -- Need N

sl-MeasConfigInfoToReleaseList-r16   SEQUENCE (SIZE (1..maxNrofSL-Dest-r16)) OF SL-DestinationIndex-r16     OPTIONAL,    -- Need N

sl-MeasConfigInfoToAddModList-r16    SEQUENCE (SIZE (1..maxNrofSL-Dest-r16)) OF SL-MeasConfigInfo-r16       OPTIONAL,    -- Need M

t400-r16                             ENUMERATED {ms100, ms200, ms300, ms400, ms600, ms1000, ms1500, ms2000} OPTIONAL,    -- Need M

sl-CSI-Acquisition-r16               ENUMERATED {enabled}                                                   OPTIONAL,    -- Need N

sl-CSI-SchedulingRequestId-r16       SchedulingRequestId                                                    OPTIONAL,    -- Need N

sl-SSB-PriorityNR-r16                INTEGER (1..8)                                                         OPTIONAL,    -- Need N

sl-PUCCH-Config-r16                  PUCCH-Config                                                           OPTIONAL,    -- Need N

sl-PDCCH-Config-r16                  PDCCH-Config                                                           OPTIONAL,    -- Need N

networkControlledSyncTx-r16          ENUMERATED {on, off}                                                   OPTIONAL,    -- Need N

sl-PHY-MAC-RLC-Config-r16            OCTET STRING (CONTAINING SL-PHY-MAC-RLC-Config-r16)                    OPTIONAL,    -- Need M

...

}

SL-PHY-MAC-RLC-Config-r16::=         SEQUENCE {

sl-ScheduledConfig-r16               SetupRelease { SL-ScheduledConfig-r16 }                                OPTIONAL,    -- Need M

sl-UE-SelectedConfig-r16             SetupRelease { SL-UE-SelectedConfig-r16 }                              OPTIONAL,    -- Need M

sl-FreqInfoToReleaseList-r16         SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF ARFCN-ValueNR                OPTIONAL,    -- Need N

sl-FreqInfoToAddModList-r16          SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF SL-FreqConfig-r16            OPTIONAL,    -- Need N

sl-RLC-BearerToReleaseList-r16       SEQUENCE (SIZE (1..maxSL-LCID-r16)) OF SL-RLC-BearerConfigIndex-r16    OPTIONAL,    -- Need N

sl-RLC-BearerToAddModList-r16        SEQUENCE (SIZE (1..maxSL-LCID-r16)) OF SL-RLC-BearerConfig-r16         OPTIONAL,    -- Need N

sl-CSI-Acquisition-r16               ENUMERATED {enabled}                                                   OPTIONAL,    -- Need N

sl-CSI-SchedulingRequestId-r16       SchedulingRequestId                                                    OPTIONAL,    -- Need N

sl-SSB-PriorityNR-r16                INTEGER (1..8)                                                         OPTIONAL,    -- Need N

sl-PUCCH-Config-r16                  PUCCH-Config                                                           OPTIONAL,    -- Need N

sl-PDCCH-Config-r16                  PDCCH-Config                                                           OPTIONAL,    -- Need N

networkControlledSyncTx-r16          ENUMERATED {on, off}                                                   OPTIONAL,    -- Need N

}

SL-DestinationIndex-r16  ::=             INTEGER (0..maxNrofSL-Dest-1-r16)

-- TAG-SL-CONFIGDEDICATEDNR-STOP

-- ASN1STOP