3GPP TSG-RAN WG2 Meeting #109bis-e                                      R2-2004083

Electronic, 20 April – 30 April 2020

Title:	LS response to SA3 on the security related issues for NR SL

Response to:

Release:	Rel-16

Work Item:	5G_V2X_NRSL-Core

Source:	RAN2

To:	SA3

Cc:	CT1

Contact Person:

#### Name:	Da Wang

#### E-mail Address:	wangda@catt.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1. Overall Description

RAN2 would like to thank SA3 for the reply LS on NR V2X Security issue and PDCP SN size (S3-200478) to inform RAN2 of the security related fields/information for AS ciphering and integrity protection.

- Regarding 5-bit input for the security algorithms derived from a specific LCID, RAN2 has made following working assumption.

1: Working assumption: the 5 least significant bits of LCID can be used as 5-bit input to the ciphering/integrity algorithms. Working assumption will be RAN2 agreement if there is no SA3 concern until next RAN2 meeting.

Please note that the 5 least significant bits of LCID can uniquely identify SLRB in this release, as specified in TS 38.321:

- Table 6.2.4-1 Values of LCID for SL-SCH

| Index | LCID values |
| --- | --- |
| 0 | SCCH carrying PC5-S messages that are not protected |
| 1 | SCCH carrying PC5-S messages "Direct Security Mode Command" and "Direct Security Mode Complete" |
| 2 | SCCH carrying other PC5-S messages that are protected |
| 3 | SCCH carrying PC5-RRC messages |
| 4-19 | Identity of the logical channel |
| 20-61 | Reserved |
| 62 | Sidelink CSI Reporting |
| 63 | Padding |

- RAN2 agreed PDCP re-establishment is triggered for re-keying procedure. In order to better understand the impact on AS layer, RAN2 respectfully ask SA3 the following question:

Question 1: What is the granularity of the re-keying procedure, e.g., per PC5 unicast link?

- Regarding Counter Check procedure, RAN2 would like to ask SA3 the following question:

Question 2: Is Counter Check procedure needed in PC5 interface for SL unicast DRB?

2. Actions:

RAN2 respectfully asks SA3 to take the above information into account for subsequent specification work and provide their feedback for above questions.

3. Date of Next RAN2 Meetings:

TSG-RAN WG2 Meeting #110	1 – 12 June 2020	Electronic

TSG-RAN WG2 Meeting #111	24 – 28 August 2020	Toulouse, France