3GPP TSG-RAN WG2 Meeting #109bis-e					R2-20xxxxx

Electronic, 20 April – 30 April 2020

Agenda Item:		6.12.2	MDT

Source:			Huawei

Title:				Open issues on MDT

WI code(s):			NR_SON_MDT-Core

Document for:		Discussion and Decision

## 1	Introduction

In section 6.12.2 MDT, there is an email discussion as below:

- [109bis-e][801] Open issues on MDT (Huawei)

Scope: Continue the discussion on MDT open issues based on R2-2003798. Focus on the following proposals coloured in red.

Intended outcome: Summary with the following sets which should be identified

§  Set of proposals with full consensus, if any (agreeable over email)

§  Set of proposals with almost full consensus to discuss in the follow up conference call

§  Set of open issues and proposals to postpone to next meeting

Deadline: 25/04/2019 22:00 UTC

This paper is to progress on open issues on MDT.

## 2	Discussion

Please companies provide comments regarding the following open issues. And then, based on companies’ inputs, the email rapporteur will group proposals to indicated categories.

It is noted that more details on these proposals can be further found in [1] and relevant contributions.

Proposal 1: For location information in SCGFailureInformationEUTRA/ SCGFailureInformationNR, UE includes available WLAN measurement results, Bluetooth measurement results, and Sensors measurement results only from MN configured wlanNameList /btNameList/SensorNameList.

| Company | Is this proposal agreeable? (Yes/No) | Comments |
| --- | --- | --- |
| Qualcomm | Yes | From the today’s discussion, we agree that there are two benefits of having location information in SCGFailureInformationEUTRA/ SCGFailureInformationNR: - For the immediate response from the MN in the case of SCG Failure - SN SCG failure related parameter optimization It was argued that for (1.) locationInformation from the MN configuration is useful and for (2.) locationInformation from the SN configuration is desired. If we look at the WLAN/BT/Sensor configurations Bluetooth: LogMeasResultListBT-r16 ::= SEQUENCE (SIZE (1..maxBT-IdReport-r16)) OF LogMeasResultBT-r16 LogMeasResultBT-r16 ::= SEQUENCE { bt-Addr-r16 BIT STRING (SIZE (48)), rssi-BT-r16 INTEGER (-128..127) OPTIONAL, ... } WLAN: LogMeasResultListWLAN-r16 ::= SEQUENCE (SIZE (1..maxWLAN-Id-Report-r16)) OF LogMeasResultWLAN-r16 LogMeasResultWLAN-r16 ::= SEQUENCE { wlan-Identifiers-r16 WLAN-Identifiers-r16, rssiWLAN-r16 WLAN-RSSI-Range-r16 OPTIONAL, rtt-WLAN-r16 WLAN-RTT-r16 OPTIONAL, ... } WLAN-Identifiers-r16 ::= SEQUENCE { ssid-r16 OCTET STRING (SIZE (1..32)) OPTIONAL, -- Need OR bssid-r16 OCTET STRING (SIZE (6)) OPTIONAL, -- Need OR hessid-r16 OCTET STRING (SIZE (6)) OPTIONAL, -- Need OR ... } Sensor: Sensor-LocationInfo-r16 ::= SEQUENCE { sensor-MeasurementInformation-r16 OCTET STRING OPTIONAL, sensor-MotionInformation-r16 OCTET STRING OPTIONAL, ... } We believe that the bluetooth and WLAN addresses are public address, and Network can easily determine the UE location based on GNSS, WLAN, bluetooth measurements. However, for imidiate actions to retrive from SCG Failure, MN configured location information is more useful. Furthermore, to capture the SN configured location in MN RCC formate, UE need extra capability for converting the one RRC formate to another. Network has no strict power and processing capability restriction as UE. Network can convert the MCG location information format to SCG location format easily. Thus, we belive that capaturin the location information from MN configuration has more advantage then capturing from SN configuration. |
| Ericsson | No | There are two aspects of locationInformation included in the SCGFailureInformation messages. 1) Location information included directly in SCGFailureInformationEUTRA/ SCGFailureInformationNR messages using MN RRC format 2) Location information included in measResultSCG-r15 of SCGFailureInformationEUTRA/ SCGFailureInformationNR messages using SN RRC format. In our understanding, the location information included in 1) is based on the MN configured WLAN AP and/or Bluetooth beacon related measurements and is in MN RRC format. Based on this information, the MN can quickly setup a new candidate SN for this UE upon receiving the SCGFailureInformation. The location information included in 2) is based on the SN configured WLAN AP and/or Bluetooth beacon related measurements and is in SN RRC format. This information is needed for SN to identify where the failure had occurred based on its configuration. This information will be used by SN to tune the handover parameters just like the MRO function uses the location information in the RLF report. It is very much possible that the WLAN AP and/or Bluetooth beacons configured by MN and SN are different. MN could be a low band cell and SN could be a high band cell. In such a scenario, the MN configured WLAN APs could be many more than the SN configured APs and they could be independent from one another. |
| Huawei, HiSilicon | No | We share the similar view as Ericsson. We also think that the WLAN AP and/or Bluetooth beacons configured by MN and SN may be different, and we see benefits to have independent configurations for MN and SN. |
| Intel | Yes | Reporting MN configuration measurement result should be baseline. Agree with QC that it requires additional UE processing so for reporting SN configuration can be delayed to later release when there is strong need. |
| Nokia, Nokia Shanghai Bell | Yes | Focusing on the EN-DC case (even though the agreed messages here go beyond EN-DC only), we note the locationInfo may result in duplicated data, which should be avoided. On the other hand, we think that its more important to understand and ensure consistency between configuration and what is reported in location information. I.e. locatonInfo included in the SCGFailureInformationNR uses, indeed, MN RRC format, but we have not found any evidence in 36.331 it will refer to SN configured AP/beacons etc. |
| CATT | No | Agree the analysis from Ericsson: 1) Location information included directly in SCGFailureInformationEUTRA/ SCGFailureInformationNR messages using MN RRC format is based on wlanNameList /btNameList/SensorNameList configuration from MN. 2) Location information included in measResultSCG-r15 of SCGFailureInformationEUTRA/ SCGFailureInformationNR messages using SN RRC format is based on wlanNameList /btNameList/SensorNameList configuration from SN. We also see some benefits to have independent configurations from MN and SN. |
| ZTE | No | We share the same understanding as Ericsson and Huawei, the location information in SCGFailureInformation is to assist both MN and SN. Since the MN and SN can independently configure the WLAN/BLNameList, the location info included in the two places is no necessary be the same. |
| vivo | Yes | Agree with Nokia, the focus should be on EN-DC. In such case MN configuration measurement result should be baseline. |
| MediaTek | Yes | Agree with Intel and Qualcomm |
| Samsung | Yes | We would like to avoid making duplicated data, and it’s UE burden and unnecessary signalling overhead. |

Proposal 2: If MN does not configure wlanNameList /btNameList/SensorNameList of immediate MDT, location information related to WLAN measurement, Bluetooth measurements, and Sensors measurement respectively of SCGFailureInformationEUTRA/ SCGFailureInformationNR are not included in the report.

| Company | Is this proposal agreeable? (Yes/No) | Comments |
| --- | --- | --- |
| Qualcomm | Yes | See previous discussion in proposal 1. |
| Ericsson | No | Based on our answer for previous question, we believe that the UE shall not include the location information based on MN configured WLAN AP and/or Bluetooth beacon information when the MN does not configure the same. But if the SN has configured the WLAN AP and/or Bluetooth beacon information, then the UE shall include the location information in measResultSCG-r15 of SCGFailureInformationEUTRA/ SCGFailureInformationNR messages. |
| Huawei, HiSilicon | No | See previous discussion in proposal 1. |
| Intel | Yes | See previous discussion in proposal 1. |
| Nokia, Nokia Shanghai Bell | Yes | In our view, current procedures in 36.331 nor signalling (while focusing on EN-DC), do not consider the case of SN configuration for BT/WLAN/Sensor having impact on reported data. If the proposal is agreed – no or limited changes to both RRCs are needed |
| CATT | No | See previous discussion in proposal 1. |
| ZTE | No | Please refer to comments in P1. |
| vivo | Yes | Please refer to comments in P1. |
| MediaTek | Yes | Refer to comments in P1 |
| Samsung | Yes |  |

Summary for P1 and P2:

- Ok: 6 companies

- Not ok: 4 companies

From the email rapporteur point of view, there is still no consensus on both P1 and P2, so it is proposed to postpone them to the next RAN2 meeting.

Proposal X-1: P1 and P2 are postponed to the next RAN2 meeting.

Proposal 8	Modify the procedural text to indicate that the UE shall log anyCellSelectionDetected flag indication and the last serving cell related measurements upon entering any cell selection state only when the UE is configured with periodical logged MDT.

| Company | Is this proposal agreeable? (Yes/No) | Comments |
| --- | --- | --- |
| Qualcomm | Yes | As mentioned by Ericsson, it was agreed in the last meeting that the UE shall not log previous serving cell upon entering any cell selection state for the OOC event as it is just the leaving condition. Thus, proposal seems oaky. |
| Ericsson | Yes | We believe that there are two parts to this proposal. 1) Whether to include the ‘anyCellSelectionDetected’ flag for the event triggered logged MDT scenarios or not 2) Whether to include the last serving cell related measurements upon entering any cell selection state or not. As of now, there are two event triggered logged MDT configurations agreed. One of them is OOC and the other being event L1. When the UE is configured with OOC, by default the UE has to be in any cell selection state and therefore, there is no benefit of this information. In the case of event L1, the purpose of co nfiguring event driven logged MDT was to record when the serving cell quality is below a threshold. However, when the UE is in any cell selection state, there is no serving cell and therefore, the event L1 is not applicable. For the issue of 2), this was already agreed that the UE shall not store the last serving cell related measurements upon entering any cell selection state. This is not the behaviour captured in the procedural text, in fact it is the opposite. So, we propose to change the procedural text to capture the agreement from previous meeting. |
| Huawei, HiSilicon | No | In LTE, we introduced this flag for OOC for logged MDT. For LTE MDT, only periodical logged MDT is supported. For NR, we think this flag is still needed for event triggered logged MDT. Here are our opinions: For OOC, if this flag is not used, how the UE indicates the OOC in its logged measurements to the network? For eventL1, it is still possible for the UE to suffer OOC, and then it is useful for the UE to use this flag to indicate the OOC (similar as the UE behaviour for periodical logged MDT). |
| Intel | Yes | Agree with E/// observation and analysis. |
| Nokia, Nokia Shanghai Bell | No | In our understanding, the proposal (and previous agreements) has good intention, but the corresponding CRs would not introduce intended behaviour. The flag for being in OOC is still needed for regular periodical reporting, as well as for event triggered reporting for event=OOC. The should log OOC flag in both cases, but for: Periodical – the UE does in periodical manner the logging, and if it is able to select a cell and camp on it, it will continue periodical measurement logging OOC-event based – the UE does not log anything, unless it detects anyCellSelecton state. If it does, it will log the state and will end the logging after leaving condition is met |
| CATT | Yes | ‘anyCellSelectionDetected’ flag is introduced in LTE R15, which is only applicable for Periodical logged MDT as only Periodical logged MDT is supported for LTE, so this is no doubt that this flag can be reused for NR Periodical logged MDT, when it comes to event triggered logged MDT, this flag is redundant: For OOC event triggered logged MDT, once OOC event triggered logged MDT fulfilled, relativeTimeStamp in each log MDT entry is mandatorily present, this IE can implicitly indicate that this is recorded for OOC event triggered logged MDT as for ‘camp on any cell’, No log MDT entry wil be recorded. For L1 event triggered logged MDT,OOC will not trigger L1 event triggered logged MDT condition, so ‘anyCellSelectionDetected’ flag is useless in this case also. As for including the last serving cell related measurements upon entering any cell selection state, we agree with Ericsson. |
| ZTE | No | We share the same understanding as Huawei and Nokia, NW need this flag to know the results is taken in case of OOC. And we can add “if available” as shown below to prevent including the last serving cell info when event type is set to OOC, since in such case this kind of information wouldn’t be available. 3> if the UE is in any cell seletion state (as specificed in TS 38.304 [20]): 4> set anyCellSelectionDetected to indicate the detection of no suitable or no acceptable cell found; 4> set the servCellIdentity if available to indicate global cell identity of the last logged cell that the UE was camping on; 4> set the measResultServCell if available to include the quantities of the last logged cell the UE was camping on; As for eventL1, we think after the log is triggered, UE might suddenly goes into OOC, in such case it is useful to log the OOC information, e.g. anyCellSelectionDetected flag, last serving cell ID and etc., when eventL1 is configured. |
| vivo | Yes | Agree with QC |
| MediaTek | Yes |  |
| Samsung | No | We would like to emphasize that NW can configure only either periodic logged MDT or event-triggered logged MDT. If the proposal is agreed, there is no way for NW to identify the information, which is so helpful for MDT purpose. Since we need not introduce something new in the technical aspect, we prefer not to support the proposal, i.e. no reason to restrict it. |

Summary for P8:

- Ok: 6 companies

- Not ok: 4 companies

It is seen that 4 companies have concerns on P8 and they think the flag is useful for both periodical and event triggered logged MDT.

From the email rapporteur point of view, there is still no consensus on both P8. 4 companies have concerns on P8 and they think the flag is useful for both periodical and event triggered logged MDT. So it is proposed to postpone it to the next RAN2 meeting.

Proposal X-2: P8 is postponed to the next RAN2 meeting.

Proposal 12: In order to ensure the management based logged MDT do not overwrite the signalling based logged MDT:

Include the logged MDT type (i.e. the management based MDT or the signalling based MDT) in the logged MDT configuration

Include the logged MDT type in the RRCConnectionComplete/RRCResumeComplete/ RRCReconfigurationComplete/RRCRestablishmentCompelete

| Company | Is this proposal agreeable? (Yes/No) | Comments |
| --- | --- | --- |
| Qualcomm | No | UE’s MDT configuration can be transferred in mobility. We believe that this topic should be handled by network. Also, we believe that the NG-RAN can easily avoid it by checking the MDT configurations. |
| Ericsson | No | This is under discussion in RAN3 and they are handling this as part of their discussion for the past two meetings. As they might converge on a solution in this meeting, we believe that the network based solution can work in this scenario and therefore we do not need the UE based solution for this issue. |
| Huawei, HiSilicon | Yes | We still think P12 is workable and it can efficiently solve the priority issue of two types of logged MDT. If RAN3 is also discussing this issue as well as solutions, we think we can wait for their progresses. |
| Intel | No | We think that type is not needed, network can know this information based on network implementation. |
| Nokia, Nokia Shanghai Bell | No | A solution should be transparent to RRC |
| CATT | No | This issue is also existing in LTE, better to wait more input from RAN3. |
| ZTE | No | We acknowledged there is an on-going dicussion in RAN3, and we think maybe we can wait for their decision on this. Even if RAN3 decide they prefer the UE-based solution, we still think there could be simpler solutions than to include the type indication since it is not always needed. Our suggestion is to wait for RAN3’s conclusion and maybe further discuss it next meeting. |
| vivo | No | This is being discussed by RAN3, further we think the configuration can be transparent to RRC |
| MediaTek | No |  |
| Samsung | No | It’s tiny optimization. Furthermore, in some cases, it could not be in line with operator’s intention. |

Summary for P12:

- Ok: 1 company

- Not ok: 9 companies

It is seen that most of companies are not fine with P12, so no action is needed for P12.

Proposal 13: To revise the agreement for signalling based MDT from:

”When the UE resumes the RRC connection in one new NG-RAN, the last serving NG-RAN can propagate the logged MDT configuration to the new NG-RAN.”

To:

”When the UE resumes the RRC connection in one new NG-RAN, the new NG-RAN can configure the MDT configuration for the UE, only if the signalling based logged MDT was received by the NG-RAN. It is not required to propagate the logged MDT configuration.”

| Company | Is this proposal agreeable? (Yes/No) | Comments |
| --- | --- | --- |
| Qualcomm | NO | We believe that the two sentences are not so different. While in the current spec. the propagation of MDT configuration from old to new NG-RAN is not mandate, both sentences capture that. If the intention of the proposal is to make it clear then it should be formatted as: When the UE resumes the RRC connection in one new NG-RAN, the new NG-RAN can configure the MDT configuration for the UE, only if the signalling based logged MDT was received by the new NG-RAN from the previous NG-RAN or AMF. Otherwise, It is not required to propagate the logged MDT configuration.” Furthermore, this topic is also under discussion whether the propagation of signalling based configuration should always be propagated to new NG-RAN from old NG-RAN or AMF. Thus, depending upon the outcome of RAN3 discussion, the sentence can be modified. Though, we don’t believe that there is any need of making the proposed change. |
| Ericsson | Yes | We agree but with a small modification. ”When the UE resumes the RRC connection in one new NG-RAN, the new NG-RAN can configure the MDT configuration for the UE, only if the signalling based logged MDT was received by the NG-RAN. It is not required to propagate the management based logged MDT configuration.” |
| Huawei, HiSilicon | Yes |  |
| Intel | No | Agree with QC that the two sentences are very similar. Change from QC looks ok to us. |
| Nokia, Nokia Shanghai Bell | Yes |  |
| CATT | Yes |  |
| ZTE | Yes,but | We agree with the intention is to only support propagate the signalling based MDT configuration therefore we suggest to revise to following sentence: ”When the UE resumes the RRC connection in one new NG-RAN, the last serving NG-RAN can propagate the signalling based logged MDT configuration to the new NG-RAN.” |
| vivo | No strong view |  |
| MediaTek | No strong view |  |
| Samsung | No |  |

Summary for P13:

- Ok: 5 companies, but with some minor improvements on the wording

- Not ok: 3 companies. One company indicates that RAN2 should wait for RAN3 discussion

- No strong opinion: 2 companies

Based on the comments received so far, even if there are more supports on changing the wording, still there are some concerns and one conern is about the ongoing RAN3 discussion. So it is suggested to continue discussing it in MDT online session in 2nd meeting week.

Proposal X-3: It is proposed RAN2 to decide on:

- Option 1: change the wording from

“When the UE resumes the RRC connection in one new NG-RAN, the last serving NG-RAN can propagate the logged MDT configuration to the new NG-RAN.”

To: (red and blue texts are from two companies)

”When the UE resumes the RRC connection in one new NG-RAN, the new NG-RAN can configure the MDT configuration for the UE, only if the signalling based logged MDT was received by the new NG-RAN from the previous NG-RAN or AMF. It is not required to propagate the management based logged MDT configuration.”

- Option 2: postpone the discussion to the next RAN2 meeting

The following proposal 9 and 10 were from [1], and there were no online agreements for both proposals. It is proposed to also discuss them in this email discussion.

Proposal 9      Create a new IE MeasQuantityResultsLogged-r16 for capturing the measurement quantities included in the logged MDT.

| Company | Is this proposal agreeable? (Yes/No) | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes |  |
| Ericsson | Yes |  |
| Qualcomm | Yes |  |
| Intel | Yes |  |
| Nokia, Nokia Shanghai Bell | Yes |  |
| CATT | No strong view |  |
| ZTE | No strong view |  |
| vivo | No strong view |  |
| MediaTek | No strong view |  |
| Samsung | No | We would like to minimize to introduce new IEs for a light NR specification. SINR value in existing IE MeasQuantityResults is optional, i.e. always not needed in the IE: MeasQuantityResults ::= SEQUENCE { rsrp RSRP-Range OPTIONAL, rsrq RSRQ-Range OPTIONAL, sinr SINR-Range OPTIONAL } Furthermore, if required, we can add a text in the field description of the IE MeasQuantityResults, e.g. sinr is not included when it is used for MDT. |

Summary for P9:

- Ok: 5 companies

- Not ok: 1 company

- No strong view: 4 companies

One company would like to re-use the existing IE MeasQuantityResults and perhaps a clarification in the field description, which could minimize the impacts to 38.331.

Proposal X-4: It is proposed RAN2 to discuss two options:

Option 1: Create a new IE MeasQuantityResultsLogged-r16 for capturing the measurement quantities included in the logged MDT.

Option 2: Re-use the existing IE MeasQuantityResults and if required, add a text in the field description of the IE MeasQuantityResults, e.g. sinr is not included when it is used for MDT.

Proposal 10    mobilityState field in included only when the serving cell broadcasts speedStateReselectionPars in SIB2.

| Company | Is this proposal agreeable? (Yes/No) | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes |  |
| Ericsson | Yes |  |
| Qualcomm | Yes |  |
| Intel | Yes |  |
| Nokia, Nokia Shanghai Bell | Yes |  |
| CATT | Yes |  |
| ZTE | Yes |  |
| vivo | Yes |  |
| MediaTek | Yes |  |
| Samsung | Yes |  |

Summary for P10:

- Ok: 10 companies

- Not ok: 0 company

It is seen that P10 is agreeable.

Proposal X-5: mobilityState field in included only when the serving cell broadcasts speedStateReselectionPars in SIB2.

## 3	Conclusions

Based on the discussions made in section 2, it can be seen that:

- For some proposals, there are not enough supports or there are strong concerns, so it is suggested to postpone them to the next RAN2 meeting

- For some proposals, there are consensuses

So it is proposed:

Proposal X-1: P1 and P2 are postponed to the next RAN2 meeting.

Proposal X-2: P8 is postponed to the next RAN2 meeting.

Proposal X-3: It is proposed RAN2 to decide on:

- Option 1: change the wording from

“When the UE resumes the RRC connection in one new NG-RAN, the last serving NG-RAN can propagate the logged MDT configuration to the new NG-RAN.”

To: (red and blue texts are from two companies)

”When the UE resumes the RRC connection in one new NG-RAN, the new NG-RAN can configure the MDT configuration for the UE, only if the signalling based logged MDT was received by the new NG-RAN from the previous NG-RAN or AMF. It is not required to propagate the management based logged MDT configuration.”

- Option 2: postpone the discussion to the next RAN2 meeting

Proposal X-4: It is proposed RAN2 to discuss two options:

Option 1: Create a new IE MeasQuantityResultsLogged-r16 for capturing the measurement quantities included in the logged MDT.

Option 2: Re-use the existing IE MeasQuantityResults and if required, add a text in the field description of the IE MeasQuantityResults, e.g. sinr is not included when it is used for MDT.

Proposal X-5: mobilityState field in included only when the serving cell broadcasts speedStateReselectionPars in SIB2.

## 4	References

[1] R2-2003798	Summary on MDT	Huawei