3GPP TSG RAN WG2#110-e	R2-2004352

Online meeting, 1st - 12th June, 2020

3GPP TSG RAN WG1#100bis-e		R1-2003040

e-Meeting, April 20th – 30th, 2020

Title:	LS on aligning RRC parameter list with TS38.213

Release:	16

Work Item:	NR_unlic-Core

Source:	RAN WG1

To:	RAN WG2

Cc:

Contact Person:

#### Name:	Alexander Golitschek

Tel. Number:

E-mail Address:	aelbwart@lenovo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None.

1. Overall Description:

For search space set switching in developed in Rel-16 NR-U WI, the following RRC parameters are defined:

- searchSpaceSwitchTrigger-r16

- searchSpaceSwitchingGroup-r16

- searchSpaceSwitchingTimer-r16

- searchSpaceGroupIdList-r16

A search space switching trigger in DCI 2_0 allows the gNB to explicitly signal switching of search space monitoring between two groups of search space sets for one or more groups of serving cells. The parameter SearchSpaceSwitchTrigger-r16 in the SlotFormatIndicator IE configures the position in DCI 2_0 of the trigger bit (positionInDCI parameter). It should also indicate to which serving cell the trigger applies. In addition, serving cells can be grouped together so that the trigger applies to all cells in the group. Each serving cell group is configured with the higher layer parameter searchSpaceSwitchingGroup-r16 containing up to 16 serving cells.

Furthermore, the parameter searchSpaceGroupIdList-r16 configured within the SearchSpace IE assigns one or two group indices, with possible values 0 or 1, to each search space set.  This parameter indicates which search space set should be monitored according to the switching procedure in [clause 10.4. in 38.213]. The searchSpaceGroupIdList-r16 configuration is only relevant within the SearchSpace IE.

In the current version of 38.331, there is a CHOICE to configure either servingCellId or groupId within the  searchSpaceSwitchTrigger-r16 parameter in SlotFormatIndicator information element. From RAN1's perspective, according to the mechanism outlined above, such a choice serves no purpose, and it is sufficient to include just servingCellId.

Based on above discussion we ask RAN2 to modify the SearchSpaceSwitchTrigger-r16 parameter within the SlotFormatIndicator information element as follows:

searchSpaceSwitchTrigger-r16     SEQUENCE {

positionInDCI        INTEGER(0..maxSFI-DCI-PayloadSize-1),

id                   CHOICE {

servingCellId          ServCellIndex,

groupId                INTEGER (0..1)

}

} OPTIONAL, -- Need N

2. Actions:

To RAN2 group.

ACTION: 	RAN1 respectfully asks RAN2 to modify the SearchSpaceSwitchTrigger-r16 parameter within the SlotFormatIndicator information element as indicated above.

3. Date of Next TSG-RAN WG1 Meetings:

TSG-RAN WG1 Meeting #101	25th – 29th May 2020	e-Meeting

TSG-RAN WG1 Meeting #102	24th – 28th October 2020	Toulouse, France