3GPP TSG-RAN WG2 Meeting #110-e	R2-2004427

# Online, 1–12 June 2020

Agenda item:	6.9.5

Source:	Samsung

Title:	Clarification on tag-Config for DAPS (subject to [H223])

Document for:	Discussion and Agreement

# 1	Introduction

From the current specification, it is not clear whether tag-ToReleaseList and tag-ToAddModList apply to the source or target cell when DAPS is configured as in [H223]. Then rapporteur have updated the field description of tag-Config by adding 'The field is not present if any DAPS bearer is configured', which seems not desirable as it requires additional RRCReconfiguration message exchanges after handover.

The contribution provides alternative TP to allow configuration of tag-Config during DAPS HO.

# 2	Discussion

As described in clause 1 above, the latest running CR [1] does not allow configuring tag-Config, which is inefficient and requires additional RRCReconfiguration message exchanges after handover. We think network should be able to configure tag-Config even during DAPS HO, and the configuration, if configured, should be applied to the MAC entity of target cell as the configuration is generated by the target cell.

Proposal 1:	Network is allowed to configure tag-Config even during DAPS HO, and if configured, it is applied to the MAC entity of the target cell.

If the Proposal 1 above is agreeable, the specification can be updated to e.g. as follows:

##### 5.3.5.5.5 MAC entity configuration The UE shall: 1> if SCG MAC is not part of the current UE configuration (i.e. SCG establishment): 2> create an SCG MAC entity; 1> if daps-Config is configured for any DRB: 2> reconfigure the MAC main configuration for the target of the cell group in accordance with the received mac-CellGroupConfig excluding tag-ToReleaseList and tag-ToAddModList; 2> if the received mac-CellGroupConfig includes the tag-ToReleaseList: 3> for each TAG-Id value included in the tag-ToReleaseList that is part of the target MAC configuration: 4> release the TAG of the target MAC configuration indicated by TAG-Id; 2> if the received mac-CellGroupConfig includes the tag-ToAddModList: 3> for each tag-Id value included in tag-ToAddModList that is not part of the target MAC configuration (TAG addition): 4> add the TAG of the target MAC configuration, corresponding to the tag-Id, in accordance with the received timeAlignmentTimer; 3> for each tag-Id value included in tag-ToAddModList that is part of the target MAC configuration (TAG modification): 4> reconfigure the TAG of the target MAC configuration, corresponding to the tag-Id, in accordance with the received timeAlignmentTimer. 1> else: 2> reconfigure the MAC main configuration of the cell group in accordance with the received mac-CellGroupConfig excluding tag-ToReleaseList and tag-ToAddModList; 2> if the received mac-CellGroupConfig includes the tag-ToReleaseList: 3> for each TAG-Id value included in the tag-ToReleaseList that is part of the current UE configuration: 4> release the TAG indicated by TAG-Id; 2> if the received mac-CellGroupConfig includes the tag-ToAddModList: 3> for each tag-Id value included in tag-ToAddModList that is not part of the current UE configuration (TAG addition): 4> add the TAG, corresponding to the tag-Id, in accordance with the received timeAlignmentTimer; 3> for each tag-Id value included in tag-ToAddModList that is part of the current UE configuration (TAG modification): 4> reconfigure the TAG, corresponding to the tag-Id, in accordance with the received timeAlignmentTimer. … ### 6.3.2 Radio resource control information elements …

Proposal 2:	Update the running CR as proposed above, and remove the text 'The field is not present if any DAPS bearer is configured.' from the field description of tag-Config.

# 3	Conclusion

Proposal 1:	Network is allowed to configure tag-Config even during DAPS HO, and if configured, it is applied to the MAC entity of the target cell.

Proposal 2:	Update the running CR as proposed above, and remove the text 'The field is not present if any DAPS bearer is configured.' from the field description of tag-Config.

# 4	References

[1]	R2-190xxxx 38331 Rel16 ASN1 review Ph 1 v111.docx