3GPP TSG-RAN WG2 Meeting #110 electronic	R2-2004488

E-Meeting, June 1 – June 12 2020

Source:	vivo

Title:	Clarification for radioBearerConfig and radioBearerConfig2

Agenda Item:	5.4.1.1

Document for:	Discussion and Decision

- Introduction

In TS 38.331, the current field descriptions of radioBearerConfig and radioBearerConfig2 in RRCReconfiguration are insufficient to clearly express the corresponding usage. In this contribution, we will discuss the usage of radioBearerConfig and radioBearerConfig2.

- Discussion

According to the following field description of radioBearerConfig in RRCReconfiguration, we can conclude that in EN-DC, this field will be absent if the RRCReconfiguration is transmitted via SRB1, e.g., it is SN RRCReconfiguration received within nr-SecondaryCellGroupConfig.

radioBearerConfig Configuration of Radio Bearers (DRBs, SRBs) including SDAP/PDCP. In EN-DC this field may only be present if the RRCReconfiguration is transmitted over SRB3.

And the field description of nr-SecondaryCellGroupConfig in RRCConnectionReconfiguration also shows that SN RRCReconfiguration transmitted over SRB1 does not include radioBearerConfig.

nr-SecondaryCellGroupConfig Includes the NR RRCReconfiguration message as specified in TS 38.331 [82]. In this version of the specification, the NR RRC message only includes fields iab-F1AP-TransferOverSRB-r16, secondaryCellGroup, conditionalReconfiguration and/ or measConfig. If nr-SecondaryCellGroupConfig is configured, the network always includes this field upon MN handover to initiate an NR SCG reconfiguration with sync and key change.

Thus, we have the following observation:

Observation 1: For EN-DC, the field descriptions of radioBearerConfig and nr-SecondaryCellGroupConfig are consistent for that, the radioBearerConfig is absent if the RRCReconfiguration is received within nr-SecondaryCellGroupConfig.

In our understanding, the current field description of radioBearerConfig does not preclude to use this IE in NR-DC, NE-DC cases.

Observation 2: the field description of radioBearerConfig does not preclude to use this IE in NR-DC and NE-DC.

Moreover, the current field description of radioBearerConfig also does not preclude to use it in the following cases:

- the field can be present if the RRCReconfiguration is received within mrdc-SecondaryCellGroup.

- the field can be present in the RRCReconfiguration is received over SRB1 not within mrdc-SecondaryCellGroup.

- the field can be present if the RRCReconfiguration is received over SRB3.

However, according to the following field description of mrdc-SecondaryCellGroup IE, the RRCReconfiguration within mrdc-SecondaryCellGroup does not contain radioBearerConfig.

mrdc-SecondaryCellGroup Includes an RRC message for SCG configuration in NR-DC or NE-DC. For NR-DC (nr-SCG), mrdc-SecondaryCellGroup contains the RRCReconfiguration message as generated (entirely) by SN gNB. In this version of the specification, the RRC message can only include fields secondaryCellGroup and measConfig. For NE-DC (eutra-SCG), mrdc-SecondaryCellGroup includes the E-UTRA RRCConnectionReconfiguration message as specified in TS 36.331 [10]. In this version of the specification, the E-UTRA RRC message can only include the field scg-Configuration.

Thus in NR-DC, the field descriptions of radioBearerConfig and mrdc-SecondaryCellGroupConfig are inconsistent, for whether the radioBearerConfig is absent if the RRCReconfiguration is received within mrdc-SecondaryCellGroup.

Observation 3: For NR-DC, the field descriptions of radioBearerConfig and mrdc-SecondaryCellGroupConfig are inconsistent, for whether the radioBearerConfig is absent if the RRCReconfiguration is received within mrdc-SecondaryCellGroup.

Thus, we propose the following:

Proposal 1: Clarify the field description of radioBearerConfig, that in NR-DC it is only absent if the RRCReconfiguration is received within mrdc-SecondaryCellGroup.

Moreover, since that both radioBearerConfig and radioBearerConfig2 can be used in NR-DC and NE-DC, it is not clear how to use them. We would like to confirm which of the following belongs to:

1. The usage of RadioBearerConfig and radioBearerConfig2 in NE-DC and NR-DC are the same;

2. The usage of RadioBearerConfig and radioBearerConfig2 in NE-DC and NR-DC are different, e.g., RadioBearerConfig is used to configure MN terminated radio bearers via SRB1 and to configure SN terminated radio bearers via SRB3, while RadioBearerConfig2 is used to configure SN terminated bearer via SRB1.

radioBearerConfig2 Configuration of Radio Bearers (DRBs, SRBs) including SDAP/PDCP. This field can only be used if the UE supports NR-DC or NE-DC.

Thus, we propose the following:

Proposal 2: Clarify which of the following usages applies to radioBearerConfig and radioBearerConfig2 in NE-DC and NR-DC:

- Option1: The usage of RadioBearerConfig and radioBearerConfig2 in NE-DC and NR-DC are the same;

- Option2: RadioBearerConfig is used to configure MN terminated radio bearers via SRB1 and to configure SN terminated radio bearers via SRB3, while RadioBearerConfig2 is only used to configure SN terminated radio bearer via SRB1.

Proposal 3: Agree the corresponding CR in R2-2004489.

- Conclusion

In the contribution, we discussed the issue of xxx, and have made the following observations and proposals:

Observation 1: For EN-DC, the field descriptions of radioBearerConfig and nr-SecondaryCellGroupConfig are consistent for that, the radioBearerConfig is absent if the RRCReconfiguration is received within nr-SecondaryCellGroupConfig.

Observation 2: the field description of radioBearerConfig does not preclude to use this IE in NR-DC and NE-DC.

Observation 3: For NR-DC, the field descriptions of radioBearerConfig and mrdc-SecondaryCellGroupConfig are inconsistent, for whether the radioBearerConfig is absent if the RRCReconfiguration is received within mrdc-SecondaryCellGroup.

Proposal 1: Clarify the field description of radioBearerConfig, that in NR-DC it is only absent if the RRCReconfiguration is received within mrdc-SecondaryCellGroup.

Proposal 2: Clarify which of the following usages applies to radioBearerConfig and radioBearerConfig2 in NE-DC and NR-DC:

- Option1: The usage of RadioBearerConfig and radioBearerConfig2 in NE-DC and NR-DC are the same;

- Option2: RadioBearerConfig is used to configure MN terminated radio bearers via SRB1 and to configure SN terminated radio bearers via SRB3, while RadioBearerConfig2 is only used to configure SN terminated radio bearer via SRB1.

Proposal 3: Agree the corresponding CR in R2-2004489.

- References

[1] TS 38.331, V16.0.0.