3GPP TSG-RAN WG2 Meeting #110 electronic	R2-2004492

E-Meeting, June 1 – June 12 2020

Source:	vivo

Title:	The submission of embedded RRCReconfigurationComplete

Agenda Item:	6.0.1

Document for:	Discussion and Decision

- Introduction

In this contribution, we will discuss the misalignment issue between NR-DC and (NG)EN-DC, regarding to the submission of the embedded RRCReconfigurationComplete message.

- Discussion

From CR in [1], the submission action of the RRCReconfigurationComplete message in NR-DC are as follows:

5.3.5 RRC reconfiguration 5.3.5.3 Reception of an RRCReconfiguration by the UE < Omit…> 1> else if the RRCReconfiguration message was received via SRB1 within the nr-SCG within mrdc-SecondaryCellGroup (UE in NR-DC, mrdc-SecondaryCellGroup was received in RRCReconfiguration via SRB1): 2> if reconfigurationWithSync was included in spCellConfig in nr-SCG: 3> initiate the Random Access procedure on the PSCell, as specified in TS 38.321 [3]; 2> else 3> the procedure ends; NOTE 2a: The order in which the UE sends the RRCReconfigurationComplete message and performs the Random Access procedure towards the SCG is left to UE implementation. 1> else if the RRCReconfiguration message was received via SRB3 (UE in NR-DC): 2> if the RRCReconfiguration message was received within DLInformationTransferMRDC: 3> if the RRCReconfiguration message was received within the nr-SCG within mrdc-SecondaryCellGroup (NR SCG RRC Reconfiguration): 4> if reconfigurationWithSync was included in spCellConfig in nr-SCG: 5> initiate the Random Access procedure on the PSCell, as specified in TS 38.321 [3]; 4> else: 5> the procedure ends; 3> else: 4> submit the RRCReconfigurationComplete message via SRB1 to lower layers for transmission using the new configuration; 2> else: 3> submit the RRCReconfigurationComplete message via SRB3 to lower layers for transmission using the new configuration; 1> else (RRCReconfiguration was received via SRB1): 2> submit the RRCReconfigurationComplete message via SRB1 to lower layers for transmission using the new configuration; < Omit…>

According to the above procedure, for NR-DC, we can conclude that the RRCReconfigurationComplete will not to be directly submitted to lower layers if the corresponding RRCReconfiguration message belongs to the following cases:

- Case 1-1: it is the SCG RRCReconfiguration received via SRB1 within the nr-SCG within mrdc-SecondaryCellGroup. (Marked in yellow)

- Case 1-2: it is the SCG RRCReconfiguration received via SRB3 within the nr-SCG within mrdc-SecondaryCellGroup within DLInformationTransferMRDC. (Marked in blue)

For both Case 1-1 and Case 1-2, SCG RRCReconfigurationComplete is embedded in the corresponding MCG RRCReconfigurationComplete, and the latter will be submitted via SRB1 to the lower layers for transmission (Marked in Green).

Observation 1: In NR-DC, the submission action did not apply to the RRCReconfigurationComplete message which was going to be embedded in another RRCReconfigurationComplete message from current spec.

However, in (NG)EN-DC as shown in the following procedure, the RRCReconfigurationComplete needs to be directly submitted to lower layers if the corresponding RRCReconfiguration message belongs to the same cases as in NR-DC, i.e.,

- Case 2-1: it is the SCG RRCReconfiguration received via SRB1 within nr-SecondaryCellGroupConfig.

- Case 2-2: it is the SCG RRCReconfiguration received via SRB3 within nr-SecondaryCellGroupConfig within DLInformationTransferMRDC.

5.3.5 RRC reconfiguration 5.3.5.3 Reception of an RRCReconfiguration by the UE < Omit…> 1> if the UE is configured with E-UTRA nr-SecondaryCellGroupConfig (UE in (NG)EN-DC): 2> if the RRCReconfiguration message was received via E-UTRA SRB1 as specified in TS 36.331 [10]; or 2> if the RRCReconfiguration message was received via SRB3 within DLInformationTransferMRDC 3> submit the RRCReconfigurationComplete via E-UTRA embedded in E-UTRA RRC message RRCConnectionReconfigurationComplete as specified in TS 36.331 [10], clause 5.3.5.3/5.3.5.4; 3> if reconfigurationWithSync was included in spCellConfig of an SCG: 4> initiate the Random Access procedure on the SpCell, as specified in TS 38.321 [3]; 3> else: 4> the procedure ends; NOTE 1: The order the UE sends the RRCConnectionReconfigurationComplete message and performs the Random Access procedure towards the SCG is left to UE implementation. 2> else (RRCReconfiguration was received via SRB3) but not within DLInformationTransferMRDC: 3> submit the RRCReconfigurationComplete message via SRB3 to lower layers for transmission using the new configuration;

Observation 2: In (NG)EN-DC, the submission action applied to the RRCReconfigurationComplete message which was going to be embedded in the RRCConnectionReconfigurationComplete message from current spec.

In our understanding, the submission action of the RRCReconfigurationComplete message being embedded in the RRCConnectionReconfigurationComplete message in (NG)EN-DC is incorrect. If doing so, the lower layer will receive both RRCReconfigurationComplete message and RRCConnectionReconfigurationComplete message, and the latter will contain the former anyway, leading to signal overhead increased. Thus we propose:

Proposal 1: In (NG)EN-DC, the submission action does not apply to the RRCReconfigurationComplete message which is going to be embedded in the RRCConnectionReconfigurationComplete message.

Proposal 2: Agree the corresponding CR in R2-200XXXX.

- Conclusion

In the contribution, we discussed the issue of the submission of the embedded RRCReconfigurationComplete message in EN-DC, and have made the following observations and proposals:

Observation 1: In NR-DC, the submission action did not apply to the RRCReconfigurationComplete message which was going to be embedded in another RRCReconfigurationComplete message from current spec.

Observation 2: In (NG)EN-DC, the submission action applied to the RRCReconfigurationComplete message which was going to be embedded in the RRCConnectionReconfigurationComplete message from current spec.

Proposal 1: In (NG)EN-DC, the submission action should not apply to the RRCReconfigurationComplete message which is going to be embedded in the RRCConnectionReconfigurationComplete message.

Proposal2: Agree the corresponding CR in R2-2004493.

- References

[1] R2-2003882, CR for 38.331 on CA/DC Enhancements.