3GPP TSG-RAN WG2 Meeting #110-e                            R2-2004506

Electronic, 1st – 12th June 2020

Source:	vivo

Agenda Item:	5.4.3.1

Title:	Correction on UE capability constraints

Document for:	Discussion and Decision

# Introduction

For CGI reporting configuration, it is only allowed to configure CGI reporting for one neighbor cell at one time.

ReportCGI-EUTRA ::=                         SEQUENCE {

cellForWhichToReportCGI         EUTRA-PhysCellId,

...,

[[

useAutonomousGaps-r16           ENUMERATED {setup}                OPTIONAL     -- Need R

]]

}

ReportCGI ::=                     SEQUENCE {

cellForWhichToReportCGI          PhysCellId,

...,

[[

useAutonomousGaps-r16            ENUMERATED {setup}                OPTIONAL   -- Need R

]]

But in Ts 38306, a number of neighbor cells “#cell for CGI reporting” is use for CGI reporting for neighbor cell. This description is not aligned with TS3x.331 CGI reporting configuration procedure. This contribution discusses this disparity between UE capability constraint description on CGI reporting.

# Discussion

The UE capability constraints for CGI reporting is described in TS 38306 as highlighted below:

| Parameter | Description | Value |
| --- | --- | --- |
| #DRBs | The number of DRBs that a UE shall support. | 16 per UE. NOTE: 8 per MAC entity with duplication. |
| #minCellperMeasObjectNR | The minimum number of neighbour cells (excluding black list cells) that a UE shall be able to store associated with a MeasObjectNR. | 32 |
| #minBlackCellRangesperMeasObjectNR | The minimum number of blacklist cell PCI ranges that a UE shall be able to store associated with a MeasObjectNR. | 8 |
| #minCellperMeasObjectEUTRA | The minimum number of neighbour cells that a UE shall be able to store associated with a MeasObjectEUTRA. | 32 |
| #minCellTotal | The minimum number of neighbour cells (excluding black list cells) that UE shall be able to store in total from all measurement objects configured. | 256 with counting CSI-RS and SSB as 2. |
| #cell for CGI reporting | the limit regarding the cells NR can configure includes the cell for which the UE is requested to report CGI. | (# minCellperMeasObjectRAT - 1), where RAT represents NR and EUTRA. |
| #maxDeprioritisationFreq | The UE shall be able to store a depriotisation request for up to 8 frequencies (applicable when receiving another frequency specific deprioritisation request via RRCRelease before T325 expiry). | 8 |

This description is confusing as it may give the impression that the” cell for CGI reporting” can be more than one at once. Further this description does not express that the configuration is per frequency. In contrast, the UE capability constraints for CGI reporting is described in TS 36331 as highlighted below:

| Parameter | Description | Value | NB-IoT |
| --- | --- | --- | --- |
| #DRBs | The number of DRBs that a UE shall support | 8, 15 | (0, 1, 2) NOTE1 |
| #RLC-AM | The number of RLC AM entities that a UE shall support | 10, 17 | (2, 3) NOTE1 |
| #minCellperMeasObjectEUTRA | The minimum number of neighbour cells (excluding black list cells) that a UE shall be able to store within a MeasObjectEUTRA. NOTE. | 32 | N/A |
| #minBlackCellRangesperMeasObjectEUTRA | The minimum number of blacklist cell PCI ranges that a UE shall be able to store within a MeasObjectEUTRA | 32 | N/A |
| #minCellperMeasObjectUTRA | The minimum number of neighbour cells that a UE shall be able to store within a MeasObjectUTRA. NOTE. | 32 | N/A |
| #minCellperMeasObjectGERAN | The minimum number of neighbour cells that a UE shall be able to store within a measObjectGERAN. NOTE. | 32 | N/A |
| #minCellperMeasObjectCDMA2000 | The minimum number of neighbour cells that a UE shall be able to store within a measObjectCDMA2000. NOTE. | 32 | N/A |
| #minCellTotal | The minimum number of neighbour cells (excluding black list cells) that UE shall be able to store in total in all measurement objects configured | 256 | N/A |
| NOTE: In case of CGI reporting, the limit regarding the cells E-UTRAN can configure includes the cell for which the UE is requested to report CGI i.e. the amount of neighbour cells that can be included is at most (# minCellperMeasObjectRAT - 1), where RAT represents EUTRA/UTRA/GERAN/CDMA2000 respectively. |  |  |  |
| NOTE1: #DRBs based on UE capability, #RLC-AM =#DRBs + 2. |  |  |  |

This description clearly highlights that there is only one cellForWhichToReportCGI in the MeasObject.

Proposal: RAN2 to align UE capability Constraint for CGI reporting in TS38306 with the corresponding description in TS36331 with CRs [1] and [2].

# Conclusion

This contribution discusses misalignment between UE Capability Constraints description in TS38306 and TS36331 and proposes :

Proposal: RAN2 to align UE capability Constraint for CGI reporting in TS38306 with the corresponding description in TS36331 with CRs [1] and [2].

# Reference

[1] 38306_CR0324_(Rel-16)_R2-2004995_on UE capability constraints, vivo

[1] 38306_CR0325_(Rel-15)_R2-2004996_on UE capability constraints, vivo