# 3GPP TSG-RAN WG2 Meeting #110 electronic											R2-2005005

# 1st – 12th June, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.306 | CR | 0326 | rev | - | Current version: | 15.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on minimum UE requirements for the number of RLC bearers |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-06-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - In TS 38.306, the minimum UE requirements for #DRBs (i.e. 16) is specified in section 8, and there is a NOTE NOTE: 8 per MAC entity with duplication. However, as a DRB is not associated with a MAC entity, it is not crystal clear of the meaning of “8 per MAC entity”. According to the previous discussion, the intention of this NOTE should be that the maximum number of DRBs configured with PDCP duplication and with RLC entity(ies) associated with a MAC entity is 8. - The minimum UE requirement for the number of RLC bearers is not clear in specification and it is better to clarify. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify the NOTE of "8 per MAC entity with duplication" to be that “the maximum number of DRBs configured with PDCP duplication and with RLC entity(ies) associated with a MAC entity is 8”; - Clarify the minimum UE requirement for the number of RLC bearers to be: - 16 RLC bearers per UE, if not supporting duplication (i.e. pdcp-DuplicationMCG-OrSCG-DRB or pdcp-DuplicationSplitDRB) or split bearers (i.e. splitDRB-withUL-Both-MCG-SCG); - 16 RLC bearers per cell group, if supporting split bearers (i.e. splitDRB-withUL-Both-MCG-SCG or pdcp-DuplicationSplitDRB) but does not support CA duplication (i.e. pdcp-DuplicationMCG-OrSCG-DRB); - 24 RLC bearers per cell group, if supporting CA duplication (i.e. pdcp-DuplicationMCG-OrSCG-DRB). Impact Analysis Impacted 5G architecture options: Standalone, (NG)EN-DC, NE-DC, NR-DC Impacted functionality: Minumum UE requirement Inter-operability: 1. If the network is implemented according to the CR and the UE is not, there is a potential inter-operability problem, since the network and the UE may have different understanding on the minimum UE requirement regarding the number of RLC bearers. 2. If the UE is implemented according to the CR and the network is not, there is a potential inter-operability problem, since the network and the UE may have different understanding on the minimum UE requirement regarding the number of RLC bearers. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The network and the UE may have different understanding on the minimum UE requirement regarding the number of RLC bearers, and may therefore cause RRC reconfiguration failure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

---------------------------------------------START OF CHANGE-------------------------------------------

# 8	UE Capability Constraints

The following table lists constraints indicating the UE capabilities that the UE shall support.

| Parameter | Description |  | Value |  |
| --- | --- | --- | --- | --- |
| #DRBs | The number of DRBs that a UE shall support. |  | 16 per UE. NOTE1, NOTE2 |  |
| #RLC bearers for DRBs | The number of RLC bearers for DRBs that a UE shall support. | 16 per UE (NOTE4, NOTE3). 16 per cell group (NOTE5, NOTE3). 24 per cell group (NOTE6, NOTE3). |  |  |
| #minCellperMeasObjectNR | The minimum number of neighbour cells (excluding black list cells) that a UE shall be able to store associated with a MeasObjectNR. | 32 |  |  |
| #minBlackCellRangesperMeasObjectNR | The minimum number of blacklist cell PCI ranges that a UE shall be able to store associated with a MeasObjectNR. | 8 |  |  |
| #minCellperMeasObjectEUTRA | The minimum number of neighbour cells that a UE shall be able to store associated with a MeasObjectEUTRA. | 32 |  |  |
| #minCellTotal | The minimum number of neighbour cells (excluding black list cells) that UE shall be able to store in total from all measurement objects configured. | 256 with counting CSI-RS and SSB as 2. |  |  |
| #cell for CGI reporting | the limit regarding the cells NR can configure includes the cell for which the UE is requested to report CGI. | (# minCellperMeasObjectRAT - 1), where RAT represents NR and EUTRA. |  |  |
| #maxDeprioritisationFreq | The UE shall be able to store a depriotisation request for up to 8 frequencies (applicable when receiving another frequency specific deprioritisation request via RRCRelease before T325 expiry). | 8 |  |  |
| NOTE 1: The maximum number of DRBs configured with PDCP duplication and with RLC entity(ies) associated with a MAC entity is 8. |  |  |  |
| NOTE 2: This requirement is applicable in NR SA, NR-DC and NE-DC. |  |  |  |
| NOTE 3: This requirement is applicable in NR SA, (NG)EN-DC, NR-DC and NE-DC. |  |  |
| NOTE 4: This requirement is applicable for the UE not supporting duplication (i.e. pdcp-DuplicationMCG-OrSCG-DRB or pdcp-DuplicationSplitDRB) or splitDRB-withUL-Both-MCG-SCG. |  |  |
| NOTE 5: This requirement is applicable for the UE supporting splitDRB-withUL-Both-MCG-SCG or pdcp-DuplicationSplitDRB, but not supporting pdcp-DuplicationMCG-OrSCG-DRB. |  |  |
| NOTE 6: This requirement is applicable for the UE supporting pdcp-DuplicationMCG-OrSCG-DRB. |  |  |

---------------------------------------------END OF CHANGE---------------------------------------------