3GPP TSG-RAN WG2 Meeting #1010-e	R2-2005085

Online, 1st  - 12th June 2020

Agenda Item:	7.1.5

Source:			Huawei, HiSilicon

Title:	RAN1 feature list and UE capabilities issues for eMTC

Document for:	Discussion and Decision

# 1	Introduction

This document discusses remaining eMTC Capabilities opens issues:

- H817 : WI Open issue: TS 36.306: Editor’s note: In RRC the 4 PUR capabilities are part of MAC parameters for eMTC, but are part of general parameters for NB-IoT. Need to align one way or another.

- H820/H852: WI Open issue: TS 36.306: Editor's note: Field names need to be aligned across TS 36.331 and TS 36.306.

This document also discusses the impact due to a plethora of new capabilities in the RAN1 features list [6]

# 2	Discussion

## RAN2 open issues

### PUR capabilities

There is an Editor’s note in the TS 36.306 correction CRs [2], [3] as follows:

Editor's note: In RRC the 4 PUR capabilities are part of MAC parameters for eMTC, but are part of general parameters for NB-IoT. Need to align one way or another.

Transmission using PUR is clearly not a MAC feature but a general feature. Thus, if alignment is needed, the PUR capabilities should be moved to general capabilities in the eMTC correction CR [4].

Proposal 1: Move the four PUR capabilities to general capabilities in the eMTC RRC correction CR.

### Group Wake Up Signal Capabilities

There is an Editor’s note in the TS 36.306 eMTC correction CR [2] as follows:

4.3.4.xa	groupWakeUpSignal-r16

This field indicates whether the UE supports Group WUS for FDD as specified in TS 36.211 [17], TS 36.213 [22] and TS 36.304 [14]. This feature is only applicable if the UE supports ce-ModeA-r13 or if the UE supports any ue-Category-NB.

Editor's note: Field names need to be aligned across TS 36.331 and TS 36.306.

It was also discussed during the TS 36.306 correction CRs [2], [3] review that it would be preferable that the capabilities names are aligned between NB-IoT and eMTC.

Proposal 2: Change the group Wake Up Signal capabilities names in the eMTC correction CR so the names align with NB-IoT and Rel-15 capabilities names.

### Other Capabilities

The naming of the capabilities specific to CE mode A/B does not align between Rel-15 and Rel-16.

PhyLayerParameters-v1530 ::=           SEQUENCE {

...                                                            OPTIONAL,

ce-Capabilities-r15                SEQUENCE {

ce-CRS-IntfMitig-r15                   ENUMERATED {supported}          OPTIONAL,

ce-CQI-AlternativeTable-r15            ENUMERATED {supported}          OPTIONAL,

ce-PDSCH-FlexibleStartPRB-CE-ModeA-r15    ENUMERATED {supported}          OPTIONAL,

ce-PDSCH-FlexibleStartPRB-CE-ModeB-r15    ENUMERATED {supported}          OPTIONAL,

ce-PDSCH-64QAM-r15                     ENUMERATED {supported}          OPTIONAL,

ce-PUSCH-FlexibleStartPRB-CE-ModeA-r15    ENUMERATED {supported}          OPTIONAL,

ce-PUSCH-FlexibleStartPRB-CE-ModeB-r15    ENUMERATED {supported}          OPTIONAL,

ce-PUSCH-SubPRB-Allocation-r15         ENUMERATED {supported}          OPTIONAL,

ce-UL-HARQ-ACK-Feedback-r15            ENUMERATED {supported}          OPTIONAL

}   OPTIONAL,

shortCQI-ForSCellActivation-r15        ENUMERATED {supported}         OPTIONAL,

…

}

PhyLayerParameters-v16xy ::=           SEQUENCE {

ce-Capabilities-v16xy   SEQUENCE {

ce-CRS-ChannelEstMPDCCH-r16        ENUMERATED {supported}         OPTIONAL,

ce-ModeA-CSI-RS-Feedback-r16       ENUMERATED {supported}         OPTIONAL,

ce-ModeA-PDSCH-MultiTB-r16         ENUMERATED {supported}         OPTIONAL,

ce-ModeA-PUSCH-MultiTB-r16         ENUMERATED {supported}         OPTIONAL,

ce-ModeB-PDSCH-MultiTB-r16         ENUMERATED {supported}         OPTIONAL,

ce-ModeB-PUSCH-MultiTB-r16         ENUMERATED {supported}         OPTIONAL,

ce-ModeA-ETWS-CMAS-RxInConn-r16    ENUMERATED {supported}         OPTIONAL,

ce-ModeB-ETWS-CMAS-RxInConn-r16    ENUMERATED {supported}         OPTIONAL,

ce-RxInLTE-ControlRegion-r16       ENUMERATED {supported}         OPTIONAL,

dl-ChannelQualityReporting-r16     ENUMERATED {supported}         OPTIONAL

}   OPTIONAL,

widebandPRG-Slot-r16               ENUMERATED {supported}         OPTIONAL,

widebandPRG-Subslot-r16            ENUMERATED {supported}         OPTIONAL,

widebandPRG-Subframe-r16           ENUMERATED {supported}         OPTIONAL

}

Proposal 3: Align the naming of ce mode A/B specific capabilities to align to those in Rel-15 (e.g. similar to ce-PDSCH-FlexibleStartPRB-CE-ModeA-r15)

## RAN1 features list

RAN1 has sent a LS with an updated Rel-16 RAN1 UE features lists for LTE [6].

Compared with the capabilities currently captured in RAN2, there are a number of differences:

### Group Wake Up signal

RAN1 has agreed two separate optional sub-features with capability signalling for eMTC. Similar agreements were made for NB-IoT.

| Index | Feature group | Components | Prerequisite feature groups | Need of FDD/TDD differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- |
| 1-1 | Group WUS without group resource alternation | 1. Group WUS without group resource alternation | Rel-15 MWUS | Yes |  | Optional with capability signalling |
| 1-2 | Group WUS with group resource alternation | 1. Group WUS with group resource alternation | 1-1 | Yes |  | Optional with capability signalling |

For WUS we provide common NB-IoT and eMTC proposals in [8].

### PUR

RAN1 agreed on 8 PUR related capabitlies for eMTC

| Index | Feature group | Components | Prerequisite feature groups | Need of FDD/TDD differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- |
| 1-3 | PUR for full-PRB in CEmodeA | 1. PUR for full-PRB in CEmodeA | CEmodeA | Yes | RAN2 has agreed that PUR with UP and CP solutions have separate indications, but this is not captured in this RAN1 UE feature list. | Optional with capability signalling |
| 1-3a | Combination of PUR for full-PRB in CEmodeA with max UL TBS 2984 bits | 1. Combination of PUR for full-PRB in CEmodeA with max UL TBS 2984 bits | 1-3, and ce-PUSCH-NB-MaxTBS | Yes |  | Optional with capability signalling |
| 1-4 | PUR for full-PRB in CEmodeB | 1. PUR for full-PRB in CEmodeB 2. Potential UE-specific cyclic shift for DMRS | 1-3, and CEmodeB | Yes | RAN2 has agreed that PUR with UP and CP solutions have separate indications, but this is not captured in this RAN1 UE feature list. | Optional with capability signalling |
| 1-5 | PUR for sub-PRB in CEmodeA | 1. PUR for sub-PRB in CEmodeA | 1-3, and UL sub-PRB | Yes | RAN2 has agreed that PUR with UP and CP solutions have separate indications, but this is not captured in this RAN1 UE feature list. | Optional with capability signalling |
| 1-6 | PUR for sub-PRB in CEmodeB | 1. PUR for sub-PRB in CEmodeB | 1-4, and UL sub-PRB | Yes | RAN2 has agreed that PUR with UP and CP solutions have separate indications, but this is not captured in this RAN1 UE feature list. | Optional with capability signalling |
| 1-7 | PUR serving cell RSRP TA validation | 1. Serving cell RSRP for TA validation for PUR | 1-3 | Yes | TA validation mechanisms based on ‘Serving cell changes’ and ‘TA timer for idle mode’ (and ‘TA always valid’) are mandatory for PUR UEs. | Optional with capability signalling |
| 1-8 | PUR frequency hopping | 1. Frequency hopping for PUR | 1-3 | Yes |  | Optional with capability signalling |
| 1-9 | PUR L1 ACK | 1. L1 ACK for PUR | 1-3 | Yes |  | Optional with capability signalling |

Of the 8 capabilities, only the first has been captured, and differentiated for CP/UP and EPC/5GS.

The following proposals related to NB-IoT/eMTC common capabilities (highlighted in blue above) are discussed in [8], we propose not to duplicate the discussion here but include here for completeness.

Proposal 6-1: For NB-IoT and eMTC, introduce a new general capability pur-RSRP-Validation-r16, conditional to support of pur-CP-EPC-r16 and/or pur-CP-5GC-r16 and/or pur-UP-EPC-r16 and/or pur-CP-EPC-r16. Proposal 6-2: For NB-IoT and eMTC, introduce a new physical layer capability pur-CP-L1Ack-r16, conditional to support of pur-CP-EPC-r16 and/or pur-CP-5GC-r16.

The remaining capabilities highlighted in yellow are pure AS mechansisms, and there is no need for EPC/5GC differentiation. There is also no need for UP/CP differentiation.

Proposal 4-1: For eMTC, introduce a new general capability pur-PUSCH-NB-MaxTBS-r16, conditional to support of (pur-CP-EPC-r16 and/or pur-CP-5GC-r16 and/or pur-UP-EPC-r16 and/or pur-CP-EPC-r16) and ce-PUSCH-NB-MaxTBS.

Proposal 4-2: For eMTC, introduce a new general capability pur-CE-ModeB-r16, conditional to support of pur-CP-EPC-r16 and/or pur-CP-5GC-r16 and/or pur-UP-EPC-r16 and/or pur-CP-EPC-r16.

Proposal 4-3: For eMTC, introduce a new physical layer capability pur-Sub-PRB-CE-ModeA-r16, conditional to support of (pur-CP-EPC-r16 and/or pur-CP-5GC-r16 and/or pur-UP-EPC-r16 and/or pur-CP-EPC-r16) and ce-PUSCH-SubPRB-Allocation-r15.

Proposal 4-4: For eMTC, introduce a new physical layer capability pur-Sub-PRB-CE-ModeB-r16, conditional to support of (pur-CP-EPC-r16 and/or pur-CP-5GC-r16 and/or pur-UP-EPC-r16 and/or pur-CP-EPC-r16) and pur-CE-ModeB-r16 and ce-PUSCH-SubPRB-Allocation-r15.

Proposal 4-5: For eMTC, introduce a new physical layer capability pur-FrequencyHopping-r16, conditional to support of pur-CP-EPC-r16 and/or pur-CP-5GC-r16 and/or pur-UP-EPC-r16 and/or pur-CP-EPC-r16.

### MultiTB scheduling

RAN1 has agreed to 12 optional sub-features with capability signalling:

| Index | Feature group | Components | Prerequisite feature groups | Need of FDD/TDD differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- |
| 1-10 | Multi-TB unicast for DL in CEmodeA | 1. Multi-TB unicast scheduling for DL in CEmodeA | CEmodeA | Yes | Following legacy capabilities are reused to support combinations. o Rel-14 feature for 2984 bits max UL TBS in 1.4 MHz in CE mode A o Rel-14 feature for new numbers of repetitions for PUSCH in CE mode A o Rel-14 feature for modulation restrictions for PDSCH/PUSCH in CE mode A o Rel-15 features for flexible starting PRB for PDSCH/PUSCH in CE mode A/B | Optional with capability signalling |
| 1-11 | Multi-TB unicast for DL in CEmodeB | 1. Multi-TB unicast scheduling for DL in CEmodeB | CEmodeB | Yes |  | Optional with capability signalling |
| 1-12 | Multi-TB unicast for UL in CEmodeA | 1. Multi-TB unicast scheduling for UL in CEmodeA | CEmodeA | Yes |  | Optional with capability signalling |
| 1-13 | Multi-TB unicast for UL in CEmodeB | 1. Multi-TB unicast scheduling for UL in CEmodeB | CEmodeB | Yes |  | Optional with capability signalling |
| 1-14 | Multi-TB unicast TB interleaving | 1. TB interleaving for multi-TB unicast scheduling | 1-10 or 1-11 or 1-12 or 1-13 | Yes |  | Optional with capability signalling |
| 1-15 | Multi-TB unicast HARQ bundling | 1. DL HARQ bundling for multi-TB unicast scheduling | 1-10 | Yes |  | Optional with capability signalling |
| 1-16 | Multi-TB unicast UL sub-PRB | 1. UL sub-PRB allocation for multi-TB unicast scheduling | 1-12 or 1-13, and UL sub-PRB | Yes |  | Optional with capability signalling |
| 1-17 | Multi-TB unicast UL early termination | 1. UL early termination for multi-TB unicast scheduling | 1-12 [and 1-25], or 1-13 [and 1-26] | Yes |  | Optional with capability signalling |
| 1-18 | Multi-TB unicast DL 64QAM for CE mode A | 1. DL 64QAM for multi-TB unicast scheduling for CE mode A | 1-10, and DL 64QAM | Yes |  | Optional with capability signalling |
| 1-19 | Multi-TB unicast frequency hopping | 1. Frequency hopping for multi-TB unicast scheduling | 1-10 or 1-11 or 1-12 or 1-13 | Yes |  | Optional with capability signalling |
| 1-21 | Multi-TB SC-MTCH in CEmodeA | 1. Multi-TB SC-MTCH scheduling in CEmodeA 2. Potential scheduling gaps for multi-TB SC-MTCH scheduling in CEmodeA | CEmodeA, and SC-PTM | Yes | The basic multicast (SC-PTM) functionality was introduced for LTE-M/NB-IoT in Rel-14 without capability signaling. | Up to RAN2 |
| 1-22 | Multi-TB SC-MTCH in CEmodeB | 1. Multi-TB SC-MTCH scheduling in CEmodeB 2. Potential scheduling gaps for multi-TB SC-MTCH scheduling in CEmodeB | CEmodeB, and SC-PTM | Yes | The basic multicast (SC-PTM) functionality was introduced for LTE-M/NB-IoT in Rel-14 without capability signaling. | Up to RAN2 |

The yellow highlighted capabitlies have not yet been capture in RAN2 specifications, and are pure AS mechansisms so there is no need for EPC/5GC differentiation.

Proposal 5-1: For eMTC, introduce a new physical layer capability ce-MultiTB-Interleaving-r16, conditional to support of ce-ModeA-PUSCH-MultiTB-r16 and/or ce-ModeB-PUSCH-MultiTB-r16 and/or ce-ModeA-PDSCH-MultiTB-r16 and/or ce-ModeB-PDSCH-MultiTB-r16.

Proposal 5-2: For eMTC, introduce a new physical layer capability ce-MultiTB-HARQ-Bundling-r16, conditional to support of ce-ModeA-PDSCH-MultiTB-r16.

Proposal 5-3: For eMTC, introduce a new physical layer capability ce-MultiTB-Sub-PRB-r16, conditional to support of (ce-ModeA-PUSCH-MultiTB-r16 and/or ce-ModeB-PUSCH-MultiTB-r16) and ce-PUSCH-SubPRB-Allocation-r15.

Proposal 5-4: For eMTC, introduce a new physical layer capability ce-MultiTB-EarlyTermination-r16, conditional to support of ce-ModeA-PUSCH-MultiTB-r16 and/or ce-ModeB-PUSCH-MultiTB-r16.

Proposal 5-5: For eMTC, introduce a new physical layer capability ce-MultiTB-64QAM-r16, conditional to support of ce-ModeA-PDSCH-MultiTB-r16 and ce-PDSCH-64QAM-r15.

Proposal 5-6: For eMTC, introduce a new physical layer capability ce-MultiTB-FrequencyHopping-r16, conditional to support of ce-ModeA-PUSCH-MultiTB-r16 and/or ce-ModeB-PUSCH-MultiTB-r16 and/or ce-ModeA-PDSCH-MultiTB-r16 and/or ce-ModeB-PDSCH-MultiTB-r16.

In [2] one capability is already introduced for Multi-TB SC-MTCH in CE-modeA, a second capability for CE Mode B is needed.

Proposal 5-7: For eMTC, introduce a new capability without radio access capability signaling for Multi-TB SC-MTCH in CE-modeB.

### Resource reservation for NR

RAN1 has defined separate sub-features for subframe level and slot level resource reservation granularity.

| Index | Feature group | Components | Prerequisite feature groups | Need of FDD/TDD differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- |
| 1-23 | Subframe level resource reservation for DL in CEmodeA | 1. Subframe-level time-domain resource reservation in DL in CEmodeA 2. RBG-level frequency-domain resource reservation in DL in CEmodeA | CEmodeA | Yes |  | Optional with capability signalling |
| 1-23a | Slot/symbol level resource reservation for DL in CEmodeA | 1. Slot/symbol-level time-domain resource reservation in DL in CEmodeA 2. RBG-level frequency-domain resource reservation in DL in CEmodeA | 1-23 | Yes |  | Optional with capability signalling |
| 1-24 | Subframe level resource reservation for DL in CEmodeB | 1. Subframe-level time-domain resource reservation in DL in CEmodeB 2. RBG-level frequency-domain resource reservation in DL in CEmodeB | CEmodeB | Yes |  | Optional with capability signalling |
| 1-24a | Slot/symbol level resource reservation for DL in CEmodeB | 1. Slot/symbol-level time-domain resource reservation in DL in CEmodeB 2. RBG-level frequency-domain resource reservation in DL in CEmodeB | 1-24 | Yes |  | Optional with capability signalling |
| 1-25 | Subframe level resource reservation for UL in CEmodeA | 1. Subframe-level time-domain resource reservation in UL in CEmodeA | CEmodeA | Yes |  | Optional with capability signalling |
| 1-25a | Slot/symbol-level resource reservation for UL in CEmodeA | 1. Slot/symbol-level time-domain resource reservation in UL in CEmodeA | 1-25 | Yes |  | Optional with capability signalling |
| 1-26 | Subframe level resource reservation for UL in CEmodeB | 1. Subframe-level time-domain resource reservation in UL in CEmodeB | CEmodeB | Yes |  | Optional with capability signalling |
| 1-26a | Slot/symbol-level resource reservation for UL in CEmodeB | 1. Slot/symbol-level time-domain resource reservation in UL in CEmodeB | 1-26 | Yes |  | Optional with capability signalling |
| 1-27 | Subcarrier puncturing for DL in CEmodeA | 1. Subcarrier puncturing for DL in CEmodeA | CEmodeA | Yes |  | Optional with capability signalling |
| 1-28 | Subcarrier puncturing for DL in CEmodeB | 1. Subcarrier puncturing for DL in CEmodeB | CEmodeB | Yes |  | Optional with capability signalling |

The CR in [2] captures 4 capabilities for resource reservation, but the RAN1 agreed capabilities splits these into 8 (separate for subframe level and for slot/symbol level). The CR in [2] already captures the 2 capabilities for subcarrier puncturing.

Proposal 6-1: Rename the four already defined capabilities to ce-SubframeResourceResvUL-CE-ModeA-r16, ce-SubframeResourceResvDL-CE-ModeA-r16, ce-SubframeResourceResvDL-CE-ModeA-r16, ce-SubframeResourceResvDL-CE-ModeB-r16

Proposal 6-2: Introduce four new physical layer capabilities ce-SlotSymbolResourceResvUL-CE-ModeA-r16, ce-SlotSymbolResourceResvUL-CE-ModeB-r16, ce-SlotSymbolResourceResvDL-CE-ModeA-r16, ce-SlotSymbolResourceResvDL-CE-ModeB-r16 to support of slot/symbol level granularity.

### DL quality report

| Index | Feature group | Components | Prerequisite feature groups | Need of FDD/TDD differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- |
| 1-29 | DL quality report in Msg3 in Idle | 1. Using 2 bits in Msg3 in Idle 2. Using 4 bits in Msg3 in Idle | CEmodeA | Yes | It is up to RAN2 whether to have separate capabilities for CE mode A and B. | Up to RAN2 |
| 1-30 | DL quality report in Connected | 1. DL quality report using 4 bits in Connected | CEmodeA | Yes | It is up to RAN2 whether to have separate capabilities for CE mode A and B. | Up to RAN2 |

The above capabilities have already been captured.

### MPDCCH performance improvement

| Index | Feature group | Components | Prerequisite feature groups | Need of FDD/TDD differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- |
| 1-31 | MPDCCH performance improvement with precoder cycling in CEmodeA | 1. MPDCCH performance improvement with precoder cycling in CEmodeA | CEmodeA | Yes |  | Optional with capability signalling |
| 1-32 | MPDCCH performance improvement with precoder cycling in CEmodeB | 1. MPDCCH performance improvement with precoder cycling in CEmodeB | CEmodeB | Yes |  | Optional with capability signalling |
| 1-33 | MPDCCH performance improvement with CSI-based mapping for CE mode A | 1. MPDCCH performance improvement with CSI-based mapping for CE mode A | 1-31 | Yes |  | Optional with capability signalling |
| 1-34 | MPDCCH performance improvement with reciprocity-based candidates in TDD | 1. MPDCCH performance improvement with reciprocity-based candidates in TDD | 1-31 or 1-32 | TDD only | FFS: whether it can apply to CE mode B | Optional with capability signalling |

Currently there is only one capability captured, ce-CRS-ChannelEstMPDCCH-r16

Proposal 7-1: Rename existing capability to ce-CRS-ChannelEstMPDCCH-CE-ModeA-r16

Proposal 7-2: Introduce a new physical layer capability ce-CRS-ChannelEstMPDCCH-CE-ModeB-r16

Proposal 7-3: Introduce a new physical layer capability ce-CRS-ChannelEstMPDCCH-CSI-CE-ModeB-r16 conditional to support of ce-CRS-ChannelEstMPDCCH-CE-ModeA-r16

Proposal 7-4: Introduce a new physical layer capability ce-CRS-ChannelEstMPDCCH-reciprocity-TDD-r16 conditional to support of ce-CRS-ChannelEstMPDCCH-CE-ModeA-r16 and/or ce-CRS-ChannelEstMPDCCH-CE-ModeB-r16

### CSI-RS-Feedback

| Index | Feature group | Components | Prerequisite feature groups | Need of FDD/TDD differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- |
| 1-35 | CSI-RS-based feedback for non-BL UE | 1. CSI-RS-based feedback for non-BL UE in CEmodeA | tm9-CE-ModeA-r13 | Yes |  | Optional with capability signalling |
| 1-35a | Codebook subset restriction for CSI-RS-based feedback for non-BL UE in CEmodeA | 1. Codebook subset restriction for CSI-RS-based feedback for non-BL UE in CEmodeA | 1-35 | Yes |  | Optional with capability signalling |

Currently only a single capability is captured, ce-ModeA-CSI-RS-Feedback-r16

Proposal 8-1: Introduce a new physical layer capability ce-ModeA-CodebookRestriction-CSI-RS-Feedback-r16 conditional to support of ce-ModeA-CSI-RS-Feedback-r16

### ETWS/CMAS indication

| Index | Feature group | Components | Prerequisite feature groups | Need of FDD/TDD differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- |
| 1-36 | ETWS/CMAS indication in connected mode for non-BL UE in CEmodeA | 1. ETWS/CMAS indication in connected mode for non-BL UE in CEmodeA | CEmodeA | Yes |  | Optional with capability signalling |
| 1-37 | ETWS/CMAS indication in connected mode for non-BL UE in CEmodeB | 1. ETWS/CMAS indication in connected mode for non-BL UE in CEmodeB | CEmodeB | Yes |  | Optional with capability signalling |

The above capabilities are already captured in [2].

### LTE Control Channel use

| Index | Feature group | Components | Prerequisite feature groups | Need of FDD/TDD differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- |
| 1-38 | LTE control region use for MPDCCH in CEmodeA | 1. LTE control region use for MPDCCH in CEmodeA | CEmodeA | Yes |  | Optional with capability signalling |
| 1-39 | LTE control region use for MPDCCH in CEmodeB | 1. LTE control region use for MPDCCH in CEmodeB | CEmodeB | Yes |  | Optional with capability signalling |
| 1-40 | LTE control region use for PDSCH in CEmodeA | 1. LTE control region use for PDSCH in CEmodeA | CEmodeA | Yes |  | Optional with capability signalling |
| 1-41 | LTE control region use for PDSCH in CEmodeB | 1. LTE control region use for PDSCH in CEmodeB | CEmodeB | Yes |  | Optional with capability signalling |

Currently only a single capability is defined, covering all 4 of the above in ce-RxInLTE-ControlRegion-r16

Proposal 9-1: Rename existing capability to ce-MPDCCH-RxInLTE-ControlRegion-CE-ModeA-r16

Proposal 9-2: Introduce 3 new capabilities ce-MPDCCH-RxInLTE-ControlRegion-CE-ModeB-r16, ce-MPDSCH-RxInLTE-ControlRegion-CE-ModeA-r16, ce-MPDSCH-RxInLTE-ControlRegion-CE-ModeB-r16

### Other

Currently none of the physical layer capabilities are differentiated for TDD/FDD in the latest RRC specification or in the RRC Release 16 CR in [4]. According to [6] all of them should be.

Proposal 10-1: Introduce UE-EUTRA-CapabilityAddXDD-Mode container for all of the newly introduced Release-16 physical layer capabilities.

In addition we noticed that this is missing from Rel-15 too – there is currently no physical layer capability differentiation between TDD and FDD even though the Rel-15 RAN1 feature list [9] indicated that it is needed for most physical capabilities.

Proposal 10-2: Correct Rel-15 to introduce UE-EUTRA-CapabilityAddXDD-Mode for physical layer capabilities for TDD/FDD differentiation.

A correction to Rel-15 is provided in [10].

# 3	Conclusion

In this document, we have discussed miscellaneous WI open issues and made the following proposals:

RAN2 open issues:

Proposal 1: Move the four PUR capabilities to general capabilities in the eMTC RRC correction CR.

Proposal 2: Change the group Wake Up Signal capabilities names in the eMTC correction CR so the names align with NB-IoT and Rel-15 capabilities names.

Proposal 3: Align the naming of ce mode A/B specific capabilities to align to those in Rel-15 (e.g. similar to ce-PDSCH-FlexibleStartPRB-CE-ModeA-r15)

RAN1 Feature List Impact:

PUR

Proposal 4-1: For eMTC, introduce a new general capability pur-PUSCH-NB-MaxTBS-r16, conditional to support of (pur-CP-EPC-r16 and/or pur-CP-5GC-r16 and/or pur-UP-EPC-r16 and/or pur-CP-EPC-r16) and ce-PUSCH-NB-MaxTBS.

Proposal 4-2: For eMTC, introduce a new general capability pur-CE-ModeB-r16, conditional to support of pur-CP-EPC-r16 and/or pur-CP-5GC-r16 and/or pur-UP-EPC-r16 and/or pur-CP-EPC-r16.

Proposal 4-3: For eMTC, introduce a new physical layer capability pur-Sub-PRB-CE-ModeA-r16, conditional to support of (pur-CP-EPC-r16 and/or pur-CP-5GC-r16 and/or pur-UP-EPC-r16 and/or pur-CP-EPC-r16) and ce-PUSCH-SubPRB-Allocation-r15.

Proposal 4-4: For eMTC, introduce a new physical layer capability pur-Sub-PRB-CE-ModeB-r16, conditional to support of (pur-CP-EPC-r16 and/or pur-CP-5GC-r16 and/or pur-UP-EPC-r16 and/or pur-CP-EPC-r16) and pur-CE-ModeB-r16 and ce-PUSCH-SubPRB-Allocation-r15.

Proposal 4-5: For eMTC, introduce a new physical layer capability pur-FrequencyHopping-r16, conditional to support of pur-CP-EPC-r16 and/or pur-CP-5GC-r16 and/or pur-UP-EPC-r16 and/or pur-CP-EPC-r16.

MultiTB scheduling

Proposal 5-1: For eMTC, introduce a new physical layer capability ce-MultiTB-Interleaving-r16, conditional to support of ce-ModeA-PUSCH-MultiTB-r16 and/or ce-ModeB-PUSCH-MultiTB-r16 and/or ce-ModeA-PDSCH-MultiTB-r16 and/or ce-ModeB-PDSCH-MultiTB-r16.

Proposal 5-2: For eMTC, introduce a new physical layer capability ce-MultiTB-HARQ-Bundling-r16, conditional to support of ce-ModeA-PDSCH-MultiTB-r16.

Proposal 5-3: For eMTC, introduce a new physical layer capability ce-MultiTB-Sub-PRB-r16, conditional to support of (ce-ModeA-PUSCH-MultiTB-r16 and/or ce-ModeB-PUSCH-MultiTB-r16) and ce-PUSCH-SubPRB-Allocation-r15.

Proposal 5-4: For eMTC, introduce a new physical layer capability ce-MultiTB-EarlyTermination-r16, conditional to support of ce-ModeA-PUSCH-MultiTB-r16 and/or ce-ModeB-PUSCH-MultiTB-r16.

Proposal 5-5: For eMTC, introduce a new physical layer capability ce-MultiTB-64QAM-r16, conditional to support of ce-ModeA-PDSCH-MultiTB-r16 and ce-PDSCH-64QAM-r15.

Proposal 5-6: For eMTC, introduce a new physical layer capability ce-MultiTB-FrequencyHopping-r16, conditional to support of ce-ModeA-PUSCH-MultiTB-r16 and/or ce-ModeB-PUSCH-MultiTB-r16 and/or ce-ModeA-PDSCH-MultiTB-r16 and/or ce-ModeB-PDSCH-MultiTB-r16.

Proposal 5-7: For eMTC, introduce a new capability without radio access capability signaling for Multi-TB SC-MTCH in CE-modeB.

Resource reservation for NR

Proposal 6-1: Rename the four already defined capabilities to ce-SubframeResourceResvUL-CE-ModeA-r16, ce-SubframeResourceResvDL-CE-ModeA-r16, ce-SubframeResourceResvDL-CE-ModeA-r16, ce-SubframeResourceResvDL-CE-ModeB-r16

Proposal 6-2: Introduce four new physical layer capabilities ce-SlotSymbolResourceResvUL-CE-ModeA-r16, ce-SlotSymbolResourceResvUL-CE-ModeB-r16, ce-SlotSymbolResourceResvDL-CE-ModeA-r16, ce-SlotSymbolResourceResvDL-CE-ModeB-r16 to support of slot/symbol level granularity.

MPDCCH Performance Improvement

Proposal 7-1: Rename existing capability to ce-CRS-ChannelEstMPDCCH-CE-ModeA-r16

Proposal 7-2: Introduce a new physical layer capability ce-CRS-ChannelEstMPDCCH-CE-ModeB-r16

Proposal 7-3: Introduce a new physical layer capability ce-CRS-ChannelEstMPDCCH-CSI-CE-ModeB-r16 conditional to support of ce-CRS-ChannelEstMPDCCH-CE-ModeA-r16

Proposal 7-4: Introduce a new physical layer capability ce-CRS-ChannelEstMPDCCH-reciprocity-TDD-r16 conditional to support of ce-CRS-ChannelEstMPDCCH-CE-ModeA-r16 and/or ce-CRS-ChannelEstMPDCCH-CE-ModeB-r16

CSI-RS Feedback

Proposal 8-1: Introduce a new physical layer capability ce-ModeA-CodebookRestriction-CSI-RS-Feedback-r16 conditional to support of ce-ModeA-CSI-RS-Feedback-r16

LTE Control Channel use

Proposal 9-1: Rename existing capability to ce-MPDCCH-RxInLTE-ControlRegion-CE-ModeA-r16

Proposal 9-2: Introduce 3 new capabilities ce-MPDCCH-RxInLTE-ControlRegion-CE-ModeB-r16, ce-MPDSCH-RxInLTE-ControlRegion-CE-ModeA-r16, ce-MPDSCH-RxInLTE-ControlRegion-CE-ModeB-r16

Other

Proposal 10-1: Introduce UE-EUTRA-CapabilityAddXDD-Mode container for all of the newly introduced Release-16 physical layer capabilities.

Proposal 10-2: Correct Rel-15 to introduce UE-EUTRA-CapabilityAddXDD-Mode for physical layer capabilities for TDD/FDD differentiation.

A correction to Rel-15 is provided in [10].

# 4	References

R2-2004058 “RAN2 agreements for Rel-16 additional enhancements for NB-IoT and MTC”, Blackberry, RAN2#109bis-e, Online, April 2020

R2-2003921 “Update of UE capabilities for eMTC” Huawei, RAN2#109bis-e, Online, April 2020

R2-2004044 “Updates for Rel-16 additional enhancements NB-IoT”, Blackberry, RAN2#109bis-e, Online, April 2020

R2-2003923 “Miscellaneous eMTC rel-16 corrections”, Qualcomm, RAN2#109bis-e, Online, April 2020

R2-2004040 “Miscellaneous corrections to 36.331 for Rel-16 NB-IoT”, Huawei, RAN2#109bis-e, Online, April 2020

R2-2004357 “LS on updated Rel-16 RAN1 UE features lists for LTE”, RAN1, RAN2#110-e, June 2020

R1-2003196 “Summary on email discussion [100b-e-LTE-UEFeatures-Remaining]”, RAN1, RAN1#100bis-e, April  2020

R2-2005030, “RAN1 feature list and UE capabilities issues for NB-IoT”, Huawei, RAN2#1010-e, Online, June 2020

R2-1813308 “LS on Rel-15 LTE UE feature list”, RAN1, RAN2#103, August 2018

R2-2005083, “Correction to the LTE Rel-15 TDD/FDD capability differentiation”, Huawei, RAN2#1010-e, Online, June 2020