3GPP RAN WG2 Meeting #110-e	R2-2005141
June 1st – 12th, 2020

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 0751 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of MPE reporting to TS 38.321 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR2_req_enh |  |  |  |  |  |  | Date: |  | 2020-05-21 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In FR2, there are requirements for maximum permitted exposure (MPE), whereby the UE measures its average energy level over a period. Currently the UEs may signal maxUplinkDutyCycle-FR2 as a capability. However, the gNB is not bounded by any rules to meet this limit, since the MPE limit can be exceeded at different power levels, being computed as an averaged power density over a certain period. The only method used today is P-MPR and this can be quite drastic, leading to possible loss of UL connection and thus followed by a Connection Release. In RAN4’s LS (R4-2005670), RAN4 is asking RAN2 to introduce MAC-CE to signal measured P-MPR MPE estimate when P-MPR is higher than a configurable threshold. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - A new single-entry MAC CE is introduced to report MPE-related P-MPR estimates over all active serving cells of the MAC entity. - A prohibit timer and event-based triggering procedure are specified in MAC for P-MPR MPE reporting |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MPE reporting functionality is not introduced |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.X, 6.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TS/TR 38.331 CR 1652 |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

# 5	MAC procedures

<Start of modification 1>

## 5.X	MPE Reporting

The MPE reporting procedure is maintained per MAC entity and includes the following information:

-	power reduction: P-MPR report for Uplink transmissions in a CG.

RRC controls MPE reporting by configuring the following parameters:

-	mpe-ProhibitTimer;

-	mpe-threshold;

An MPE report shall be triggered if the UE requires MPE management (as allowed by TS 38.101-1 [14], TS 38.101-2 [15], and TS 38.101-3 [16]) and any of the following events occur:

-	mpe-ProhibitTimer is not running and the measured P-MPR has exceeded mpe-threshold for at least one activated Serving Cell of any MAC entity since the last transmission of a MPE report in this MAC entity when the MAC entity has UL resources for new transmission;

-	upon configuration or reconfiguration of the MPE reporting functionality by upper layers, which is not used to disable the function;

If the MAC entity has UL resources allocated for a new transmission, the MAC entity shall:

1>	if the MPE reporting procedure determines that at least one report has been triggered and not cancelled; and

1>	if the allocated UL resources can accommodate the MPE reporting MAC CE which the MAC entity is configured to transmit, plus its subheader, as a result of LCP as defined in subclause 5.4.3.1:

2>	obtain the value of the MPE report from the physical layer for the corresponding cell group;

2>	obtain the value for the corresponding P-MPR,cg,f field from the physical layer;

2>	instruct the Multiplexing and Assembly procedure to generate and transmit the MPE reporting MAC CE as defined in subclause 6.1.3.x based on the values reported by the physical layer.

2>	start or restart mpe-ProhibitTimer;

2>	cancel the reported triggered MPE report;

<End of modification 1>

### 6.1.3	MAC Control Elements (CEs)

<Start of modification 2>

#### 6.1.3.X	MPE MAC CE

The MPE MAC CE is identified by a MAC PDU subheader with LCID as specified in Table 6.2.1-2.

It has a fixed size and consists of two octets defined as follows (figure 6.1.3.x-1):

-	P-MPR, cg: This field indicates the P-MPR, cg (as specified in TS 38.101). The reported P-MPR, cg values range are shown in Table 6.1.3.x-1 (the corresponding measured values in dBm are specified in TS 38.133).

-	Serving cell ID: the identity of the serving cell that triggered the MPE report. The serving cell ID corresponds to the ServCellIndex as specified in in TS 38.331).

Figure 6.1.3.x-1: MPE MAC CE

| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Serving cell ID |  |  |  |  | P-MPR, cg |  |  |

Table 6.1.3.x-1: Effective power reduction for P-MPR

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| P-MPR_00 | P-MPR = 0 | dBm |
| P-MPR_01 | PMP-R < 3 | dBm |
| P-MPR_02 | 3  PMP-R < 6 | dBm |
| P-MPR_03 | 6  PMP-R < 9 | dBm |
| P-MPR_04 | 9  PMP-R < 12 | dBm |
| P-MPR_05 | 12  PMP-R < 15 | dBm |
| P-MPR_06 | 15  PMP-R | dBm |

<end of modification 2>