3GPP RAN WG2 Meeting #110-e	R2-2005151
June 1st – 12th, 2020

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 1652 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of MPE reporting to TS 38.331 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR2_req_enh |  |  |  |  |  |  | Date: |  | 2020-05-21 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In FR2, there are requirements for maximum permitted exposure (MPE), whereby the UE measures its average energy level over a period. Currently the UEs may signal maxUplinkDutyCycle-FR2 as a capability. However, the gNB is not bounded by any rules to meet this limit, since the MPE limit can be exceeded at different power levels, being computed as an averaged power density over a certain period. The only method used today is P-MPR and this can be quite drastic, leading to possible loss of UL connection and thus followed by a Connection Release. In RAN4’s LS (R4-2005670), RAN4 is asking RAN2 to introduce MAC-CE to signal measured P-MPR MPE estimate when P-MPR is higher than a configurable threshold. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - IE MPE-Config is added to configure parameters for MPE reporting, including the MPE P-MPR thershold and the a prohibit timer for reporting. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MPE reporting functionality is not introduced |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.X, 6.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TS/TR 38.321 CR 0751 |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

# 5	MAC procedures

<Start of modification 1>

The IE MPE-Config is used to configure parameters for MPE reporting.

MPE-Config information element

-- ASN1START

-- TAG-MPE-CONFIG-START

MPE-Config ::=                      SEQUENCE {

mpe-ProhibitTimer                   ENUMERATED { sf10, sf20, sf50, sf100, sf200, sf400,

sf600, sf800, sf900, sf1000, sf1500, sf2000},

mpe-threshold            			ENUMERATED {0, 3, 6, 9},

...

}

-- TAG-MPE-CONFIG-STOP

-- ASN1STOP

MPE-Config field descriptions

mpe-ProhibitTimer Value in number of subframes for MPE reporting as specified in TS 38.321 [3]. Value sf100 corresponds to 100 subframes, and so on.

mpe-threshold Value of the P-MPR threshold in dB.

<End of modification 1>