3GPP TSG-RAN WG2 Meeting #110 Electronic	R2-2005308

# 1st – 12th June, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR | 4266 | rev | 2 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | upperLayerIndication enhancements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, BT, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core, TEI16 |  |  |  |  |  | Date: |  |  | 2020-05-21 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In RAN #86, a LS was received from GSMA in RP-193053 requesting further work from 3GPP to enhance the existing 5G indicator handling. RAN plenary requested RAN2 to work on this in RP-193265 and RAN concluded that the following changes to the E-UTRA RRC specification are required: - Introduce signalling to enable a UE camped on an E-UTRA cell to be informed, with frequency band granularity, of the NR frequency bands available for configuration of EN-DC operation within the area of this cell. In the case of RAN sharing, it must be possible to provide the NR frequency bands independently per PLMN. RAN2 can involve other groups as necessary to introduce the appropriate signalling. - For a UE in RRC Idle (or Inactive), specify that the presence of the upperLayerIndication is only provided to upper layers if the UE supports EN-DC operation for one or more of the indicated frequency bands. - For a UE in RRC Connected, specify that the presence or absence of the upperLayerIndication is provided to upper layers depending on whether or not the UE is configured by RRC for EN-DC operation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following changes are made: - Add SIBxy to broadcast NR band information for EN-DC operation - Modify UE’s action on forwarding upperLayerIndication to upper layers after receiving SIB2 - Add action description of RRC_CONNECTED UE to forward upperLayerIndication to upper layers Implementation of this CR from Rel-15 will not cause interoperability issues |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A UE that doesn’t support any NR band for EN-DC in one area will pass the upperLayerIndication to upper layers (in order to display 5G icon) as per legacy. This will make the user confused. The upperLayerIndication will not be passed to upper layers when the UE is configured by RRC for EN-DC operation |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.9, 5.2.2.xy, 5.3.5.3, 5.3.5.4, 6.2.2, 6.3.1, 6.4, Annex G |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 5.2.2.9	Actions upon reception of SystemInformationBlockType2

Upon receiving SystemInformationBlockType2, the UE shall:

<Partially omitted>

1>	else:

2>	indicate to upper layers that up-CIoT-EPS-Optimisation is not present;

- if SystemInformationBlockTypexy is not present:

- to upper layers either forward upperLayerIndication, if present for the selected PLMN, or otherwise indicate absence of this field;

NOTE:	upperLayerIndication is an indication to upper layers that the UE has entered a coverage area that offers 5G capabilities.

<Next modification>

#### 5.2.2.34	Actions upon reception of SystemInformationBlockPos

No UE requirements related to the contents of the SystemInformationBlockPos apply other than those specified elsewhere e.g. within TS 36.355 [54], and/or within the corresponding field descriptions.

#### 5.2.2.35	Actions upon reception of SystemInformationBlockType27

No UE requirements related to the contents of this SystemInformationBlock (SystemInformationBlockType27 or SystemInformationBlockType27-NB) apply other than those specified elsewhere e.g. within procedures using the concerned system information, and/ or within the corresponding field descriptions.

#### 5.2.2.36	Actions upon reception of SystemInformationBlockType28

Upon receiving SystemInformationBlockType28, the UE shall perform actions as specified in 5.2.2.4.x in TS 38.331 [82].

#### 5.2.2.xy	Actions upon reception of SystemInformationBlockTypexy

Upon receiving SystemInformationBlockTypexy the UE shall:

1>	if nrBandList is included for the selected PLMN and the UE supports to operate in EN-DC using the serving cell and at least one of NR bands in nrBandList:

2>	forward upperLayerIndication, as if the UE receives this field from SIB2, to upper layers;

1>	else:

2>	indicate upper layers absence of upperLayerIndication;

<Next modification>

#### 5.3.5.3	Reception of an RRCConnectionReconfiguration not including the mobilityControlInfo by the UE

If the RRCConnectionReconfiguration message does not include the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

<Partially omitted>

1>	set the content of RRCConnectionReconfigurationComplete message as follows:

2>	if the RRCConnectionReconfiguration message includes perCC-GapIndicationRequest:

3>	include perCC-GapIndicationList and numFreqEffective;

2>	if the frequencies are configured for reduced measurement performance:

3>	include numFreqEffectiveReduced;

2>	if the received RRCConnectionReconfiguration message included nr-SecondaryCellGroupConfig:

3>	include scg-ConfigResponseNR in accordance with TS 38.331 [82], clause 5.3.5.3;

2>	if the received RRCConnectionReconfiguration message was included in an NR RRCResume message:

3>	include the RRCConnectionReconfigurationComplete message in the NR MCG RRC message RRCResumeComplete in accordance with TS 38.331 [82], clause 5.3.13.4;

1> if the UE is configured to operate in EN-DC as result of this procedure, forward upperLayerIndication, as if the UE receives this field from SIB2, to upper layers, otherwise indicate upper layers absence of this field;

1>	if the UE is configured with NE-DC:

2>	transfer the RRCConnectionReconfigurationComplete message via SRB1 embedded in NR RRC message RRCReconfigurationComplete as specified in TS 38.331 [82];

1>	else:

2>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission using the new configuration, upon which the procedure ends;

#### 5.3.5.4	Reception of an RRCConnectionReconfiguration including the mobilityControlInfo by the UE (handover)

If the RRCConnectionReconfiguration message includes the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

<Partially omitted>

1>	if the RRCConnectionReconfiguration message includes rclwi-Configuration:

2>	perform the WLAN traffic steering command procedure as specified in 5.6.16.2;

1>	if the RRCConnectionReconfiguration message includes lwa-Configuration:

2>	perform the LWA configuration procedure as specified in 5.6.14.2;

1>	if the RRCConnectionReconfiguration message includes lwip-Configuration:

2>	perform the LWIP reconfiguration procedure as specified in 5.6.17.2;

1>	if the RRCConnectionReconfiguration message includes the sl-V2X-ConfigDedicated or mobilityControlInfoV2X:

2>	perform the V2X sidelink communication dedicated configuration procedure as specified in 5.3.10.15a;

NOTE 2d:	In case of conditional reconfiguration the text "if the received RRCConnectionReconfiguration. . ." corresponds to applying the stored RRCConnectionReconfiguration message (according to 5.3.5.9.4).

1>	if the UE is configured to operate in EN-DC as result of this procedure, forward upperLayerIndication, as if the UE receives this field from SIB2, to upper layers, otherwise indicate upper layers absence of this field;

1>	set the content of RRCConnectionReconfigurationComplete message as follows:

<Next modification>

### 6.2.2	Message definitions

<Partially omitted>

#### –	SystemInformation

The SystemInformation message is used to convey one or more System Information Blocks or Positioning System Information Blocks. All the SIBs or posSIBs included are transmitted with the same periodicity. SystemInformation-BR and SystemInformation-MBMS use the same structure as SystemInformation.

Signalling radio bearer: N/A

RLC-SAP: TM

Logical channels: BCCH and BR-BCCH

Direction: E-UTRAN to UE

SystemInformation message

-- ASN1START

SystemInformation-BR-r13 ::=	SystemInformation

SystemInformation-MBMS-r14 ::=	SystemInformation

SystemInformation ::=				SEQUENCE {

criticalExtensions					CHOICE {

systemInformation-r8				SystemInformation-r8-IEs,

criticalExtensionsFuture-r15		CHOICE {

posSystemInformation-r15			PosSystemInformation-r15-IEs,

criticalExtensionsFuture			SEQUENCE {}

}

}

}

SystemInformation-r8-IEs ::=		SEQUENCE {

sib-TypeAndInfo						SEQUENCE (SIZE (1..maxSIB)) OF CHOICE {

sib2								SystemInformationBlockType2,

sib3								SystemInformationBlockType3,

sib4								SystemInformationBlockType4,

sib5								SystemInformationBlockType5,

sib6								SystemInformationBlockType6,

sib7								SystemInformationBlockType7,

sib8								SystemInformationBlockType8,

sib9								SystemInformationBlockType9,

sib10								SystemInformationBlockType10,

sib11								SystemInformationBlockType11,

...,

sib12-v920							SystemInformationBlockType12-r9,

sib13-v920							SystemInformationBlockType13-r9,

sib14-v1130							SystemInformationBlockType14-r11,

sib15-v1130							SystemInformationBlockType15-r11,

sib16-v1130							SystemInformationBlockType16-r11,

sib17-v1250							SystemInformationBlockType17-r12,

sib18-v1250							SystemInformationBlockType18-r12,

sib19-v1250							SystemInformationBlockType19-r12,

sib20-v1310							SystemInformationBlockType20-r13,

sib21-v1430							SystemInformationBlockType21-r14,

sib24-v1530							SystemInformationBlockType24-r15,

sib25-v1530							SystemInformationBlockType25-r15,

sib26-v1530							SystemInformationBlockType26-r15,

sibxy-v16xy							SystemInformationBlockTypexy-r16,

sib27-v16xy							SystemInformationBlockType27-r16,

sib28-v16xy							SystemInformationBlockType28-r16

},

nonCriticalExtension				SystemInformation-v8a0-IEs		OPTIONAL

}

<Next modification>

#### –	SystemInformationBlockType1

SystemInformationBlockType1 contains information relevant when evaluating if a UE is allowed to access a cell and defines the scheduling of other system information. SystemInformationBlockType1-BR uses the same structure as SystemInformationBlockType1.

Signalling radio bearer: N/A

RLC-SAP: TM

Logical channels: BCCH and BR-BCCH

Direction: E-UTRAN to UE

SystemInformationBlockType1 message

-- ASN1START

SystemInformationBlockType1-BR-r13 ::=	SystemInformationBlockType1

SystemInformationBlockType1 ::=		SEQUENCE {

cellAccessRelatedInfo				SEQUENCE {

plmn-IdentityList					PLMN-IdentityList,

trackingAreaCode					TrackingAreaCode,

cellIdentity						CellIdentity,

cellBarred							ENUMERATED {barred, notBarred},

intraFreqReselection				ENUMERATED {allowed, notAllowed},

csg-Indication						BOOLEAN,

csg-Identity						CSG-Identity			OPTIONAL	-- Need OR

},

cellSelectionInfo					SEQUENCE {

q-RxLevMin							Q-RxLevMin,

q-RxLevMinOffset					INTEGER (1..8)			OPTIONAL	-- Need OP

},

p-Max								P-Max						OPTIONAL,			-- Need OP

freqBandIndicator					FreqBandIndicator,

schedulingInfoList					SchedulingInfoList,

tdd-Config							TDD-Config					OPTIONAL,	-- Cond TDD

si-WindowLength						ENUMERATED {

ms1, ms2, ms5, ms10, ms15, ms20,

ms40},

systemInfoValueTag					INTEGER (0..31),

nonCriticalExtension				SystemInformationBlockType1-v890-IEs	OPTIONAL

}

SystemInformationBlockType1-v890-IEs::=	SEQUENCE {

lateNonCriticalExtension			OCTET STRING (CONTAINING SystemInformationBlockType1-v8h0-IEs)			OPTIONAL,

nonCriticalExtension				SystemInformationBlockType1-v920-IEs	OPTIONAL

}

-- Late non critical extensions

SystemInformationBlockType1-v8h0-IEs ::=	SEQUENCE {

multiBandInfoList					MultiBandInfoList		OPTIONAL,	-- Need OR

nonCriticalExtension				SystemInformationBlockType1-v9e0-IEs	OPTIONAL

}

SystemInformationBlockType1-v9e0-IEs ::= SEQUENCE {

freqBandIndicator-v9e0				FreqBandIndicator-v9e0		OPTIONAL,	-- Cond FBI-max

multiBandInfoList-v9e0				MultiBandInfoList-v9e0		OPTIONAL,	-- Cond mFBI-max

nonCriticalExtension				SystemInformationBlockType1-v10j0-IEs	OPTIONAL

}

SystemInformationBlockType1-v10j0-IEs ::= SEQUENCE {

freqBandInfo-r10					NS-PmaxList-r10				OPTIONAL,	-- Need OR

multiBandInfoList-v10j0				MultiBandInfoList-v10j0		OPTIONAL,	-- Need OR

nonCriticalExtension				SystemInformationBlockType1-v10l0-IEs					OPTIONAL

}

SystemInformationBlockType1-v10l0-IEs ::= SEQUENCE {

freqBandInfo-v10l0					NS-PmaxList-v10l0			OPTIONAL,	-- Need OR

multiBandInfoList-v10l0				MultiBandInfoList-v10l0		OPTIONAL,	-- Need OR

nonCriticalExtension				SEQUENCE {}					OPTIONAL

}

-- Regular non critical extensions

SystemInformationBlockType1-v920-IEs ::=	SEQUENCE {

ims-EmergencySupport-r9				ENUMERATED {true}			OPTIONAL,	-- Need OR

cellSelectionInfo-v920				CellSelectionInfo-v920		OPTIONAL,	-- Cond RSRQ

nonCriticalExtension				SystemInformationBlockType1-v1130-IEs	OPTIONAL

}

SystemInformationBlockType1-v1130-IEs ::=	SEQUENCE {

tdd-Config-v1130				TDD-Config-v1130			OPTIONAL,	-- Cond TDD-OR

cellSelectionInfo-v1130			CellSelectionInfo-v1130		OPTIONAL,	-- Cond WB-RSRQ

nonCriticalExtension			SystemInformationBlockType1-v1250-IEs	OPTIONAL

}

SystemInformationBlockType1-v1250-IEs ::=	SEQUENCE {

cellAccessRelatedInfo-v1250					SEQUENCE {

category0Allowed-r12						ENUMERATED {true}		OPTIONAL	-- Need OP

},

cellSelectionInfo-v1250					CellSelectionInfo-v1250		OPTIONAL,	-- Cond RSRQ2

freqBandIndicatorPriority-r12			ENUMERATED {true}			OPTIONAL,	-- Cond mFBI

nonCriticalExtension			SystemInformationBlockType1-v1310-IEs	OPTIONAL

}

SystemInformationBlockType1-v1310-IEs ::=	SEQUENCE {

hyperSFN-r13								BIT STRING (SIZE (10))		OPTIONAL,	-- Need OR

eDRX-Allowed-r13							ENUMERATED {true}			OPTIONAL,	-- Need OR

cellSelectionInfoCE-r13					CellSelectionInfoCE-r13	OPTIONAL,	-- Need OP

bandwidthReducedAccessRelatedInfo-r13	SEQUENCE {

si-WindowLength-BR-r13					ENUMERATED {

ms20, ms40, ms60, ms80, ms120,

ms160, ms200, spare},

si-RepetitionPattern-r13				ENUMERATED {everyRF, every2ndRF, every4thRF,

every8thRF},

schedulingInfoList-BR-r13				SchedulingInfoList-BR-r13	OPTIONAL,	-- Cond SI-BR

fdd-DownlinkOrTddSubframeBitmapBR-r13	CHOICE {

subframePattern10-r13					BIT STRING (SIZE (10)),

subframePattern40-r13					BIT STRING (SIZE (40))

}																	OPTIONAL,	-- Need OP

fdd-UplinkSubframeBitmapBR-r13			BIT STRING (SIZE (10))		OPTIONAL,	-- Need OP

startSymbolBR-r13						INTEGER (1..4),

si-HoppingConfigCommon-r13				ENUMERATED {on,off},

si-ValidityTime-r13						ENUMERATED {true}	OPTIONAL,			-- Need OP

systemInfoValueTagList-r13				SystemInfoValueTagList-r13	OPTIONAL	-- Need OR

}																OPTIONAL,	-- Cond BW-reduced

nonCriticalExtension						SystemInformationBlockType1-v1320-IEs	OPTIONAL

}

SystemInformationBlockType1-v1320-IEs ::=	SEQUENCE {

freqHoppingParametersDL-r13				SEQUENCE {

mpdcch-pdsch-HoppingNB-r13				ENUMERATED {nb2, nb4}		OPTIONAL,	-- Need OR

interval-DLHoppingConfigCommonModeA-r13	CHOICE {

interval-FDD-r13					ENUMERATED {int1, int2, int4, int8},

interval-TDD-r13					ENUMERATED {int1, int5, int10, int20}

}																	OPTIONAL,	-- Need OR

interval-DLHoppingConfigCommonModeB-r13	CHOICE {

interval-FDD-r13					ENUMERATED {int2, int4, int8, int16},

interval-TDD-r13					ENUMERATED { int5, int10, int20, int40}

}																	OPTIONAL,	-- Need OR

mpdcch-pdsch-HoppingOffset-r13			INTEGER (1..maxAvailNarrowBands-r13)	OPTIONAL	-- Need OR

}																OPTIONAL,	-- Cond Hopping

nonCriticalExtension						SystemInformationBlockType1-v1350-IEs					OPTIONAL

}

SystemInformationBlockType1-v1350-IEs ::=	SEQUENCE {

cellSelectionInfoCE1-r13				CellSelectionInfoCE1-r13	OPTIONAL,	-- Need OP

nonCriticalExtension					SystemInformationBlockType1-v1360-IEs				OPTIONAL

}

SystemInformationBlockType1-v1360-IEs ::=	SEQUENCE {

cellSelectionInfoCE1-v1360				CellSelectionInfoCE1-v1360	OPTIONAL,	-- Cond QrxlevminCE1

nonCriticalExtension						SystemInformationBlockType1-v1430-IEs		OPTIONAL

}

SystemInformationBlockType1-v1430-IEs ::=	SEQUENCE {

eCallOverIMS-Support-r14				ENUMERATED {true}			OPTIONAL,	-- Need OR

tdd-Config-v1430						TDD-Config-v1430			OPTIONAL,	-- Cond TDD-OR

cellAccessRelatedInfoList-r14			SEQUENCE (SIZE (1..maxPLMN-1-r14)) OF

CellAccessRelatedInfo-r14	OPTIONAL,	-- Need OR

nonCriticalExtension					SystemInformationBlockType1-v1450-IEs				OPTIONAL

}

SystemInformationBlockType1-v1450-IEs ::=	SEQUENCE {

tdd-Config-v1450						TDD-Config-v1450		OPTIONAL,	-- Cond TDD-OR

nonCriticalExtension					SystemInformationBlockType1-v1530-IEs					OPTIONAL

}

SystemInformationBlockType1-v1530-IEs ::=	SEQUENCE {

hsdn-Cell-r15						ENUMERATED {true}			OPTIONAL,	-- Need OR

cellSelectionInfoCE-v1530			CellSelectionInfoCE-v1530	OPTIONAL,	-- Need OP

crs-IntfMitigConfig-r15				CHOICE {

crs-IntfMitigEnabled-15				NULL,

crs-IntfMitigNumPRBs-r15			ENUMERATED {n6, n24}

}	OPTIONAL,	-- Need OR

cellBarred-CRS-r15					ENUMERATED {barred, notBarred},

plmn-IdentityList-v1530				PLMN-IdentityList-v1530		OPTIONAL,	-- Need OR

posSchedulingInfoList-r15			PosSchedulingInfoList-r15	OPTIONAL,	-- Need OR

cellAccessRelatedInfo-5GC-r15		SEQUENCE {

cellBarred-5GC-r15					ENUMERATED {barred, notBarred},

cellBarred-5GC-CRS-r15				ENUMERATED {barred, notBarred},

cellAccessRelatedInfoList-5GC-r15	SEQUENCE (SIZE (1..maxPLMN-r11)) OF

CellAccessRelatedInfo-5GC-r15

}				OPTIONAL,	-- Need OP

ims-EmergencySupport5GC-r15			ENUMERATED {true}			OPTIONAL,	-- Need OR

eCallOverIMS-Support5GC-r15			ENUMERATED {true}			OPTIONAL,	-- Need OR

nonCriticalExtension				SystemInformationBlockType1-v1540-IEs		OPTIONAL

}

SystemInformationBlockType1-v1540-IEs ::=	SEQUENCE {

si-posOffset-r15								ENUMERATED {true}		OPTIONAL,	-- Need ON

nonCriticalExtension							SystemInformationBlockType1-v16xy-IEs	OPTIONAL

}

SystemInformationBlockType1-v16xy-IEs ::=	SEQUENCE {

eDRX-Allowed-5GC-r16						ENUMERATED {true}		OPTIONAL,	-- Need OR

bandwidthReducedAccessRelatedInfo-v16xy		SEQUENCE {

transmissionInControlChRegion-r16			ENUMERATED {true}		OPTIONAL	-- Need OR

}						OPTIONAL,	-- Cond BW-reduced

plmn-IdentityList-v16xy						PLMN-IdentityList-v16xy		OPTIONAL,	-- Need OR

nonCriticalExtension						SEQUENCE {}			OPTIONAL

}

PLMN-IdentityList ::=					SEQUENCE (SIZE (1..maxPLMN-r11)) OF PLMN-IdentityInfo

PLMN-IdentityInfo ::=					SEQUENCE {

plmn-Identity							PLMN-Identity,

cellReservedForOperatorUse				ENUMERATED {reserved, notReserved}

}

PLMN-IdentityList-v1530 ::=				SEQUENCE (SIZE (1..maxPLMN-r11)) OF PLMN-IdentityInfo-v1530

PLMN-IdentityInfo-v1530 ::=				SEQUENCE {

cellReservedForOperatorUse-CRS-r15		ENUMERATED {reserved, notReserved}

}

PLMN-IdentityList-r15::=			SEQUENCE (SIZE (1..maxPLMN-r11)) OF PLMN-IdentityInfo-r15

PLMN-IdentityList-v16xy::=	SEQUENCE (SIZE (1..maxPLMN-r11)) OF PLMN-IdentityInfo-v16xy

PLMN-IdentityInfo-r15 ::=			SEQUENCE {

plmn-Identity-5GC-r15				CHOICE{

plmn-Identity-r15					PLMN-Identity,

plmn-Index-r15						INTEGER (1..maxPLMN-r11)

},

cellReservedForOperatorUse-r15			ENUMERATED {reserved, notReserved},

cellReservedForOperatorUse-CRS-r15		ENUMERATED {reserved, notReserved}

}

PLMN-IdentityInfo-v16xy ::=	SEQUENCE {

cp-CIoT-5GS-Optimisation-r16	ENUMERATED {true}			OPTIONAL,	-- Need OR

up-CIoT-5GS-Optimisation-r16	ENUMERATED {true}			OPTIONAL,	-- Need OR

iab-support						ENUMERATED {true}			OPTIONAL	-- Need OR

}

SchedulingInfoList ::= SEQUENCE (SIZE (1..maxSI-Message)) OF SchedulingInfo

SchedulingInfo ::=	SEQUENCE {

si-Periodicity				ENUMERATED {rf8, rf16, rf32, rf64, rf128, rf256, rf512},

sib-MappingInfo				SIB-MappingInfo

}

SchedulingInfoList-BR-r13 ::= SEQUENCE (SIZE (1..maxSI-Message)) OF SchedulingInfo-BR-r13

SchedulingInfo-BR-r13 ::=	SEQUENCE {

si-Narrowband-r13		INTEGER (1..maxAvailNarrowBands-r13),

si-TBS-r13				ENUMERATED {b152, b208, b256, b328, b408, b504, b600, b712, b808, b936}

}

SIB-MappingInfo ::= SEQUENCE (SIZE (0..maxSIB-1)) OF SIB-Type

SIB-Type ::=						ENUMERATED {

sibType3, sibType4, sibType5, sibType6,

sibType7, sibType8, sibType9, sibType10,

sibType11, sibType12-v920, sibType13-v920,

sibType14-v1130, sibType15-v1130,

sibType16-v1130, sibType17-v1250, sibType18-v1250,

..., sibType19-v1250, sibType20-v1310, sibType21-v1430,

sibType24-v1530, sibType25-v1530, sibType26-v1530,

sibTypexy-v16xy, sibType27-v16xy, sibType28-v16xy}

SystemInfoValueTagList-r13 ::=		SEQUENCE (SIZE (1..maxSI-Message)) OF SystemInfoValueTagSI-r13

SystemInfoValueTagSI-r13 ::=		INTEGER (0..3)

CellSelectionInfo-v920 ::=			SEQUENCE {

q-QualMin-r9						Q-QualMin-r9,

q-QualMinOffset-r9					INTEGER (1..8)						OPTIONAL	-- Need OP

}

CellSelectionInfo-v1130 ::=			SEQUENCE {

q-QualMinWB-r11						Q-QualMin-r9

}

CellSelectionInfo-v1250 ::=			SEQUENCE {

q-QualMinRSRQ-OnAllSymbols-r12		Q-QualMin-r9

}

CellAccessRelatedInfo-r14 ::=	SEQUENCE {

plmn-IdentityList-r14				PLMN-IdentityList,

trackingAreaCode-r14				TrackingAreaCode,

cellIdentity-r14					CellIdentity

}

CellAccessRelatedInfo-5GC-r15 ::=	SEQUENCE {

plmn-IdentityList-r15			PLMN-IdentityList-r15,

ran-AreaCode-r15					RAN-AreaCode-r15 OPTIONAL,	-- Need OR

trackingAreaCode-5GC-r15			TrackingAreaCode-5GC-r15,

cellIdentity-5GC-r15				CellIdentity-5GC-r15

}

CellIdentity-5GC-r15 ::= CHOICE{

cellIdentity-r15	CellIdentity,

cellId-Index-r15	INTEGER (1..maxPLMN-r11)

}

PosSchedulingInfoList-r15 ::= SEQUENCE (SIZE (1..maxSI-Message)) OF PosSchedulingInfo-r15

PosSchedulingInfo-r15 ::=	SEQUENCE {

posSI-Periodicity-r15		ENUMERATED {rf8, rf16, rf32, rf64, rf128, rf256, rf512},

posSIB-MappingInfo-r15		PosSIB-MappingInfo-r15

}

PosSIB-MappingInfo-r15 ::= SEQUENCE (SIZE (1..maxSIB)) OF PosSIB-Type-r15

PosSIB-Type-r15 ::= SEQUENCE {

encrypted-r15		ENUMERATED { true }				OPTIONAL,		-- Need OP

gnss-id-r15			GNSS-ID-r15						OPTIONAL,		-- Need OP

sbas-id-r15			SBAS-ID-r15						OPTIONAL,		-- Need OP

posSibType-r15		ENUMERATED {	posSibType1-1,

posSibType1-2,

posSibType1-3,

posSibType1-4,

posSibType1-5,

posSibType1-6,

posSibType1-7,

posSibType2-1,

posSibType2-2,

posSibType2-3,

posSibType2-4,

posSibType2-5,

posSibType2-6,

posSibType2-7,

posSibType2-8,

posSibType2-9,

posSibType2-10,

posSibType2-11,

posSibType2-12,

posSibType2-13,

posSibType2-14,

posSibType2-15,

posSibType2-16,

posSibType2-17,

posSibType2-18,

posSibType2-19,

posSibType3-1,

...,

posSibType1-8-v16xy,

posSibType2-20-v16xy,

posSibType2-21-v16xy,

posSibType2-22-v16xy,

posSibType2-23-v16xy,

posSibType2-24-v16xy,

posSibType2-25-v16xy,

posSibType4-1-v16xy,

posSibType5-1-v16xy

},

...

}

-- ASN1STOP

<Next modification>

### 6.3.1	System information blocks

<Partially omitted>

#### –	SystemInformationBlockType28

The IE SystemInformationBlockType28 contains NR sidelink communication configuration.

SystemInformationBlockType28 information element

-- ASN1START

SystemInformationBlockType28-r16 ::= SEQUENCE {

sl-ConfigCommonNR-r16				OCTET STRING				OPTIONAL,	-- Need OR

lateNonCriticalExtension			OCTET STRING				OPTIONAL,

...

}

-- ASN1STOP

SystemInformationBlockType28 field descriptions

sl-ConfigCommonNR Container for the configuration for NR sidelink communication, this fieild includes the SL-ConfigCommonNR IE as specified in TS 38.331 [82].

#### –	SystemInformationBlockTypexy

The IE SystemInformationBlockTypexy contains NR bands list which can be used for EN-DC operation with the serving cell.

SystemInformationBlockTypexy information element

-- ASN1START

SystemInformationBlockTypexy-r16 ::= SEQUENCE {

plmn-InfoList-r16					PLMN-InfoList-r16,

bandListENDC-r16					BandListENDC-r16,

...

}

BandListENDC-r16 ::=		SEQUENCE (SIZE (1.. maxBandsENDC-r16)) OF FreqBandIndicatorNR-r15

PLMN-InfoList-r16 ::=		SEQUENCE (SIZE (1..maxPLMN-r11)) OF PLMN-Info-r16

PLMN-Info-r16 ::=			SEQUENCE {

nrBandList-r16				BIT STRING (SIZE(maxBandsENDC-r16))        OPTIONAL		-- Need OR

}

-- ASN1STOP

SystemInformationBlockTypexy field descriptions

bandListENDC A list of NR bands which can be configured as SCG in EN-DC operation with serving cell for the forwarding of upperLayerIndication to upper layers.

plmn-InfoList This field includes the same number of entries, and listed in the same order as PLMNs across the plmn-IdentityList fields plmn-IdentityList and plmn-IdentityList-r14 included in SIB1. I.e. the first entry corresponds to the first entry of the combined list that results from concatenating the entries included in the second to the original plmn-IdentityList field in SIB1.

nrBandList This field is encoded as a bitmap, where the bit N is set to “1” if the current serving cell supports EN-DC operation with the N-th NR band in bandListENDC. The bits which have no corresponding bands in bandListENDC shall be set to 0; bit 1 of the bitmap is the leading bit of the bit string.

<Partially omitted>

<Next modification>

## 6.4	RRC multiplicity and type constraint values

### –	Multiplicity and type constraint definitions

-- ASN1START

ffsValue					INTEGER ::= 65536 -- Placeholder for all FFS value

hiFFS						INTEGER ::= 64		-- Highest value of a range that still is FFS. To be removed.

maxAccessCat-1-r15			INTEGER ::=	63	-- Maximum number of Access Categories - 1

maxACDC-Cat-r13				INTEGER ::=	16	-- Maximum number of ACDC categories (per PLMN)

maxAvailNarrowBands-r13		INTEGER ::=	16	-- Maximum number of narrowbands

maxBandComb-r10				INTEGER ::=	128	-- Maximum number of band combinations.

maxBandComb-r11				INTEGER ::=	256	-- Maximum number of additional band combinations.

maxBandComb-r13				INTEGER ::=	384 -- Maximum number of band combinations in Rel-13

maxBands					INTEGER ::= 64	-- Maximum number of bands listed in EUTRA UE caps

maxBandsNR-r15				INTEGER ::= 1024	-- Maximum number of NR bands listed in EUTRA UE caps

maxBandsENDC-r16			INTEGER ::= 10	-- Maximum number of NR bands from across all the PLMNs sharing the serving cell in EN-DC for the forwarding of upperLayerIndication.

maxBandwidthClass-r10		INTEGER ::=	16	-- Maximum number of supported CA BW classes per band

maxBandwidthCombSet-r10		INTEGER ::=	32	-- Maximum number of bandwidth combination sets per

-- supported band combination

<Partially omitted>

-- ASN1STOP

<Next modification>

Annex G (normative): List of CRs Containing Early Implementable Features and Corrections

This annex lists the Change Requests (CRs) whose changes may be implemented by a UE of an earlier release than which the CR was approved in (i.e. CRs that contain on their coversheets the sentence "Implementation of this CR from Rel-N will not cause interoperability issues").

Table G-1: List of CRs Containing Early Implementable Features and Corrections

| TDoc Number (RP-xxxxxx): CR Title | CR Number(s) | CR Revision Number(s) | Earliest Implementable Release | Additional Information |
| --- | --- | --- | --- | --- |
| RP-181233: Successful acknowledgement of RRCConnectionRelease for BL and CE UE | 3324 | 1 | Release 13 | RRCConnectionRelease message, for which the poll bit is not set, can be considered succesfully acknowledged when UE has sent HARQ ACK feedback. |
| RP-182674: CR for T312 on LTE HetNet mobility | 3506 | 5 | Release 12 | Remove T312 in leaving condition for event trigger. |
| RP-182671: Corrections on paging monitoring and SI acquisition in RRC_CONNECTED for BL UEs and UEs in CE | 3647 | 2 | Release 13 |  |
| RP-190548: Update description of ack-NACK-NumRepetitions | 3899 | 2 | Release 13 |  |
| RP-190548: Corrections of NB-IoT Access Barring | 3900 | 2 | Release 13 |  |
| RP-191382: SI update notification and access barring in NB-IoT | 4020 | 2 | Release 13 |  |
| RP-192195 : Correction on handling of SCell(s) during Make Before Break handover | 3986 | 3 | Release 14 |  |
| RP-192940: Stop using redirectedCarrierOffsetDedicated after reselection to another frequency | 4144 | 1 | Release 14 |  |
| RP-200338: Corrections to T312 and Discovery Signals measurement | 4198 | 1 | Release 12 |  |
| RP-200367: Correction on H1 and H2 events | 4103 | 2 | Release 15 |  |
| RP-xxxxxx: upperLayerIndication enhancements | 4266 | 2 | Release 15 |  |
| NOTE 1: In case a CR has mirror CR(s), the mirror CR(s) are not listed. NOTE 2: The Additional Information column briefly describes the content of a CR in cases where the CR title may not be descriptive enough. If the CR title is descriptive enough, then the Additional Information column may be left blank. |  |  |  |  |