3GPP TSG-RAN2 Meeting #110 electronic	R2-2005344

# Online, 1st – 12th June, 2020

Source: 			ZTE Corporation, Sanechips

Title: 	On stopping evaluating execution condition once triggering the legacy HO

Agenda item:		6.9.2

Document for:		Discussion and Decision

# Introduction

In this contribution, we discuss whether to stop evaluating CHO execution condition once triggering the legacy HO.

# Discussion

At last meeting, we agreed that “Rely on existing Stage-2 text that UE stops evaluating execution condition and capture nothing additional in NR/LTE RRC specification about CHO execution conditions after the CHO condition is met and CHO execution is started.”[1]. However, since the legacy HO command can be received before triggering the CHO execution and the UE may continue measurements for candidate cells configured for execution condition during the execution of legacy HO, we think the UE should also stop evaluating CHO execution condition once triggering the execution of legacy HO, as triggering the CHO execution.

Proposal 1: The UE stops evaluating CHO execution condition once triggering the legacy HO, if configured.

In current stage-2 spec, we captured that “The UE starts evaluating the execution condition(s) upon receiving the CHO configuration, and stops evaluating the execution condition(s) once the execution condition(s) is met.” [2]. Considering the UE should stop evaluating the CHO execution condition once either CHO or legacy HO is triggered and both two operations can be defined as “handover” in stage-2 spec, we propose to capture “stops evaluating the execution condition(s) once a handover is triggered” for simplicity.

Proposal 2: Capture “stops evaluating the execution condition(s) once a handover is triggered” in stage-2 CR.

The corresponding impact on specs can be found in the annex for information.

# Conclusion and proposals

In this contribution, we discuss on stopping evaluating CHO execution condition once triggering the legacy HO with the following proposals:

Proposal 1: The UE stops evaluating CHO execution condition once triggering the legacy HO, if configured.

Proposal 2: Capture “stops evaluating the execution condition(s) once a handover is triggered” in stage-2 CR.

# Reference

- RAN2#109bis-e chairman notes.

- R2-200xxxx  Corrections on NR mobility enhancements  Intel.

# Annex

The proposed TP based on the 38.300 RRC CR in [2] is as follows:

CHANGE START

#### 9.2.3.4	Conditional Handover

##### 9.2.3.4.1	General

A Conditional Handover (CHO) is defined as a handover that is executed by the UE when one or more handover execution conditions are met. The UE starts evaluating the execution condition(s) upon receiving the CHO configuration, and stops evaluating the execution condition(s) once a handover is triggered.

CHANGE END