3GPP TSG-RAN WG2 Meeting# 110 electronic	R2-2005480

Online, 1 – 12 June 2020

Agenda Item:	6.4.2.2

Source: 	Huawei, HiSilicon

Title: 	Miscellaneous Stage-2 corrections for NR SL communication in TS 36.300

Document for:	Discussion and decision

Introduction

This paper discusses some Stage-2 aspects which may not be appropriately captured in the current Stage-2 specification [1] for NR SL communication, and proposes corresponding changes with reasons elaborated. A TP is attached to implement the proposed changes accordingly.

Discussion – Reasons for changes in TS 36.300

- Proposed change 1

It is first seen that, now the EUTRA control of NR sidelink communication is merged in the subclause “23.14	Support of V2X service” in TS 36.300 [1].

## 23.14 Support for V2X services ### 23.14.1 General #### 23.14.1.0 Overview Vehicular communication services, represented by V2X services, can consist of the following four different types: V2V, V2I, V2N and V2P, as specified in TS 22.185 [71]. V2X services can be provided by PC5 interface and/or Uu interface. Support of V2X services via PC5 interface is provided by V2X sidelink communication as specified in TS 23.285 [72] and/or NR sidelink communication as specified in TS 23.287 [93], which are modes of communication whereby UEs can communicate with each other directly over the PC5 interface. Both communications mode may be supported when the UE is served by E-UTRAN and when the UE is outside of E-UTRA coverage. Only the UEs authorised to be used for V2X services can perform V2X sidelink communication and/or NR sidelink communications for V2X services. NR sidelink communication may be used to support other services than V2X services. [...] - 23.14.1.3 E-UTRAN control for NR sidelink communication When the UE is served by E-UTRAN, if the UE supports and is authorized to perform NR sidelink communication in E-UTRAN, NR sidelink communication can be configured and controlled by E-UTRAN via dedicated signaling and/or system information, using the procedures specified in TS 38.300 [79], with the following restrictions to operation of NR sidelink communication controlled by E-UTRAN: - Dynamic sidelink scheduling and the configured sidelink grant with type 2 are not supported for the UE served by E-UTRAN.

However, this is a bit “self-contradictory”, as the title of 23.14 clearly says “for V2X services”, but it is also says that NR sidelink communication included therein is Not limited to V2X services, but can be used for other services.

To deal with this, a more proper way seems to give a separate subclause in clause 23 for E-UTRA control for NR sidelink communication, instead of merging it into suclause 23.14 dedicated for V2X. Therefore, the below is proposed:

Proposal 11: Introduce a new subclause in clause 23 and move the descriptions for “E-UTRAN control for NR sidelink communication” into this new subclause.

- Proposed change 2

It is seen in above table that a restriction is listed for the E-UTRAN control for NR sitelink communication, i.e. dynamic scheduling and configured sidelink grant Type 2 in 38-series of TS are not supported herein. This is from RAN1 perspective. In addition, actually there is another restriction from RAN2 perspective which corresponds to UL and SL transmission prioritization. Specifically, compared with NR SL and NR UL transmission prioritization, where priorities of both SL and UL are considered (i.e. compared to respective thresholds), the prioritization between NR SL and LTE UL transmission, in the case of E-UTRA control for NR SL, reuses the legacy LTE V2X SL scheme, i.e. only the priority of the NR SL transmission is considered. This is except for the case where “UL transmission is prioritized by upper layer as specified in TS 24.386 [75] or random access procedure is performed”

Therefore, this restriction should also be reflected in TS 36.300.

Proposal 12: For E-UTRAN control for NR sidelink communication, add the restriction that the prioritization between E-UTRA UL transmission and NR SL transmission, if needed, is performed based on the priorities in the sidelink MAC PDU only, except that the UL transmission is prioritized by upper layer as specified in TS 24.386 or random access procedure is performed.

The detailed TP based on the above proposals to TS 36.300 is given in Annex A attached.

Conclusion

In this paper, changes are proposed to TS 36.300 for NR sidelink communication. It is proposed that RAN2 adopt the TPs given in Annex A, and agree them in a formal CR.

References

- [1]	TS 36.300, V16.1.0

- Annex A: TP for 36.300

- ================= Start of the TP =================

# 23	Others

## 23.1	Support for real time IMS services

### 23.1.1	IMS Emergency Call

- [...]

## 23.14	Support for V2X services

### 23.14.1	General

#### 23.14.1.0	Overview

Vehicular communication services, represented by V2X services, can consist of the following four different types: V2V, V2I, V2N and V2P, as specified in TS 22.185 [71].

V2X services can be provided by PC5 interface and/or Uu interface. Support of V2X services via PC5 interface is provided by V2X sidelink communication as specified in TS 23.285 [72] and/or NR sidelink communication as specified in TS 23.287 [93], which are modes of communication whereby UEs can communicate with each other directly over the PC5 interface. Both communications mode may be supported when the UE is served by E-UTRAN and when the UE is outside of E-UTRA coverage. Only the UEs authorised to be used for V2X services can perform V2X sidelink communication and/or NR sidelink communications for V2X services. NR sidelink communication may be used to support other services than V2X services.

- [...]

- 23.14.1.3	E-UTRAN control for NR sidelink communication

When the UE is served by E-UTRAN, if the UE supports and is authorized to perform NR sidelink communication in E-UTRAN, NR sidelink communication can be configured and controlled by E-UTRAN via dedicated signaling and/or system information, using the procedures specified in TS 38.300 [79], with the following restrictions to operation of NR sidelink communication controlled by E-UTRAN:

-	Dynamic sidelink scheduling and the configured sidelink grant with type 2 are not supported for the UE served by E-UTRAN.

## 23.15	Support for MMTEL voice and video enhancements

- [...]

## 23.18	PDCP packet duplication

PDCP packet duplication is configured for a RB by RRC where two logical channels are configured for the RB. The two logical channels can either belong to the same MAC entity (referred to as CA duplication) or different MAC entities (referred to as DC duplication). When activated, PDCP packet duplication allows sending the same PDCP PDU on two independent transmission paths: via the primary RLC entity and a secondary RLC entity, thus increasing reliability and reducing latency.

PDCP packet duplication is supported in the following cases:

-	for SRBs using RLC AM;

-	for DRBs using RLC UM or AM.

For DRBs, duplication can be activated and deactivated by a MAC CE. In addition, for DRBs, PDCP packet duplication can be activated upon configuration by RRC signalling. For SRBs, once duplication is configured, it is always activated.

When PDCP packet duplication is activated, the associated logical channels are restricted to be sent only on certain serving cells to ensure the duplicates are sent on different serving cells. The restriction is lifted when PDCP packet duplication is deactivated. When CA duplication is configured for an SRB, one of the logical channels associated to the SRB is restricted to be sent only on the serving cells including SpCell.

At the receiver, PDCP enables reordering and duplication detection when PDCP packet duplication is configured.

When activating duplication for a DRB, E-UTRAN should ensure that at least one serving cell is activated for each logical channel of the DRB; and when the deactivation of SCells leaves no serving cells activated for a logical channel of the DRB, E-UTRAN should ensure that duplication is also deactivated.

## 23.X	E-UTRAN control for NR sidelink communication

NR sidelink communication may be used to support other services than V2X services as in subclause 23.14.1.0. When the UE is served by E-UTRAN, if the UE supports and is authorized to perform NR sidelink communication in E-UTRAN, NR sidelink communication can be configured and controlled by E-UTRAN via dedicated signaling and/or system information, using the procedures specified in TS 38.300 [79], with the following restrictions to operation of NR sidelink communication controlled by E-UTRAN:

-	Dynamic sidelink scheduling and the configured sidelink grant with type 2 are not supported for the UE served by E-UTRAN.

-	The prioritization between EUTRA UL transmission and NR SL transmission, if needed, is performed based on the priorties in the sidelink MAC PDU only, except that the UL transmission is prioritized by upper layer as specified in TS 24.386 [75] or random access procedure is performed.

# 24	Support for 5GC

- [...]

- ================= End of the TP =================