3GPP TSG-RAN2 Meeting #110-electronic	  R2-2005614

# E-meeting, 1st – 12th June 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 0756 | rev |  | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38321 on RACH Prioritization for MPS and MCS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core, TEI16 |  |  |  |  |  | Date: |  |  | 2020-05-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In RAN2#107, it was agreed to support the RACH prioritization procedure for MCS and MPS in Rel-16 NR. With this, the network is allowed to configure RACH prioritization parameters (e.g. ra-Prioritization) for MPS and MCS via SIB1. At the UE side, it checks whether the configured RACH prioritization parameters can be applied or not based on the Access Identity(ies) during the RACH initialization procedure. According to the following highlighted text quoted from MAC specification, the Access Identity(ies) needs to be mandatorily provided to MAC by RRC during/before the initialization of RA procedure. Unfortunately, the Access Identity(ies) cannot be explicitly provided to MAC according to the current RRC specification. Consequently, the highlighted conditional branch below is always not satisfied. The RACH prioritization procedure for MCS and MPS can never be applied. TS 38.321 subclause 5.1.1a: 2> if ra-PrioritizationForAccessIdentity is configured for the selected carrier; and 2> if one or more Access Identities has been explicitly provided by RRC; and 2> if for at least one of these Access Identities the corresponding bit in the ra-PriorizationForAI is set to one: 3> if powerRampingStepHighPriority is configured in the ra-PrioritizationForAccessIdentity: 4> set PREAMBLE_POWER_RAMPING_STEP to the powerRampingStepHighPriority. 3> if scalingFactorBI is configured in the ra-PrioritizationForAccessIdentity: 4> set SCALING_FACTOR_BI to the scalingFactorBI. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Considering that the Access Identity(ies) is selected and provided to lower layers by the NAS, the terminology “RRC” in clause 5.1.1a of TS 38.321 is replaced by the terminology “upper layers”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The RACH prioritization procedure for MCS and MPS can never be applied. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

5.1.1a	Initialization of variables specific to Random Access type

The MAC entity shall:

1>	if RA_TYPE is set to 2-stepRA:

2>	set PREAMBLE_POWER_RAMPING_STEP to msgA-PreamblePowerRampingStep;

2>	set SCALING_FACTOR_BI to 1;

2>	apply preambleTransMax included in the RACH-ConfigGenericTwoStepRA;

2> if msgA-TransMax is included in the RACH-ConfigCommonTwoStepRA

3> apply msgA-TransMax included in the RACH-ConfigCommonTwoStepRA.

2> else:

3> consider that msgA-TransMax is not configured.

2>	if the Random Access procedure was initiated for SpCell beam failure recovery (as specified in clause 5.17); and

2>	if beamFailureRecoveryConfig is configured for the active UL BWP of the selected carrier:

3>	if ra-PrioritizationTwoStep is configured in the beamFailureRecoveryConfig:

4>	set PREAMBLE_POWER_RAMPING_STEP to the powerRampingStepHighPriority included in the ra-PrioritizationTwoStep in beamFailureRecoveryConfig.

4>	if scalingFactorBI is configured in the ra-PrioritizationTwoStep in beamFailureRecoveryConfig:

5>	set SCALING_FACTOR_BI to the scalingFactorBI.

2>	else if the Random Access procedure was initiated for handover; and

2>	if rach-ConfigDedicated is configured for the selected carrier:

3>	if ra-PrioritizationTwoStep is configured in the rach-ConfigDedicated:

4>	set PREAMBLE_POWER_RAMPING_STEP to the powerRampingStepHighPriority included in the ra-PrioritizationTwoStep in rach-ConfigDedicated.

4>	if scalingFactorBI is configured in ra-PrioritizationTwoStep in the rach-ConfigDedicated:

5>	set SCALING_FACTOR_BI to the scalingFactorBI.

3>	if msgA-TransMax is configured in the rach-ConfigDedicated:

4>	apply msgA-TransMax configured in the rach-ConfigDedicated.

3> else:

4> consider that msgA-TransMax is not configured.

2>	if ra-PrioritizationForAccessIdentityTwoStep is configured for the selected carrier; and

2>	if one or more Access Identities has been explicitly provided by upper layers; and

2>	if for at least one of these Access Identities the corresponding bit in the ra-PriorizationForAI is set to one:

3>	if powerRampingStepHighPriority is configured in the ra-PrioritizationForAccessIdentityTwoStep:

4>	set PREAMBLE_POWER_RAMPING_STEP to the powerRampingStepHighPriority.

3>	if scalingFactorBI is configured in the ra-PrioritizationForAccessIdentityTwoStep:

4>	set SCALING_FACTOR_BI to the scalingFactorBI.

2>	set MSGA_PREAMBLE_POWER_RAMPING_STEP to PREAMBLE_POWER_RAMPING_STEP.

1>	else (i.e. RA_TYPE is set to 4-stepRA):

2>	set PREAMBLE_POWER_RAMPING_STEP to powerRampingStep;

2>	set SCALING_FACTOR_BI to 1;

2>	set preambleTransMax to preambleTransMax included in the RACH-ConfigGeneric;

2>	if the Random Access procedure was initiated for SpCell beam failure recovery (as specified in clause 5.17); and

2>	if beamFailureRecoveryConfig is configured for the active UL BWP of the selected carrier:

3>	start the beamFailureRecoveryTimer, if configured;

3>	apply the parameters powerRampingStep, preambleReceivedTargetPower, and preambleTransMax configured in the beamFailureRecoveryConfig;

3>	if ra-Prioritization  is configured in the beamFailureRecoveryConfig:

4>	set PREAMBLE_POWER_RAMPING_STEP to the powerRampingStepHighPriority included in the ra-Prioritization in beamFailureRecoveryConfig.

4>	if scalingFactorBI is configured in the ra-Prioritization in beamFailureRecoveryConfig:

5>	set SCALING_FACTOR_BI to the scalingFactorBI.

2>	else if the Random Access procedure was initiated for handover; and

2>	if rach-ConfigDedicated is configured for the selected carrier:

3>	if ra-Prioritization is configured in the rach-ConfigDedicated:

4>	set PREAMBLE_POWER_RAMPING_STEP to the powerRampingStepHighPriority included in the ra-Prioritization in rach-ConfigDedicated.

4>	if scalingFactorBI is configured in the ra-Prioritization in rach-ConfigDedicated:

5>	set SCALING_FACTOR_BI to the scalingFactorBI.

2>	if ra-PrioritizationForAccessIdentity is configured for the selected carrier; and

2>	if one or more Access Identities has been explicitly provided by upper layers; and

2>	if for at least one of these Access Identities the corresponding bit in the ra-PriorizationForAI is set to one:

3>	if powerRampingStepHighPriority is configured in the ra-PrioritizationForAccessIdentity:

4>	set PREAMBLE_POWER_RAMPING_STEP to the powerRampingStepHighPriority.

3>	if scalingFactorBI is configured in the ra-PrioritizationForAccessIdentity:

4>	set SCALING_FACTOR_BI to the scalingFactorBI.

2>	if RA_TYPE is switched from 2-stepRA to 4-step RA during this Random Access procedure:

3>	set POWER_OFFSET_2STEP_RA to (PREAMBLE_POWER_RAMPING_COUNTER – 1) × (MSGA_PREAMBLE_POWER_RAMPING_STEP – PREAMBLE_POWER_RAMPING_STEP).

END OF CHANGE