3GPP TSG-RAN WG2 Meeting #110-e	R2-2005635

1 June – 12 June 2020

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 1686 | rev | - | Current version: | 15.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MAC default configuration for SCG |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-06-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The MAC-CellGroupConfig IE is an optional Need M, therefore network may or may not include it when configuring or adding a cell group. Per the spec, when configuring or adding the MCG (with FullConfig or during establishment or during IRAT HO to NR) the UE shall apply the default configuration per table 9.2.2 below. ### 9.2.2 Default MAC Cell Group configuration Parameters Name; Value; Semantics description; Ver. MAC Cell Group configuration. bsr-Config. >periodicBSR-Timer; sf10. >retxBSR-Timer; sf80. phr-Config. >phr-PeriodicTimer; sf10. >phr-ProhibitTimer; sf10. >phr-Tx-PowerFactorChange; dB1. However, when network adds an SCG, if MAC-CellGroupConfig IE was not included (or partially included) in the reconfiguration message, the expected UE behavior is not defined as it’s not clear in the spec whether UE can apply the default MAC Cell Group configuration or not. #### 5.3.5.11 Full configuration The UE shall: 1> release/ clear all current dedicated radio configurations except for the following: - the MCG C-RNTI; - the AS security configurations associated with the master key; NOTE 1: Radio configuration is not just the resource configuration but includes other configurations like MeasConfig. In case NR-DC or NE-DC is configured, this also includes the entire NR or E-UTRA SCG configuration which are released according to the MR-DC release procedure as specified in 5.3.5.10. The radio configuration does not include SRB1/SRB2 configurations and DRB configurations as configured by radioBearerConfig or radioBearerConfig2. 1> if the spCellConfig in the masterCellGroup includes the reconfigurationWithSync (i.e., SpCell change): 2> release/ clear all current common radio configurations; 2> use the default values specified in 9.2.3 for timers T310, T311 and constants N310, N311; 1> else (full configuration after re-establishment or during RRC resume): 2> use values for timers T301, T310, T311 and constants N310, N311, as included in ue-TimersAndConstants received in SIB1; 1> apply the default L1 parameter values as specified in corresponding physical layer specifications except for the following: - parameters for which values are provided in SIB1; 1> apply the default MAC Cell Group configuration as specified in 9.2.2; |
| Summary of change: |  | Applying the default MAC configuration upon SCG establishment. Impact Analysis: Impacted 5G architecture options: NR-DC, NR-SA Impacted functionality: BSR configuration Interoperability issue: - if the network is implemented according to the CR and the UE is not, the MAC configuration applied by the UE may differ from network configuration. - if the UE is implemented according to the CR and the network is not, the MAC configuration applied by the UE may differ from network configuration. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MAC configuration applied by UE stays undefined when NR SCG is added, and no or partial MAC configuration is provided by network. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.5.5.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Change Start

##### 5.3.5.5.5	MAC entity configuration

The UE shall:

1>	if SCG MAC is not part of the current UE configuration (i.e. SCG establishment):

2>	create an SCG MAC entity;

2> apply the default MAC Cell Group configuration as specified in 9.2.2;

1>	reconfigure the MAC main configuration of the cell group in accordance with the received mac-CellGroupConfig excluding tag-ToReleaseList and tag-ToAddModList;

1>	if the received mac-CellGroupConfig includes the tag-ToReleaseList:

2>	for each TAG-Id value included in the tag-ToReleaseList that is part of the current UE configuration:

3>	release the TAG indicated by TAG-Id;

1>	if the received mac-CellGroupConfig includes the tag-ToAddModList:

2>	for each tag-Id value included in tag-ToAddModList that is not part of the current UE configuration (TAG addition):

3>	add the TAG, corresponding to the tag-Id, in accordance with the received timeAlignmentTimer;

2>	for each tag-Id value included in tag-ToAddModList that is part of the current UE configuration (TAG modification):

3>	reconfigure the TAG, corresponding to the tag-Id, in accordance with the received timeAlignmentTimer.

Change End