3GPP TSG-RAN WG2 Meeting #110e	R2-2005685

Online, 1 – 12 June 2020	Revision of R2-2002905

Agenda Item		:	7.3.3

Source		:	LG Electronics Inc.

Title		:	Consideration on DAPS Capability

Document for		:	Discussion and Decision

1. Introduction

In RAN2#109 meeting, RAN2 agreed to followings for DAPS capability [1]:

Agreements [AT109e][211][MOB] (R2-2001727)

5: Below RAN4 capabilities are introduced as baseline, and may be revised if more inputs are received from RAN4.

•	Per BC: AsyncDAPS, supportedNumberTAG, singleUL-Transmission;

•	Per Band per BC: intraBandDiffSCS, intraFreq-DAPS;

In this contribution, for Rel-16, we discuss DAPS capability issue based on a reply from RAN4.

2. Discussion

In the RAN2#109e meeting, RAN2 received a reply LS on UE capabilities on DAPS HO from RAN4 [2]:

RAN4 has only considered DAPS HO with PCell in source cell and PCell in target cell and has concluded that UE needs to indicate the following: For both LTE and NR capabilities: - Capability for DAPS HO per combination of bands where simultaneous downlink connectivity with source and target cells are supported - Synchronous vs. asynchronous support for DAPS HO - Support of simultaneous UL transmission to source and target cell per band combination - Support of indication of intra-band (separate signalling for intra-frequency and inter-frequency) HO - Support of multi TAG (i.e., different TAG in source and target cells) - Any baseband and RF capability that is per CC, per band, per band combination, or per band of band combination which is currently used for the purpose of dual connectivity capabilities For NR only capabilities: - Support of different SCS in source and target cells in intra-band (separate signalling for intra-frequency and inter-frequency) HO

Based on the reply LS from RAN4, we think two points should be updated from the previous agreements.

Firstly, RAN2 need to consider changing capability name from singleUL-Transmission to a new one e.g. simulUL-transmission to imply simultaneous UL transmission. Normally UE does not indicate a lower capability than its baseline capability. In our view it is reasonable for UE to have a single UL transmission as a baseline capability and to optionally support simultaneous UL transmissions. Given this observation and reasoning, we propose:

Proposal 1: Change the capability name of singleUL-Transmission to new one e.g. simulUL-transmission to imply simultaneous UL transmission

Secondly, RAN2 agreed that the intraFreq-DAPS capability is the type of Per Band per BC. However, according to RAN4 response, capability of the intra-band HO is to be defined Per BC, rather than Per Band per BC. Considering that the intraFreq-DAPS capability is a subset of the intra-band HO capability, we think it is reasonable to change the granularity of the intraFreq-DAPS capability from Per Band per BC to Per BC.

Proposal 2: Change the type of capability intraFreq-DAPS to Per BC.

3. Conclusion

In this contribution, we have the following conclusion:

Proposal 1: Change the capability name of singleUL-Transmission to new one e.g. simulUL-transmission to imply simultaneous UL transmission

Proposal 2: Change the type of capability intraFreq-DAPS to Per BC.

4. Reference

[1] R2-2001661 Minutes LTE legacy, LTE TEI16 and NR/LTE Rel-16 Mobility topics, Vice Chairman (Nokia)

[2] R2-2000037 Reply to LS on UE capabilities on DAPS HO, RAN4

5. Appendix

RAN2 UE feature list for Mobility enhancement is referenced by the discussion of [AT109e][211], R2-2001727. For readability, some parts are merged or removed.

RAN2 UE feature list on NR mobility enhancement

| Features | Index | Feature group | Components | Need for the gNB to know if the feature is supported | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| x. Rel-16 NR Mobility Enhancement | X-0 | DAPS | - FFS Support of DAPS handover Consequence if the feature is not supported by the UE: The network cannot configure UE with DAPS HO | Yes | FFSPer BC | Yes | Yes | N/A | This is the general capability to cover default capabilities of DAPS HO, e.g. sync; dual UL, etc | Optional with capability signalling |
|  | x-1 | CHO | - Handling of CHO configuration, including execution condition and candidate cell configuration; - FFS Maximum candidate cells the UE can support; Default value FFS - CHO based failure handling; - FFS support of 2 trigger events for one execution condition Consequence if the feature is not supported by the UE: 1) The network cannot configure UE with CHO. 2) the network cannot configure candidate cells more than the max number the UE supported; 3) the network cannot configure the UE to perform CHO based failure handling 4) the network cannot configure 2 trigger events for one execution condition | Yes | per UE | Yes | Yes | FFS |  | Optional with capability signalling (1 and, 3) |
|  | x-2 | T312 based PCell fast failure recovery | - T312 based PCell fast failure recovery Consequence if the feature is not supported by the UE: The network cannot configure UE with T312 based PCell fast failure recovery. | Yes | per UE | Yes | Yes | support mixture of FDD/TDD and/or FR1/FR2 |  | Optional with capability signalling (1) |
|  | X-3 | T312 based PSCell fast failure recovery | - T312 based PSCell fast failure recovery Consequence if the feature is not supported by the UE: The network cannot configure UE with T312 based PSCell fast failure recovery | Yes | per UE | Yes | Yes | support mixture of FDD/TDD and/or FR1/FR2 |  | Optional with capability signalling (1) |

RAN2 UE feature list on LTE mobility enhancement

| Features | Index | Feature group | Components | Need for the eNB to know if the feature is supported | Type | Need of FDD/TDD differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| x. Rel-16 LTE Mobility Enhancement | X-0 | DAPS | - FFS Support of DAPS handover Consequence if the feature is not supported by the UE: The network cannot configure UE with DAPS HO | Yes | FFS Per BC | N/A | This is the general capability to cover default capabilities of DAPS HO, e.g. sync; dual UL, etc | Optional with capability signalling |
|  | x-1 | CHO | - Handling of CHO configuration, including execution condition and candidate cell configuration; - FFS Maximum candidate cells the UE can support; Default value FFS - CHO based failure handling; - FFS support of Multiple trigger events for one execution condition Consequence if the feature is not supported by the UE: 1) The network cannot configure UE with CHO. 2) the network cannot configure candidate cells more than the max number the UE supported; 3) the network cannot configure the UE to perform CHO based failure handling 4) the network cannot configure 2 trigger events for one execution condition | Yes | per UE | Yes |  | Optional with capability signalling (1 and 3) |