3GPP TSG RAN WG2#110-e	R2-2006057

Online meeting, 1st - 12th June, 2020

3GPP TSG-RAN WG1 Meeting #101-e	R1-2004808

E-meeting, 25 May – 5 June 202

Title:	Reply LS on Conflicting configurations

Response to:	R1-2004665 (R2-2004271)

Release:	Rel-16

Work Item:	NR_L1enh_URLLC-Core, NR_eMIMO-Core, NR_unlic-Core

Source:	RAN1

To:	RAN2

Cc:

Contact Person:

#### Name:	Xiuqiang Xu

#### E-mail Address:	xuxiuqiang@huawei.com

1. Overall Description:

RAN1 would like to thank RAN2 for the LS on conflicting configurations.  RAN1 discussed these questions and provides the corresponding answers as below:

1)	dmrs-UplinkTransformPrecoding-r16

dmrs-UplinkTransformPrecoding-r16 is in the IE DMRS-UplinkConfig and is optional with the condition that "tp-pi2BPSK is included in PUSCH-Config". DMRS-UplinkConfig is used for several fields:

-	in PUSCH-Config: dmrs-UplinkForPUSCH-MappingTypeA/B and dmrs-UplinkForPUSCH-MappingTypeA/B-ForDCI-Format0-2-r16

-	in ConfiguredGrantConfig: for cg-DMRS-Configuration

Q1-1)	Can dmrs-UplinkTransformPrecoding-r16 be configured for DCI format 0-2?

[Answer]: Yes.

Q1-2)	Is it possible to configure dmrs-UplinkTransformPrecoding-r16 independently for each mapping type of DCI formats other than 0-2 and for each mapping type of DCI format 0-2 (if the answer to Q1-2 is "yes") or what are the restrictions?

[Answer]: Yes. There is no restriction for the configuration of the parameter from RAN1 perspective.

2)	dmrs-Downlink-r16

dmrs-Downlink-r16 is in DMRS-DownlinkConfig which is used for several fields in PDSCH-Config: dmrs-DownlinkForPDSCH-MappingTypeA/B and dmrs-DownlinkForPDSCH-MappingTypeA/BForDCI-Format1-2-r16.

DCI format 1-2 is introduced in URLLC WI but dmrs-Downlink-r16 is introduced in eMIMO WI.

Q2-1)	Can dmrs-Downlink-r16 be used for DCI format 1-2?

[Answer]: Yes.

Q2-2)	Is it possible to configure dmrs-Downlink-r16 independently for each mapping type of DCI formats other than 1-2 and for each mapping type of DCI format 1-2 (if the answer to Q2-1 is "yes") or what are the restrictions?

[Answer]: Yes. There is no restriction for the configuration of the parameter from RAN1 perspective.

3)	PDSCH time domain resource allocation

PDSCH-TimeDomainResourceAllocation can be configured with repetitionNumber. Meanwhile, pdsch-TimeDomainAllocationListForDCI-Format1-2-r16 was introduced in PDSCH-Config.

Q3-1)	Can the PDSCH time domain resource allocation for DCI format 1-2 support the use of repetitionNumber?

[Answer]: Yes.

Q3-2)	If the answer to Q3-1 is yes, can repetitionNumber be configured in the PDSCH time domain resource allocation for DCI format 1-2 if it is not configured in the time domain resource allocation for other DCI formats (and vice-versa), or should it be configured in the PDSCH time domain resource allocation for all DCI formats or for none?

[Answer]: Yes. The configuration of the repetitionNumber in pdsch-TimeDomainResourceAllocationList and in pdsch-TimeDomainAllocationListForDCI-Format1-2-r16 is independent.

4)	PUSCH time domain resource allocation

For URLLC, a new Rel-16 IE, PUSCH-TimeDomainResourceAllocationNew-r16 (name will have to be changed to avoid "New"), was defined which includes the parameters of PUSCH-TimeDomainResourceAllocation plus startSymbol, length and numberOfRepetitions. In addition, mappingType and startSymbolAndLength, which were mandatory in the Rel-15 IE PUSCH-TimeDomainResourceAllocationList, are optional in the Rel-16 IE.

For NR-U, a new Rel-16 IE, PUSCH-TimeDomainResourceAllocation (name will have to be changed as well), was defined (in this meeting, so not in 38.331 v 16.0.0) which includes multiplePUSCH-Allocations where each allocation is defined by mappingType and startSymbolAndLength.

The new URLLC Rel-16 IE is used in PUSCH-Config for pusch-TimeDomainAllocationListForDCI-Format0-2-r16 and pusch-TimeDomainAllocationListForDCI-Format0-1-r16.

The Rel-15 version PUSCH-TimeDomainResourceAllocationList is used for pusch-TimeDomainAllocationList in PUSCH-Config and pusch-TimeDomainAllocationList in PUSCH-ConfigCommon.

Q4-1)	Can the multiplePUSCH-Allocations (introduced for NR-U) and startSymbol, length and numberOfRepetitions (introduced for URLLC) be configured in the same PUSCH time domain resource allocation table, used for one of the 2 above underlined fields?

[Answer]: No for Rel-16.

Q4-2)	Can the multiplePUSCH-Allocations (introduced for NR-U) be used for one of the 2 above underlined fields while startSymbol, length and numberOfRepetitions (introduced for URLLC) are used in another of the above underlined fields?

In Q4-3 and Q4-4, if the answer is "yes", please indicate all the associated restrictions if any.

[Answer]: No for Rel-16.

5)	DCI format 1_2 applicability to features introduced in NR_eMIMO WI

The IE ControlResourceSet includes both tci-PresentInDCI and tci-PresentInDCI-ForDCI-Format1-2. Currently both parameters can be configured in all or some CORESETs of the UE and these CORESETs may be configured with CORESETPoolIndex (mPDCCH mTRP). Further, eMIMO WI introduced a new TCI state mapping MAC CE in TS 38.321 6.1.3.24 where two TCI states can be mapped to one DCI codepoint. Currently, there is no limitation which DCI format this new MAC CE in TS 38.321 6.1.3.24 applies to.

Q5-1) Can the UE be configured with both DCI format 1_1 and DCI format 1_2 with TCI field, either in the same or different CORESETs? And can the value of tci-PresentInDCI-ForDCI-Format1-2 be different in different CORESETs?

[Answer]: Yes to both questions.

Q5-2) Can the UE be configured with mPDCCH mTRP (have at least on CORESET with CORESETPoolIndex=1) and the parameter tci-PresentInDCI-ForDCI-Format1-2?

[Answer]: Yes.

Q5-3) Does the Enhanced TCI state MAC CE in TS 38.321 6.1.3.24 apply to DCI1_2?

[Answer]: Yes.

2. Actions:

To RAN2

ACTION: RAN1 respectfully asks RAN2 to take the above information into account in their future work.

3. Date of Next TSG-RAN WG1 Meetings:

TSG-RAN WG1#102 		24 August – 4 September 2020			E-Meeting